# 3GPP TSG-RAN WG4 Meeting # 108                               R4-2311504

# Toulouse, France, August 21 – August 25, 2023

# Title: 	Discussion on ATG UE demodulation

# Source: 	ZTE Corporation

# Agenda item:	8.13.5.2

# Document for:	Approval

# 1 Introduction

In RAN#99e meeting[1], the revised work item on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

During the last RAN4 meeting, some conclusions which need further discussion for ATG DL demodulation are listed below[2].

Issue 2-1-1: Test scope for PDSCH - New PDSCH test cases for ATG with FDD slot pattern can be as follows. FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - New PDSCH test cases with static TDD slot pattern can be as follows (if introduced): TDD 40MHz 30kHz SCS; FFS on new TDD pattern: 30D4S6U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - For existing PDSCH test cases: - Choose 1 test case per Modulation Order for PDSCH Mapping Type A only, for FDD and TDD, FFS on which test cases to choose. - Consider Single Carrier requirements only Issue 2-1-4: MCS&Rank Agreement: - Cover 256QAM for ATG demod - Further discuss the MCS value based on simulation results from companies. Issue 2-1-5: Applicability rule - Option 1: New UE feature for ATG scenario and corresponding applicability rule shall be introduced, for instance UE capability; Test type; Test list. ATG with 2RX; FR1 FDD; PDSCH; All tests in Clause 5.2.1.1.x. PDCCH; All tests in Clause 5.3.2.1.2. FR1 TDD; PDSCH; All tests in Clause 5.2.1.2.x. PDCCH; All tests in Clause 5.3.2.2.2. ATG with 4RX; FR1 FDD; PDSCH; All tests in Clause 5.2.2.1.x. PDCCH; All tests in Clause 5.3.3.1.2. FR1 TDD; PDSCH; All tests in Clause 5.2.1.2.x. PDCCH; All tests in Clause 5.3.3.2.2. - Option 2: For the applicability of the ATG UE Demod requirements, follow ongoing discussion in RF on related ATG UE capabilities; PDCCH requirements Issue 2-2-1: Test scope for PDCCH Agreement: - No new test cases for PDCCH under the assumption that new PDSCH test cases will be specified. - FFS on the selected legacy test cases for ATG UEs CSI reporting requirements Issue 2-3-1: Test scope for CSI reporting Agreement: - FFS: The feasibility of the CSI reporting given the large propagation delay in the ATG scenario.

This contribution shares our views for the issues listed above.

# 2	Discussion

With regard to PDCCH requirements, it was agreed in RAN4 #107 meeting that no new test cases will be defined. Which legacy test cases for ATG UEs are FFS.

As AWGN+Doppler is agreed as the channel model for ATG scenario, the legacy requirement for PDCCH with TDL channel model can be reused for ATG requirements. The following shows some requirements which are copied from 38101-4[3].

Table 5.3.2.1.2-1: Minimum performance for PDCCH with 15 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 10; 24; 2; 4; R.PDCCH. 1-2.2 FDD; TDLC300-100; 2x2 Low; 1; 2.0. 2; 10; 48; 2; 8; R.PDCCH. 1-2.5 FDD; TDLC300-100; 2x2 Low; 1; -1.3. 3; 10; 48; 1; 8; R.PDCCH.1-1.3 FDD; TDLA30-10; 2x2 Low; 1; -0.2. Table 5.3.3.1.2-1: Minimum performance for PDCCH with 15 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 10; 24; 2; 4; R.PDCCH. 1-2.2 FDD; TDLC300-100; 2x4 Low; 1; -1.9. 2; 10; 48; 2; 8; R.PDCCH. 1-2.5 FDD; TDLC300-100; 2x4 Low; 1; -4.5. 3; 10; 48; 1; 4; R.PDCCH.1-1.2 FDD; TDLA30-10; 2x4 Low; 1; -1.0. Table 5.3.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x2 Low; 1; -1.2. Table 5.3.3.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x4 Low; 1; -4.3.

The above requirements can be considered to be reused for ATG PDCCH requirements.

Proposal 1. To consider legacy PDCCH requirements for ATG PDCCH requirements.

The propagation delay is 1ms assuming the distance between the aircraft and the BS is 300km and the speed is 1200kmph, the distance within 1ms has little effect on the angle which is shown in figure 2-1. Figure 2-1a illustrates the distance relationship for ATG scenario, where point A is the starting position of the cell coverage of BS1 along the flight direction of the aircraft, and point B is the ending position of the cell coverage of BS1. It can be found that the angle variation between UE and BS is very small when the aircraft enters the starting position of the coverage area.

Figure 2-1a, Location and distance relationship of ATG deployment

Figure 2-1b, Angle between UE and BS when the aircraft enters the starting point of the coverage area and the angle between the two after 1ms

From figure 2-1, it can be seen that the change in angle within 1ms (propagation delay) is negligible. And table 2-1 lists some angle variation relative to starting point for different time elapsed.

Table 2-1 time elapse vs angle variation

| Time elapsed | Angle variation(degree) |
| --- | --- |
| 1ms | 0.0001 |
| 10ms | 0.0004 |
| 100ms | 0.0038 |
| 1000ms | 0.0381 |

It can be found that the angle variation within up to 1000ms is negligible based on the above analysis. Therefore the PMI reporting can be considered for ATG deployment.

Similar to PMI analysis, it is expected that CQI does not change significantly within a propagation delay time.

Observation 1. CSI reporting such PMI and CQI is feasible for ATG scenario.

# 3 Conclusion

In this contribution, we give some conclusions for ATG UE demodulation. The conclusions are:

Observation 1. CSI reporting such PMI and CQI is feasible for ATG scenario.

Proposal 1. To consider legacy PDCCH requirements for ATG PDCCH requirements.

# 4	Reference

[1]  RP-230279, Revised WID on Air-to-ground network for NR, CMCC

[2]  R4-2309796, WF for ATG demodulation requirements, CMCC

[3]  3GPP TS 38101 v17.8.0