# 3GPP TSG-RAN WG4 Meeting # 108                               R4-2311505

# Toulouse, France, August 21 – August 25, 2023

# Title: 	Simulation results for ATG UE demodulation

# Source: 	ZTE Corporation

# Agenda item:	8.13.5.2

# Document for:	Discussion

# 1 Introduction

In RAN#99e meeting[1], the revised work item on Air-to-ground network for NR was approved.

During the last RAN4 meeting, some conclusions which need simulation for ATG DL demodulation are listed below[2].

Issue 2-1-1: Test scope for PDSCH - New PDSCH test cases for ATG with FDD slot pattern can be as follows. FDD 10 MHz 15kHz SCS; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - New PDSCH test cases with static TDD slot pattern can be as follows (if introduced): TDD 40MHz 30kHz SCS; FFS on new TDD pattern: 30D4S6U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 7D1S2U; 2T2R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). 2T4R; 16QAM (MCS [13] in table 1). 64QAM (MCS [22] in table 1). 256QAM (MCS [24] in table 2). - For existing PDSCH test cases: - Choose 1 test case per Modulation Order for PDSCH Mapping Type A only, for FDD and TDD, FFS on which test cases to choose. - Consider Single Carrier requirements only Issue 2-1-4: MCS&Rank Agreement: - Cover 256QAM for ATG demod - Further discuss the MCS value based on simulation results from companies.

This contribution shares our views for the issues listed above.

- Simulation results

Based on the simulation assumption which is listed in Annex, we provide the initial simulation results summarized in table 2-1 for reference. The throughput curves are listed in Annex B.

Table 2-1 simulation results for ATG DL demodulation

| Index | SCS | TDD pattern | Bandwidth | MCS | Antenna configuration | Propagation condition | SNR@70% max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 15kHz | - | 10MHz | MCS13 Table1 | 2x2 | AWGN + 220Hz Doppler | 0.80 |
| 1-2 | 15kHz | - | 10MHz | MCS22 Table1 | 2x2 | AWGN + 220Hz Doppler | 6.81 |
| 1-3 | 15kHz | - | 10MHz | MCS20 Table2 | 2x2 | AWGN + 220Hz Doppler | 12.80 |
| 1-4 | 15kHz | - | 10MHz | MCS24 Table2 | 2x2 | AWGN + 220Hz Doppler | 16.79 |
| 1-5 | 15kHz | - | 10MHz | MCS27 Table2 | 2x2 | AWGN + 220Hz Doppler | 18.80 |
| 1-6 | 30kHz | 7D1S2U | 40MHz | MCS13 Table1 | 2x2 | AWGN + 500Hz Doppler | 0.80 |
| 1-7 | 30kHz | 7D1S2U | 40MHz | MCS22 Table1 | 2x2 | AWGN + 500Hz Doppler | 6.81 |
| 1-8 | 30kHz | 7D1S2U | 40MHz | MCS20 Table2 | 2x2 | AWGN + 500Hz Doppler | 12.80 |
| 1-9 | 30kHz | 7D1S2U | 40MHz | MCS24 Table2 | 2x2 | AWGN + 500Hz Doppler | 16.80 |
| 1-10 | 30kHz | 7D1S2U | 40MHz | MCS27 Table2 | 2x2 | AWGN + 500Hz Doppler | 18.80 |
| 1-11 | 30kHz | 30D4S6U | 40MHz | MCS13 Table1 | 2x2 | AWGN + 500Hz Doppler | 0.81 |
| 1-12 | 30kHz | 30D4S6U | 40MHz | MCS22 Table1 | 2x2 | AWGN + 500Hz Doppler | 6.81 |
| 1-13 | 30kHz | 30D4S6U | 40MHz | MCS20 Table2 | 2x2 | AWGN + 500Hz Doppler | 12.81 |
| 1-14 | 30kHz | 30D4S6U | 40MHz | MCS24 Table2 | 2x2 | AWGN + 500Hz Doppler | 16.81 |
| 1-15 | 30kHz | 30D4S6U | 40MHz | MCS27 Table2 | 2x2 | AWGN + 500Hz Doppler | 18.81 |
| 2-1 | 15kHz | - | 10MHz | MCS13 Table1 | 2x4 | AWGN + 220Hz Doppler | -2.20 |
| 2-2 | 15kHz | - | 10MHz | MCS22 Table1 | 2x4 | AWGN + 220Hz Doppler | 3.81 |
| 2-3 | 15kHz | - | 10MHz | MCS20 Table2 | 2x4 | AWGN + 220Hz Doppler | 9.80 |
| 2-4 | 15kHz | - | 10MHz | MCS24 Table2 | 2x4 | AWGN + 220Hz Doppler | 13.79 |
| 2-5 | 15kHz | - | 10MHz | MCS27 Table2 | 2x4 | AWGN + 220Hz Doppler | 15.80 |
| 2-6 | 30kHz | 7D1S2U | 40MHz | MCS13 Table1 | 2x4 | AWGN + 500Hz Doppler | -2.20 |
| 2-7 | 30kHz | 7D1S2U | 40MHz | MCS22 Table1 | 2x4 | AWGN + 500Hz Doppler | 3.81 |
| 2-8 | 30kHz | 7D1S2U | 40MHz | MCS20 Table2 | 2x4 | AWGN + 500Hz Doppler | 9.80 |
| 2-9 | 30kHz | 7D1S2U | 40MHz | MCS24 Table2 | 2x4 | AWGN + 500Hz Doppler | 13.80 |
| 2-10 | 30kHz | 7D1S2U | 40MHz | MCS27 Table2 | 2x4 | AWGN + 500Hz Doppler | 15.80 |
| 2-11 | 30kHz | 30D4S6U | 40MHz | MCS13 Table1 | 2x4 | AWGN + 500Hz Doppler | -2.19 |
| 2-12 | 30kHz | 30D4S6U | 40MHz | MCS22 Table1 | 2x4 | AWGN + 500Hz Doppler | 3.82 |
| 2-13 | 30kHz | 30D4S6U | 40MHz | MCS20 Table2 | 2x4 | AWGN + 500Hz Doppler | 9.81 |
| 2-14 | 30kHz | 30D4S6U | 40MHz | MCS24 Table2 | 2x4 | AWGN + 500Hz Doppler | 13.81 |
| 2-15 | 30kHz | 30D4S6U | 40MHz | MCS27 Table2 | 2x4 | AWGN + 500Hz Doppler | 15.81 |

# 3 Conclusion

In this contribution, we give some simulation results for ATG UE DL demodulation .

# 4	Reference

[1]  RP-230279, Revised WID on Air-to-ground network for NR, CMCC

[2]  R4-2309796, WF for ATG demodulation requirements, CMCC

# Annex A

Table A-1 Simulation assumption

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 |
| Bandwidth |  | MHz | 10 | 40 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U |
|  |  |  |  | 30D4S6U S:40G |
| Propagation channel |  |  | AWGN+220Hz doppler | AWGN+500Hz doppler |
| Antenna configuration |  |  | 2x2 | 2x2 |
|  |  |  | 2x4 | 2x4 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| MCS/Rank |  |  | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 20/24/27 | Rank 1 16QAM Table 1 MCS13 64QAM Table 1 MCS 22 256QAM Table 2 MCS 20/24/27 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 |
|  | PRB bundling type |  | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 3 | 3 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  | TCI state #1 | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  | 5 | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 4 | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | 'FD-CDM2' | 'FD-CDM2' |
|  | Density (ρ) |  | 1 | 1 |
|  | CSI-RS periodicity | Slots | 20 | 40 |
|  | CSI-RS offset | Slots | 0 | 0 |
|  | Frequency Occupation |  | Start PRB 0 | Start PRB 0 |
|  |  |  | Number of PRB = ceil(BWP size/4)*4 | Number of PRB = ceil(BWP size/4)*4 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Number of HARQ Processes |  |  | 4 | 8 for 7DS2U 32 for 30D4S6U |
| Test metric |  |  | 70% of max throughput at target SNR. |  |

# Annex B

Figure B-1 Normalized throughput of MCS 13(table 1) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-2 Normalized throughput of MCS 22(table 1) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-3 Normalized throughput of MCS 20(table 2) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-4 Normalized throughput of MCS 24(table 2) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-5 Normalized throughput of MCS 27(table 2) for FDD 15kHz SCS 10MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-6 Normalized throughput of MCS 13(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-7 Normalized throughput of MCS 22(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-8 Normalized throughput of MCS 20(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-9 Normalized throughput of MCS 24(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-10 Normalized throughput of MCS 27(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-11 Normalized throughput of MCS 13(table 1) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-12 Normalized throughput of MCS 22(table 1) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-13 Normalized throughput of MCS 20(table 2) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-14 Normalized throughput of MCS 27(table 2) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

Figure B-15 Normalized throughput of MCS 27(table 2) for TDD(30D4S6U) 30kHz SCS 40MHz BW with 2X2 antenna under AWGN with 0.1ppm Doppler shift

2X4

Figure B-16 Normalized throughput of MCS 13(table 1) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-17 Normalized throughput of MCS 22(table 1) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-18 Normalized throughput of MCS 20(table 2) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-19 Normalized throughput of MCS 24(table 2) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-20 Normalized throughput of MCS 27(table 2) for FDD 15kHz SCS 10MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-21 Normalized throughput of MCS 13(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-22 Normalized throughput of MCS 22(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-23 Normalized throughput of MCS 20(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-24 Normalized throughput of MCS 24(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-25 Normalized throughput of MCS 27(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-26 Normalized throughput of MCS 13(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-27 Normalized throughput of MCS 22(table 1) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-28 Normalized throughput of MCS 20(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-29 Normalized throughput of MCS 24(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift

Figure B-30 Normalized throughput of MCS 27(table 2) for TDD(7D1S2U) 30kHz SCS 40MHz BW with 2X4 antenna under AWGN with 0.1ppm Doppler shift