# 3GPP TSG-RAN WG4 Meeting # 107                               R4-2311506

# Toulouse, France, August 21 – August 25, 2023

# Title: 	Simulation results for 4Tx of NR_ENDC_RF_FR1_enh2

# Source: 	ZTE Corporation

# Agenda item:	8.4.3.2

# Document for:	Discussion

# 1 Introduction

The objectives for 4Tx of further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1) are listed in the following [1]:

Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4] - Investigate framework and architecture Example bands: - TDD bands: n41, n77/n78 - FDD bands: n1 - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 4Tx - First priority: 4x4 UL MIMO - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port - PA configuration assumption: - First priority: 4x23dBm - Second priority: 2x23dBm + 2x26dBm, 4x26dBm - UE power class - First priority: PC 1.5 - Second priority: PC2/PC3, and/or new power class if needed - Note 1: PC1.5 is only applicable for TDD bands - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97 - NOTE1: Requirements are specified with phase approach. Objectives with 1st priority are considered first. - NOTE2: It is assumed the devices are equipped with 4Rx antennas for band n1 requirement definition. The objectives of performance part for Rel-18 RF FR1 requirement focus evolution include: - Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices - Specify the BS demodulation performance requirements to support UL 4-layer MIMO UE operation - Specify release independence requirements in TS 38.307 if needed.

The following simulation assumptions were reached in last RAN4 meeting[2]:

Issue 1-1-1: Bandwidth selection between 20MHz and 50MHz Agreement: - For 15kHz: - Introduce requirements for 5MHz and 50MHz, FFS on 20MHz - For 30kHz: - Introduce requirements for 10MHz and 100MHz, FFS on 40MHz Issue 1-1-2: MCS Agreement: - For TDLB100-400: MCS2 - For TDLA30-10: MCS 17 - For TDLC300-100: MCS12 Issue 1-1-3: Antenna configuration Agreement: - Define requirements for both 4Rx and 8Rx. Issue 1-1-4: Whether to define same requirement for PUSCH mapping type A and B Agreement: - Define separate requirements for PUSCH mapping type A and mapping type B.

In this contribution, we share our initial simulation results for 4Tx.

- Discussion

The simulation parameters agreed in RAN4 #106 and RAN4 #106bis-e are summarized in table 2-1 and the operating SNR is summarized in section 2.1, 2.2 and 2.3. The simulation curves are appended in Annex for reference.

Table 2-1 Simulation parameter for 4Tx

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth 15kHz SCS: 5MHz and 50MHz, FFS on 20MHz 30kHz SCS: 10MHz and 100MHz, FFS on 40MHz |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Channel model and MCS |  | TDLB100-400: MCS2 TDLC300-100: MCS12 TDLA30-10: MCS17 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

## 2.1 Results for TDLA30-10: MCS17

| Parameter |  |  | Operating SNR (@ 70% TPUT) |  |
| --- | --- | --- | --- | --- |
| Ant. | SCS | BW | Mapping Type A | Mapping Type B |
| 4T4R | 15kHz | 5MHz | 17.1 | 17.5 |
|  |  | 20MHz | 17.9 | 17.9 |
|  |  | 50MHz | 19.1 | 19.3 |
|  | 30kHz | 10MHz | 17.1 | 17.2 |
|  |  | 40MHz | 17.5 | 17.8 |
|  |  | 100MHz | 18.5 | 18.7 |
| 4T8R | 15kHz | 5MHz | 9.7 | 10.0 |
|  |  | 20MHz | 10.1 | 10.3 |
|  |  | 50MHz | 10.8 | 10.9 |
|  | 30kHz | 10MHz | 9.7 | 10.1 |
|  |  | 40MHz | 10.0 | 10.5 |
|  |  | 100MHz | 10.6 | 10.7 |

## 2.2 Results for TDLB100-400: MCS2

| Parameter |  |  | Operating SNR (@ 70% TPUT) |  |
| --- | --- | --- | --- | --- |
| Ant. | SCS | BW | Mapping Type A | Mapping Type B |
| 4T4R | 15kHz | 5MHz | -0.8 | -0.5 |
|  |  | 50MHz | 0 | 0.5 |
|  | 30kHz | 10MHz | -1 | -0.6 |
|  |  | 100MHz | 0.3 | 0.6 |
| 4T8R | 15kHz | 5MHz | -3.3 | -3.1 |
|  |  | 50MHz | -3.2 | -3.0 |
|  | 30kHz | 10MHz | -3.3 | -3.2 |
|  |  | 100MHz | -3.1 | -2.9 |

## 2.3 Results for TDLC300-100: MCS12

| Parameter |  |  | Operating SNR (@ 70% TPUT) |  |
| --- | --- | --- | --- | --- |
| Ant. | SCS | BW | Mapping Type A | Mapping Type B |
| 4T4R | 15kHz | 5MHz | 12.2 | 12.4 |
|  |  | 50MHz | 12.7 | 12.8 |
|  | 30kHz | 10MHz | 12.8 | 13 |
|  |  | 100MHz | 14.0 | 14.2 |
| 4T8R | 15kHz | 5MHz | 6.2 | 6.5 |
|  |  | 50MHz | 6.7 | 6.7 |
|  | 30kHz | 10MHz | 6.4 | 6.7 |
|  |  | 100MHz | 6.8 | 6.8 |

# 3 Conclusion

In this contribution, we give some simulation results on the 4Tx of NR_ENDC_RF_FR1_enh2.

# 4	Reference

[1]  RP-223478 Revised WID Rel-18 NR_ENDC_ RF_FR1_enh2, Huawei, HiSilicon, NTT DOCOMO

[2]  R4-2305890 WF on FR1 4Tx demodulation requirement, Ericsson

# Annex

Figure A-1 Throughput comparison for MCS17, type A, 4X4, TDLA30-10 with different BW/SCS

Figure A-2 Throughput comparison for MCS2, type A, 4X4, TDLB100-400 with different BW/SCS

Figure A-3 Throughput comparison for MCS12, type A, 4X4, TDLC300-100 with different BW/SCS

Figure A-4 Throughput comparison for MCS17, type B, 4X4, TDLC30-10 with different BW/SCS

Figure A-5 Throughput comparison for MCS12, type B, 4X4, TDLB100-400 with different BW/SCS

Figure A-6 Throughput comparison for MCS12, type B, 4X4, TDLC300-100 with different BW/SCS

Figure A-7 Throughput comparison for MCS17, type A, 4X8, TDLA30-10 with different BW/SCS

Figure A-8 Throughput comparison for MCS2, type A, 4X8, TDLB100-400 with different BW/SCS

Figure A-9 Throughput comparison for MCS12, type A, 4X8, TDLC300-100 with different BW/SCS

Figure A-10 Throughput comparison for MCS17, type B, 4X8, TDLA30-10 with different BW/SCS

Figure A-11Throughput comparison for MCS2, type B, 4X8, TDLB100-400 with different BW/SCS

Figure A-12 Throughput comparison for MCS17, type B, 4X8, TDLA30-10 with different BW/SCS