# 3GPP TSG-RAN WG4 Meeting # 108                                  R4-2311508

# Toulouse, France, Aug 21 – Aug 25, 2023

# Title: 	Simulation results for PDSCH demodulation requirements for 8Rx

# Source: 	ZTE Corporation

# Agenda item:	8.4.3.1.2

# Document for:	Discussion

# 1 Introduction

- In RAN4#107 meeting, 8Rx UE demodulation and CSI requirements were discussed and a WF was agreed[1]. In this paper, we will provide our simulation results on 8Rx PDSCH for FR1.

# 2 Discussion

  - 2.2 Simulation results

  - In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in following Tables.

  - 2.2.1 TDD

Table 2-1. Summary of PDSCH 8Rx simulation results for TDD

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL/DL Pattern | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | 64QAM, 0.5 | FR1.30-1 | 2 | TDLC300-100 | 2x8, ULA Medium | 70 | 10.5 |
| 1-2 | 40 / 30 | 256QAM, 0.67 | FR1.30-1 | 2 | TDLC300-100 | 2x8, ULA Medium | 70 | 20.5 |
| 1-3 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 10.6 |
| 1-3 | 40 / 30 | 64QAM, 0.85 | FR1.30-1 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 20.4 |
| 1-5 | 40 / 30 | 64QAM, 0.43 | FR1.30-1 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |

Table 2-2. PDSCH 8Rx simulation results for TDD

| - Rank 2, MCS 19, TDLC300-100 ULA Medium B | - Rank 2, MCS 20, TDLC300-100 ULA Medium B |
| --- | --- |
| - Rank 4, MCS 17, TDLA30-10 ULA Low | - Rank 4, MCS 26, TDLA30-10 ULA Low |
| - Rank 8, MCS 17, TDLA30-10 ULA Low |  |

  - 2.2.2 FDD

Table 2-3. Summary of PDSCH 8Rx simulation results for FDD

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 10 / 15 | 64QAM, 0.5 | 2 | TDLC300-100 | 2x8, ULA Medium | 70 | 10.44 |
| 1-2 | 10 / 15 | 256QAM, 0.67 | 2 | TDLC300-100 | 2x8, ULA Medium | 70 | 19.3 |
| 1-3 | 10 / 15 | 64QAM, 0.43 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 10.2 |
| 1-3 | 10 / 15 | 64QAM, 0.85 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 19.0 |
| 1-5 | 10 / 15 | 64QAM, 0.43 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |

Table 2-4. PDSCH 8Rx simulation results for FDD

| - Rank 2, MCS 19, TDLC300-100 ULA Medium B | - Rank 2, MCS 20, TDLC300-100 ULA Medium B |
| --- | --- |
| - Rank 4, MCS 17, TDLA30-10 ULA Low | - Rank 4, MCS 26, TDLA30-10 ULA Low |
| - Rank 8, MCS 17, TDLA30-10 ULA Low |  |

  - 2.2.3 CA

  - And for 8Rx CA, we also did some simulations as shown in following Table 2-5 and Table 2-6 which covered MCS 13(Table 1) and MCS 17(Table 1) with rank 8.

Table 2-5. Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 13.6 |
| 5 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 |
| 10 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 13.7 |
| 10 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| 15 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 13.9 |
| 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.0 |
| 20 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.2 |
| 20 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |
| 25 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.2 |
| 25 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 |
| 30 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.4 |
| 30 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 35 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.9 |  |
| 35 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 |  |
| 40 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.9 |
| 40 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 50 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.8 |
| 50 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.5 |

Table 2-6. Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 13.3 |
| 5 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 18.5 |
| 10 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 13.9 |
| 10 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 15 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.2 |
| 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 |
| 20 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.1 |
| 20 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 |
| 25 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.1 |
| 25 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 |
| 30 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.1 |
| 30 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 |
| 35 | 16QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 14.3 |
| 35 | 64QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 19.8 |
| 40 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.4 |
| 40 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |
| 50 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.5 |
| 50 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 |
| 60 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.9 |
| 60 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 |

| 70 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.9 |
| --- | --- | --- | --- | --- | --- |
| 70 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 80 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 14.9 |
| 80 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |
| 90 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 15.1 |
| 90 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 |
| 100 | 16QAM, 0.48 | TDLA30-10 | 8x8, ULA Low | 70 | 15.0 |
| 100 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.0 |

# 3 Conclusion

In this contribution, we provide our simulation results for FR1 PDSCH 8Rx .

# 4 References

- R4-2309806, WF for 8Rx UE performance requirements. RAN4#107 meeting.