# 3GPP TSG-RAN WG4 Meeting # 108                                   R4-2311513

# Toulouse, France, Aug 21 – Aug 25, 2023

# Title: 	Test parameters and simulation results for MU-MIMO

# Source: 	ZTE Corporation

# Agenda item:	8.18.1.2

# Document for:	Approval

# 1 Introduction

- In RAN4#107 meeting, some conclusions has been reached for advanced receiver and a WF was agreed[1]. In this paper, we will provide our simulation results for MU-MIMO.

# 2 Discussion

# 2.1 Simulation assumptions

- The simulation assumptions are summarized in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 4,13,17 | Rank 1+1: QPSK Rank 2+2: QPSK, 16QAM and 64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R ULA Medium For Rank 2+2: 4T4R ULA Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Option 1: Select the precoding matrix to ensure orthogonality with target UE - Option 2: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100,TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC,E-MMSE-IRC, R-ML | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

  - 2.2 Simulation results

  - 2.2.1 Rank 1+1

In this section, we compared the throughput performance of R-ML receiver, MMSE-IRC receiver and E-MMSE-IRC receiver in case of all information of co-schedule UE is known to target UE.

  - The results of link level analysis are shown in Figure 2-1 and Figure 2-2. And for Figure 2-2 which is partial CHBW for co-schedule UE.

| - Figure 2-1. Rank 1+1 MCS4 2T2R TDLC300-100 Medium, co-schedule UE: QPSK | - Figure 2-2. Rank 1+1 MCS13 2T2R TDLC300-100 Medium, co-schedule UE: QPSK |
| --- | --- |

  - 2.2.2 Rank 2+2

In this section, we compare the throughput performance of R-ML receiver, MMSE-IRC receiver and E-MMSE-IRC receiver in case of all information of co-schedule UE is known to target UE.

  - The results of link level analysis for Rank 2+2 are shown in Figure 2-3 and Figure 2-4.

| - Figure 2-3. Rank 2+2 MCS13 4T4R TDLA30-10 Low, co-schedule UE: 16QAM | - Figure 2-4. Rank 2+2 MCS17 4T4R TDLA30-10 Low, co-schedule UE: 16QAM |
| --- | --- |

  - And summary results are shown in Table 2-1.

Table 2-1. Summary of simulation results

| Rank allocation | Modulation Order |  | Propagation conditions | PMI selection | Antenna configuration | MMSE-IRC | E-MMSE-IRC |  | R-ML |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Target UE | Co-schedule UE(s) |  |  |  | SNR (dB) | SNR (dB) | Gain (dB) | SNR (dB) | Gain (dB) |
| Rank1+1 | MCS 4 | QPSK | TDLC300-100 ULA medium | Random | 2T2R | 4.7 | 4.1 | 0.6 | 3.2 | 1.5 |
| Rank1+1 | MCS 13 | QPSK(partial CHBW) | TDLC300-100 ULA medium | Random | 2T2R | 15.2 | 14.1 | 1.1 | 10.8 | 4.4 |
| Rank2+2 | MCS 13 | 16QAM | TDLA30-10 ULA Low | Orthogonal | 4T4R | 13.3 | 13 | 0.3 | 11.8 | 1.5 |
| Rank2+2 | MCS 17 | 16QAM | TDLA30-10 ULA Low | Orthogonal | 4T4R | 17.7 | 17.3 | 0.4 | 16.3 | 1.4 |

We can make following conclusions based on the observations from above simulation results:

Observation 1. E-MMSE-IRC receiver has less performance gain over MMSE-IRC for TDLA30-10 and TDLC300-100.

Observation 2. In partial CHBW scenario, R-ML receiver has significant performance gain over E-MMSE-IRC.

## 2.2.3 Blind detection

In this section, we provide the summary of link-level analysis of the co-schedule UE(s) modulation order blind detection performance and advanced receiver throughput performance. The simulation results for the co-schedule UE(s) modulation detection error rate and normalized throughput are illustrated below:

| - Figure 2-5. Modulation order detection error rate | - Figure 2-6. Rank 1+1 MCS13 2T2R TDLC300-100 Medium, co-schedule UE: QPSK |
| --- | --- |

And for Figure 2-7 and Figure 2-8 which covered two co-schedule UEs for blind detection.

| - Figure 2-7. Modulation order detection error rate | - Figure 2-8. Rank 1+1 MCS13 2T2R TDLC300-100 Medium, co-schedule UEs: QPSK and 16QAM |
| --- | --- |

  - And summary results are shown in Table 2-2.

Table 2-2. Summary of simulation results

| Rank allocation | Modulation Order |  | Propagation conditions | PMI selection | Antenna configuration | MMSE-IRC | E-MMSE-IRC |  | R-ML(BD) |  | R-ML (Genie) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Target UE | Co-schedule UE(s) |  |  |  | SNR (dB) | SNR (dB) | Gain (dB) | SNR (dB) | Gain (dB) | SNR (dB) |
| Rank1+1 | MCS 13 | QPSK | TDLC300-100 ULA medium | Random | 2T2R | 22.4 | 19.8 | 3.6 | 12.7 | 9.7 | 12.7 |
| Rank1+1 | MCS 13 | QPSK,16QAM | TDLC300-100 ULA medium | Random | 2T2R | 23.4 | 17.9 | 5.5 | 12.2 | 11.2 | 12.2 |

Based on these results we make the following observations with respect to the modulation order detection for Rank 1+1 :

Observation 3. The modulation order detection error rate decreases with SNR growth.

Observation 4. Modulation order blind detection has almost no performance degradation for R-ML receiver  in Rank 1+1 scenario.

Regarding blind detection complexity, the R-ML receiver based modulation classification involves search all possible combinations of the modulation order which include QPSK, 16QAM, 64QAM and 256QAM for co-schedule UE(s). Since blind detection needs to search all candidate constellation points, the total complexity should be 16×(4+16+64+256) in each data REs for Rank 1+1.

Also for Rank 2+2 with MCS 17 for target UE, the complexity should be 64×64×(4+16+64+256)×(4+16+64+256) for each data REs. So in Rank 2+2 or more layers, the R-ML receiver based blind detection may become a bottleneck factor from the complexity perspective.

Observation 5. Blind detection may imply significant computational complexity burden on UE implementation in multi-layers.

# 3 Conclusion

In this contribution, we give some simulation results on demodulation performance requirements for MU-MIMO , The conclusions are:

Observation 1. E-MMSE-IRC receiver has less performance gain over MMSE-IRC for TDLA30-10 and TDLC300-100.

Observation 2. In partial CHBW scenario, R-ML receiver has significant performance gain over E-MMSE-IRC.

Observation 3. The modulation order detection error rate decreases with SNR growth.

Observation 4. Modulation order blind detection has almost no performance degradation in Rank 1+1 scenario.

Observation 5. Blind detection may imply significant computational complexity burden on UE implementation in multi-layers.

# 4 References

- R4-2309892, WF on advanced receiver for MU-MIMO scenario, China Telecom.