# 3GPP TSG-RAN WG4 Meeting # 108                                  R4-2311516

# Toulouse, France, Aug 21 – Aug 25, 2023

# Title: 	Simulation results for NCR-MT demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	8.28.6

# Document for:	Discussion

# 1 Introduction

During the last RAN4 meeting[1], some conclusions has been reached for NCR-MT demodulation requirements. In this contribution, we want to share some simulation results for NCR-MT which include PDSCH and PDCCH requirements.

# 2 Discussion

# 2.1 Simulation assumptions

  - 2.1.1  PDSCH Simulation assumptions and results

### 2.1.1.1  FR1

According previous meeting[1][2] conclusions, we summarized the following Table 2-1 and Table 2-2 for PDSCH FR1 simulation assumptions and FR2 simulation assumptions.

- Table 2-1. Simulation assumptions for PDSCH FR1

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | FDD | TDD |
| Channel Bandwidth/SCS |  | 10/15 | 40/30 |
| TDD pattern |  |  | 7D1S2U S=6D+4G+4U |
| MCS |  | 4 | 4 |
| Rank |  | 1 | 1 |
| DMRS port allocation |  | Rank 1: Port 1000 | Rank 1: Port 1000 |
| MIMO configuration |  | 2T2R ULA Low and 2T4R ULA Low |  |
| Number of CDM groups without data |  | 1 |  |
| HARQ process number |  | 4 for FDD and 8 for TDD(70% of maximum Throughput) 1( 1% BLER ) |  |
| Precoding model |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Random precoding with Single panel Type 1 per PRB bundling size per slot |
| PDSCH configuration | Mapping type | Type A |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PRB bundling size | 2 |  |
|  | PRB bundling type | Static |  |
| PDSCH DMRS configuration | DMRS Type | DMRS Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |  |
| Propagation conditions |  | TDLA30-10 |  |
| Receiver type |  | MMSE-IRC |  |
| Test metric |  | SNR @ 70% of maximum Throughput SNR @ 1% BLER |  |

- Table 2-2. Simulation assumptions for PDSCH FR2

| Parameter |  | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 for 70% of maximum Throughput and 1 for 1% BLER |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | DM-RS port(s) | {1000} for Rank 1 tests |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 1 |
|  | Allocation length | 13 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Propagation conditions |  | TDLA30-75 |
| Receiver type |  | MMSE-IRC |
| Test metric |  | SNR @ 70% of maximum Throughput SNR @ 1% BLER |

- The following table lists all need requirements for PDSCH FR1.

- ‘

- Table 2-3. Minimum performance for Rank 1

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation conditions | Antenna configuration | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70% maximum throughput | -3.2 |
| 1-2 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70% maximum throughput | -6.0 |
| 1-3 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 1% BLER | 1.9 |
| 1-4 | 10/15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 1% BLER | -2.5 |
| 1-5 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70% maximum throughput | Available in 38.101-4 |
| 1-6 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70% maximum throughput | Available in 38.101-4 |
| 1-7 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 1% BLER | 2.1 |
| 1-8 | 40/30 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 1% BLER | -3.2 |

- From above Table 2-3, it can be seen that new simulation are needed for 6 cases. In our understanding, for test 1-5 and 1-6, we notice that current TS 38.101-4 requirements defined in different patterns(FR1.30-2, FR1.30-3). Considering same requirements are applicable to TDD with different UL-DL patterns. Since no new simulations are needed in two cases.

- Also simulation results are shown in the following Table 2-4.

- Table 2-4. Simulation results for PDSCH FR1

| - 10M/15K MCS 4 Rank 1 TDLA30-10 | - 10M/15K MCS 4 Rank 1 TDLA30-10 |
| --- | --- |
| - 40M/30K MCS 4 Rank 1 TDLA30-10 | - 10M/15K MCS 4 Rank 1 TDLA30-10 |
| - 40M/30K MCS 4 Rank 1 TDLA30-10 |  |

- 2.1.1.2  FR2

- For FR2, PDSCH requirements are needed for 120kHz SCS. Considering the antenna configuration is 2RX Since only 1% BLER and 70% maximum throughput were considered. All required requirements are listed in the following table.

- Table 2-5. Minimum performance for Rank 1

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation conditions | Antenna configuration | Test metric | SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 100/120 | QPSK, 0.30 | TDLA30-75 | 2x2, ULA Low | 70% maximum throughput | -3.2 |
| 2-2 | 100/120 | QPSK, 0.30 | TDLA30-75 | 2x2, ULA Low | 1% BLER | 0.4 |

- Also simulation results are shown in the following Table 2-6.

- Table 2-6. Simulation results for PDSCH FR2

| - 100M/120K MCS 4 Rank 1 TDLA30-75 | - 100M/120K MCS 4 Rank 1 TDLA30-75 |
| --- | --- |

  - 2.1.2  PDCCH Simulation assumptions and results

### 2.1.1.2  FR1

- According previous meeting[1][2] conclusions, we summarized the following Table 2-7 for PDCCH FDD simulation assumptions.

- Table 2-7. Test parameters for PDCCH FDD

| Parameter | Value |
| --- | --- |
| Cyclic prefix | Normal |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Not-Interleaved |
| Interleaver size | 3 |
| REG bundle size | 6 for test with 1Tx and 2Tx |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| DCI Format | 1_1 |
| Payload (without CRC) | 52 |

  - In RAN4#106 bis-e meeting and RAN4 #107 meeting, we discussed almost all the parameters for PDCCH and had agreements in[1][2]. Since we would like to list all requirements in following Table 2-8 which is needed for NCR-MT PDCCH FDD demodulation requirements.

- Table 2-8. Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x2 Low | Available in 38.101-4 |
| 2 | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 Low | Available in 38.101-4 |
| 3 | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 Low | Available in 38.101-4 |
| 4 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x4 Low | Available in 38.101-4 |
| 5 | 10 | 48 | 2 | 4 | TDLA30-10 | 1x4 Low | Available in 38.101-4 |
| 6 | 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | -7.3 |

  - After we sorted out the FDD requirements for TS 38.101-4 and TS 38.174. It can be seen that the existing requirements were covered Test 1 - Test 5. And for Test 6, we notice that existing requirements only covered aggregation level 4 in TS 38.101-4 not aggregation level 8.

  - Since we need a new simulation for Test 6. And the simulation results as shown in following Table 2-9.

  - Table 2-9. Simulation results for PDCCH with 15kHz

| - 10M/15K Aggregation level = 8 |  |
| --- | --- |

  - For TDD requirements, we can reuse existing requirements from TS 38.174. Thus we don’t need new simulations for PDCCH TDD.

- Table 2-10. Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 102 | 1 | 2 | TDLA30-10 | 1x2 Low | Available in 38.174 |
| 2 | 40 | 102 | 1 | 4 | TDLA30-10 | 1x2 Low | Available in 38.174 |
| 3 | 40 | 90 | 1 | 8 | TDLA30-10 | 2x2 Low | Available in 38.174 |
| 4 | 40 | 102 | 1 | 2 | TDLA30-10 | 1x4 Low | Available in 38.174 |
| 5 | 40 | 102 | 1 | 4 | TDLA30-10 | 1x4 Low | Available in 38.174 |
| 6 | 40 | 90 | 1 | 8 | TDLA30-10 | 2x4 Low | Available in 38.174 |

### 2.1.1.2  FR2

- Table 2-11. Minimum performance for PDCCH with 120 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 60 | 1 | 2 | TDLA30-75 | 1x2 Low | Available in 38.174 |
| 2 | 100 | 60 | 1 | 4 | TDLA30-75 | 1x2 Low | Available in 38.174 |
| 3 | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 Low | Available in 38.174 |

  - For FR2 requirements, we can reuse existing requirements from 38.174. Thus we don’t need new simulations for 120 kHz.

### 2.1.1.3  DCI 2_8

Agreement: - Reuse IAB-MT and legacy UE requirements as much as possible as baseline. - FFS for adapt PDCCH requirement for testing of DCI type 2_8 used for access link beam change indication. - Interesting companies can bring simulation results for DCI type 2_8.

  - And for new DCI type 2_8, according to the RAN4 #107 meeting[1] conclusions, we did some simulations for new DCI 2_8.

- Firstly, we did some simulations for PDCCH with 10M/15kHz. Simulation results were shown in Table 2-14 and Figure 2-1. In addition, simulation assumptions were shown in Table 2-7. The only difference is that payload size is 128.

- Table 2-12. Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x2 Low | 11.6 |
| 2 | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 Low | 8.0 |
| 3 | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 Low | 0 |
| 4 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x4 Low | 4.0 |
| 5 | 10 | 48 | 2 | 4 | TDLA30-10 | 1x4 Low | 1.7 |
| 6 | 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | -4.4 |

- Figure 2-1. DCI 2_8 simulation results for 10M/15K with Rank 1

- And for 40M/30kHz, we also did some simulations for the all cases which were covered in Table 2-14. Simulation results are shown in Table 2-14 and Figure 2-2. In addition, simulation assumptions were shown in Table 2-13.

- Table 2-13. Test parameters for PDCCH with 30kHz

| Parameter | Value |
| --- | --- |
| Cyclic prefix | Normal |
| Default TDD UL-DL pattern (Note 1) | 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Interleaved |
| Interleaver size | 3 |
| REG bundle size | 2 for test with 1Tx 6 for test with 2Tx |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| Payload size | 128 |

- Table 2-14. Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 102 | 1 | 2 | TDLA30-10 | 1x2 Low | 9.9 |
| 2 | 40 | 102 | 1 | 4 | TDLA30-10 | 1x2 Low | 4.2 |
| 3 | 40 | 90 | 1 | 8 | TDLA30-10 | 2x2 Low | -1.3 |
| 4 | 40 | 102 | 1 | 2 | TDLA30-10 | 1x4 Low | 3.5 |
| 5 | 40 | 102 | 1 | 4 | TDLA30-10 | 1x4 Low | -0.5 |
| 6 | 40 | 90 | 1 | 8 | TDLA30-10 | 2x4 Low | -5.3 |

- Figure 2-2. DCI 2_8 simulation results for 40M/30K with Rank 1

- With regard to FR2 requirements, considering we can reuse existing simulation assumptions from IAB-MT. So the simulation assumption is shown in following Table 2-15. Also simulation results are shown in Table 2-16 and Figure 2-3.

- Table 2-15. Simulation assumptions for PDCCH with 120kHz

| Parameter | Value |
| --- | --- |
| Cyclic prefix | Normal |
| Default TDD UL-DL pattern (Note 1) | 3D1S1U, S=10D:2G:2U |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Interleaved |
| Interleaver size | 2 for test with Aggregation level 4 3 for others |
| REG bundle size | 6 for test with Aggregation level 4 2 for others |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| Payload size | 128 |

- Table 2-16. Minimum performance for PDCCH with 120 kHz

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 60 | 1 | 2 | TDLA30-75 | 1x2 Low | 9.6 |
| 2 | 100 | 60 | 1 | 4 | TDLA30-75 | 1x2 Low | 3.0 |
| 3 | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 Low | -1.1 |

- Figure 2-3. DCI 2_8 simulation results for 100M/120K with Rank 1

# 3 Conclusion

In this contribution, we give some simulation results on demodulation performance requirements for NCR-MT ,

# 4 References

- R4-2309832 WF for NCR-MT demodulation requirements. ZTE Corporation.

- R4-2305934 WF for NCR-MT demodulation requirements. ZTE Corporation.

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V17.8.0.

- R4-223505, Revised WID on NR network-controlled repeaters, ZTE, Sanechips, 3GPP TSG RAN Meeting #98-e, Electronic Meeting, December 12-16, 2022.