3GPP TSG-RAN WG4 Meeting#108						R4-2311524

21st Aug, 2023 – 25th Aug, 2023

Source: 	Qualcomm Incorporated

Title: 	NTN NB-IoT/eMTC demodulation performance requirements

Agenda item:	6.8.5.1

Document for:	Approval

- Introduction

In this paper, a minor typo in Test2 of eMTC results is noted and request to capture this in the spec. requirements.

- eMTC Scenarios

Observation1: SNR requirement for Test 2 is incorrectly captured in simulation collection  in RAN#107 [1]

Observation 2: The SNR requirements in TS36.102 is based on the wrong values in [1]

The shared numbers in Table 2.1 are identical to those shared in R4-2307436 [2], RAN4#107, and only the typo in the simulation sheet needs to be corrected.

Table 2.1

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 1.4MHz | 16QAM 1/2 | TM1 | 1 | NTN-TDLC5- 30 | 1x1 | 8ms | 7.1 |
| 2 | FDD 1.4MHz | QPSK 1/3 | TM1 | 8 | NTN-TDLA100-200 | 1x1 | 8ms | -6.3 |
| 3 | FDD 1.4MHz | QPSK 1/10 | TM1 | 64 | NTN-TDLA100-10 | 1x1 | 8ms | -13.5 |

Based on [1] and the correct value in [2], for both alignment and impairments, we revise the SNR value of test 2 for Cat-M1. The final requirement for test 2 is -4.2 dB correspondingly.

Proposal 1: Set the requirement for test2 of Cat-M1 as -4.2dB.

- Conclusions

Observation1: SNR requirement for Test 2 is incorrectly captured in simulation collection  in RAN#107 [1]

Observation 2: The SNR requirements in TS36.102 is based on the wrong values in [1]

Proposal 1: Set the requirement for test2 of Cat-M1 as -4.2dB.

# References

[1] R4-2309872, “Simulation results collection for IoT over NTN UE demod”, MediaTek

[2] R4-2307436, “NTN NB-IoTeMTC demodulation performance requirements”, Qualcomm