3GPP TSG-RAN WG4 Meeting#108					R4-2311525

22nd May, 2023 – 26th May, 2023

Source: 	Qualcomm Incorporated

Title: 	8Rx for CPE/FWA/vehicle/industrial devices: Demodulation requirements

Agenda item:	 8.4.3.1

Document for:	Approval

# 1 Introduction

In this paper, our proposals on the open items mentioned in WF of #107 [1] are shared.

# 2 PDSCH Demodulation Requirements

## 2.1 Propagation conditions

Issue 2-2: MCS for Rank 2 test

- Agree [MCS 20] (Table 2) for this meeting, if any issues are figured out for next meeting, MCS 19 (Table 1) will be selected.

Observation1: From our simulation results, maximum throughput is not achievable for MCS 20, Table2.

As agreed in RAN4 #107, we propose to use only MCS19, Table1 for Rank 2 scenario.

Proposal 1: Rank2: RAN4 to use only MCS19, Table1 with TDLC300-100 Medium B for both TDD and FDD.

For Rank4, we are open to discuss both MCS 17 and MCS 26 options of Table1.

Proposal 2: Rank4: RAN4 to reuse TDD PDSCH demodulation requirement scenarios for FDD too.

# 3 CSI requirements

Issue 4-2: SNR points

- Proposals

  - Option 1: [4,5] dB and [10,11] dB

  - Option 2: [1, 2] dB and [7,8] dB

3.1 SNR points

From simulation studies, we support Option1.

Proposal 3: RAN4 to use (4,5) and (10,11) as SNR points in dB for 4L, 8Rx static channel CQI reporting.

# 5. Conclusions

Following is the summary of the proposals.

Proposal 1: Rank2: RAN4 to use only MCS19, Table1 with TDLC300-100 Medium B for both TDD and FDD.

Proposal 2: Rank4: RAN4 to reuse TDD PDSCH demodulation requirement scenarios for FDD too.

Proposal 3: RAN4 to use (4,5) and (10,11) as SNR points in dB for 4L, 8Rx static channel CQI reporting.

# References

[1] WF R4-2309806, “WF for 8Rx UE performance requirements_final”, 3GPP TSG RAN WG4 meeting, #107, May 21 – May 25, 2023.