3GPP TSG RAN WG4 #108 meeting 	 	R4-2311528

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	8.30.2.1.2 & 8.30.2.1.3

Source:	Meta Ireland

Title:	Consideration on SL-U UE’s remaining RF requirements

Document for:	Approval

- Introduction

In RAN4 #107 meeting, the WF on system parameters [1] is discussed and approved. The following open issues will be further discussed in this meeting to finalize how to capture the SL-U RF requirements in TS38.101-1. Also, the remaining open issues [2][4] for the Tx/Rx requirements for SL-U operation will be further analyzed and decided based on the RAN4 consensus.

- System parameters for SL-U [2]

### : There was no open issues in Rel-18 SL evolution WI

- Tx/Rx Requirements for SL-U [2]

Open issue:

- Maximum output power for SL-U

- TP contents are needed in TR38.786

- Sub-topic 3-2 MPR/A-MPR simulation

- Based on simulation campaign, RAN4 will define the MPR and A-MPR requirements

- Need to down selections to support SL-U in n46/n96/n102

- Issue 4-1-1: REFSENS requirement

- FFS on RB numbers for 100MHz with 60kHz SCS

In this paper, we propose the detailed TP contents on Maximum output power of SL-U UE and propose to down selection to specify the A-MPR requirements with NS signaling in each unlicensed band i.e. n46/n96/n102. Also, we share our preference for the remaining open issues in SL-U operation in Rel-18.

- Discussion on Down selection of supporting A-MPR requirements for SL-U operation in unlicensed bands

After RAN4 #107 meeting, some interested companies discussed how to specify a lot of A-MPR requirements based on different regulatory limits for SL-U operation in unlicensed bands as shown in Table 2-1.

Table 2-1. NS signaling for NR-U UE [7]

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | NS_64 |
| UK | N/A | NS_01 | NS_65 |
| Morocco | N/A | NS_01 | NS_65 |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Kenya | N/A | NS_01 | NS_68 |
| Qatar | N/A | NS_01 | NS_65 |
| Jordan | N/A | NS_01 | NS_65 |
| Russian Federation | N/A | NS_01 | NS_68 |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | [NS_66] |
| Canada | NS_54 | NS_59 | NS_66 |
| Brazil | N/A | NS_53 | NS_67 |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | NS_y2 |
| Colombia | N/A | NS_53 | N/A |
| Dominican Republic | N/A | NS_60 | NS_66 |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | NS_64 |
| Australia | N/A | NS_01 | NS_68 |
| New Zealand | N/A | NS_01 | NS_68 |
| Malaysia | N/A | NS_01 | NS_65 |
| Japan | N/A | NS_63 | NS_69 |

Table 2-2. NS signaling for NR-U UE in TS38.101-1

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  |  |
| n96 | NS_01 | NS_53 | NS_54 | NS_59 | NS_60 | NS_61 |  |  |
| n102 | NS_01 | NS_58 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

In here, we share our preference to specify the A-MPR requirements with NS signaling in Rel-18.

Basically, we propose the priority of the supporting NS signaling according to geographical regions in Rel-18 as follow.

1st Priority to derive A-MPR requirements in Rel-18

- n46: NS_31 (Asia) or NS_29 (Asia)

- n96: NS_53 (US)

- n102: NS_58 (EU)

Proposal #1: The above NS signaling values will be considered to derive A-MPR requirements for SL-U operation in Rel-18.

Also, RAN4 need to consider SCS 60kHz waveform for MPR/A-MPR requirement with contiguous RB allocation only in single CC in Rel-18. The A-MPR requirements for the non-contiguous RB interlace with 60kHz SCS will be supported in future release based on RAN1 supported the interlaced RB allocation in sub-channels.

Proposal #2: The A-MPR simulation assumption with 60kHz SCS waveform can be supported with contiguous RB allocation only in single CC.

- Discussion on remaining RF requirements for NR SL-U in TR38.786

In the section, we propose the detail contents on the Maximum output power requirements and REFSENS RB configuration for NR SL-U UE as follows.

  - Maximum output power of SL-U

===============  Proposed TP contents in Section 6.1 in TR38.786 v0.2.0     ================

- 6.1	Tx requirements for NR SL single carrier operation in unlicensed band

6.1.1	Maximum output power for NR SL-U

The SL-U UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth in unlicensed bands. The period of measurement shall be at least one sub frame (1ms).

Table 6.1.1-1: SL-U UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| n102 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The SL-U UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 in TS38.101-1 when NS is signaled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Proposal #3: The above Maximum output power requirements will be captured in TR38.786 to support PC5 as default power class of SL-U.

  - REFSENS requirements for SL-U with 60kHz SCS

In NR V2X operation, RAN4 defined the RB configuration up to 40MHz CBW. But, the SL-U will support up to 100MHz CBW in unlicensed bands.

So we propose the RB configuration for REFSENS requirements as follow

Table 3-1: Proposed RB configuration for REFSENS requirements for SL-U UE

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |

Specially, the RB configuration of 100MHz CBW with SCS 60kHz proposed with 132 RBs not to use the full RB configuration (135RBs) in our proposal since the full RB configuration for 100MHz CBW will be impacted by edge frequency impact with wide channel bandwidth with high order 60kHz SCS. Therefore, we propose to reduce the RB size with 132 RBs for REFSENS requirements for SL-U UE.

This approach can be covered the edge problem with high order 60kHz SCS in the REFSENS requirements.

Proposal #4: RAN4 need to consider the RB configuration to derive the REFSENS requirements for SL-U UE as shown in Table 3-1.

- Conclusions

In this contribution, we propose the detailed TP contents on Maximum output power of SL-U UE and propose to down selection to specify the A-MPR requirements with NS signaling in the candidate unlicensed bands i.e. n46/n96/n102. Also, we shared our preference for the remaining open issues as follow to complete NR SL evolution WI in Rel-18.

Proposal #1: The following NS_signaling values will be considered to derive A-MPR requirements for SL-U operation in Rel-18.

- n46: NS_31 (Asia) or NS_29 (Asia)

- n96: NS_53 (US)

- n102: NS_58 (EU)

Proposal #2: The A-MPR simulation assumption with 60kHz SCS waveform can be supported with contiguous RB allocation only in single CC.

Proposal #3: The above Maximum output power requirements will be captured in TR38.786 to support PC5 as default power class of SL-U.

Proposal #4: RAN4 need to consider the RB configuration to derive the REFSENS requirements for SL-U UE as shown in Table 3-1.

Reference

[1] R4-2306630, “WF on SLU single CC system parameters, ” vivo

[2] R4-2310310, “WF on SL_enh2_UE RF_part1,” OPPO

[3] R4-2310030, “Topic summary for [107][147] NR_SL_enh2_UERF_part1,” Moderator

[4] RP-230908, “Status Report on NR SL evolution WI in Rel-18,” OPPO

[5] TR38.786 v0.2.0

[6] TR38.785

[7] TR38.849