3GPP TSG RAN WG4 #108 meeting 	 	R4-2311530

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	8.30.2.2

Source:	Meta Ireland

Title:	Remaining RF requirements for inter-band concurrent operation between NR Uu @n78  and SL-U @n46

Document for:	Approval

- Introduction

In RAN4 #107 meeting, the WF on NR SL inter-band concurrent operation between licensed band and unlicensed bands was approved [1]. Based on the updated WF [1], RAN4 needs to complete the remaining Tx/Rx RF requirements to support NR SL inter-band concurrent NR Uu and SL-U operation as follows.

1-2-1: MOP for Uu@Licensed and SL@Un-licensed WF: Further discuss in next meeting with issue 1-2-3 together - Option 1: Define MOP as the total transmitted power from each operating band - Option 2: Define MOP per each operating band in Rel-18 1-2-3: Configured transmitted power for Uu@Licensed and SL@Un-licensed WF: Further discuss in next meeting - Option 1: Apply the principle of the configured transmitted power of NR inter-band CA for the total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR sidelink - PCMAX_L (p,q) = MIN {10log10 [pCMAX_L,c,NR (p)+ pCMAX_L,c,SL (q)], PPowerClass_CA, PEMAX,CA} - PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL (q)], PPowerClass_CA, PEMAX,CA} - Option 2: Consider the maximum total transmit power to be used by the UE across all carriers in Uu and SL-U in FR1 which is indicated by NW (e.g., PEMAX,con-current) - Option 3: Follow the same approach as Rel-17 inter-band concurrent operation 2-1-5: RF requirement impact due to NR 2nd slot power limitation of RAN1 agreement for LTE SL and NR SL co-channel coexistence scenario - Proposals - Option 1 - RAN4 should analyze if additional functionality or requirements are needed to ensure that NR SL UEs follow the behavior as RAN1 agreement. - RAN4 should analyze if this behavior that ensures the co-channel coexistence can impact some other minimum performance requirements. One example here can be the PUMAX,f,c. - In case that clarification in PUMAX,f,c requirement is deemed necessary it could be clarified that PUMAX,f,c tolerances are increased in case of co-channel coexistence or that PUMAX.f.c needs to be measured using the 1st slot only WF: Further discuss in next meeting

In this paper, we propose our preference of the remaining open RF issues based on the existing specification of the legacy inter-band concurrent operation.

- Discussion on inter-band concurrent SL operation

  - Target power class for inter-band concurrent SL operation

In the NR V2X con-current operation, RAN4 defined PC2/PC3 con-current operation as the target power classes between licensed bands and ITS spectrum from Rel-16.

However, RAN4 agreed not to support PC2 @ licensed band at last #107 meeting for the inter-band con-current SL operation. Hence RAN4 only supports the power combinations between PC3 Uu@Licensed + PC5 SL@Unlicensed band in Rel-18.

One of the open issues is how to define the maximum output power for NR SL inter-band con-current operation UE.

In the current 3GPP specification of NR V2X UE in TS38.101, the maximum output power was defined as follow

TS38.101-1 v18.2.0

In the Note 3 in Table 6.2E.1.2-1, we can see the aggregated power applied to the total transmitted power over all component carriers (Per UE).

Also, all power classes of inter-band CA band combinations UE were defined as total aggregated power for the maximum output power of CA UE in licensed bands.

Based on this analysis, we proposed as follow

Proposal #1: RAN4 can support the following power classes as total transmitted power for Maximum output Power of inter-band concurrent UE in Rel-18.

- 1st priority

  - PC3 Uu@Licensed + PC5 SL@Un-licensed

- RAN4 defines MOP per UE as the total transmitted power from each operating band.

  - Configured Tx power for inter-band concurrent operation

#### TS38.101-1 v18.2.0

In NR V2X inter-band concurrent operation, RAN4 defined the configured Tx power [3] as follows based on the configured Tx power for inter-band CA band combinations UE.

So, RAN4 can apply the principle of the configured Tx power of the NR inter-band CA band combinations for inter-band SL-U operation in el-18.

#### 6.2E.xF.x.x	Configured transmitted power for inter-band SL-U concurrent operation

When a UE is configured for simultaneous NR sidelink and NR uplink transmissions for inter-band con-current operation, the UE is allowed to set its configured maximum output power PCMAX,c,NR and PCMAX,c,SL for the configured NR uplink carrier and the configured NR SL carrier, respectively, and its total configured maximum output power PCMAX,c.

The configured maximum output power PCMAX c,NR(p) in slot p for the configured NR uplink carrier shall be set within the bounds:

PCMAX_L,c,NR (p) ≤  PCMAX,c,NR (p) ≤  PCMAX_H,c,NR (p)

where PCMAX_L,c,NR and PCMAX_H,c,NR are the limit as specified in clause 6.2E.4.1.

The configured maximum output power PCMAX c,SL (q) in slot q for the configured NR SL carrier shall be set within the bounds:

PCMAX_L,c,SL (q) ≤ PCMAX,c,SL (q) ≤  PCMAX_H,c,SL (q)

where PCMAX_H,c,SL is the limit as specified in clause 6.2E.4.

The total UE configured maximum output power PCMAX (p,q) in a slot p of NR uplink carrier and a slot q of NR  sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  MIN {10log10 [pCMAX_L,c,NR (p)+ pCMAX_L,c,SL (q)], PPowerClass_CA, PEMAX,CA}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,NR (p) + pCMAX_H,c,SL (q)], PPowerClass_CA, PEMAX,CA}

where pCMAX_L,c,NR and pCMAX_L,c,SL are the respective limits PCMAX_L,c,NR (p) and PCMAX_L,c,SL (q) expressed in linear scale. Also the pCMAX_H,c,NR and pCMAX_H,c,SL are the respective limits PCMAX_H,c,NR (p) and PCMAX_H,c,SL (q) expressed in linear scale.

The measured total maximum output power PUMAX over both the NR uplink and NR SL carriers is

PUMAX = 10 log10 [pUMAX,c,NR + pUMAX,c,SL],

where pUMAX,c,NR  denotes the measured output power of serving cell c for the configured NR uplink carrier, and pUMAX,c,SL  denotes the measured output power for the configured NR SL carrier expressed in linear scale.

When a UE is configured for synchronous NR sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2E.x.x-1. PCMAX_L may be modified for any overlapping portion of slots (p, q) and (p +1, q+1).

Proposal #2: RAN4 need to follow the inter-band CA principle to define the configured Tx power for inter-band con-current SL-U operation UE as follows.

  - Other RF issues for inter-band concurrent operation

In the last RAN4 meeting, RAN4 could not discuss the RAN1 agreements for the NR 2nd slot power limitation in the LTE SL and NR SL co-channel coexistence scenario.

Based on the RAN1 agreements and proponent proposals [5], RAN4 needs to decide whether to define the additional RF requirements or not in Rel-18.

Agreement (RAN1#112bis-e) For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. - Note: How to ensure the above condition is up to UE implementation - FFS on whether same or different frequency allocation may be used in the second overlapping slot

Analysis figures from [5], we can see the impact in LTE UE3 Rx performance by the higher power at 2nd slot of NR V2X UE in figure (A) and (B) in 2.3.1.

Figure 2.3-1 AGC impact on the co-channel operation with LTE SL + NR SL (Case 2)

This problem can be solved by UE implementation to restrict the 2nd slot power set to equal or less than 1st slot of NR V2X UE.

Also, the configured Tx power is not impacted since all of Tx power was considered as maximum output power to verify the conformance test. So the Pumax equation will cover the available range of  the NR V2X UE. Based on above analysis, we propose as follow

Proposal #3: RAN4 can solve the RAN1 agreement as UE implementation and the configured Tx power for inter-band con-current operation UE do not have any impact by the RAN1 agreements.

- Conclusions

In this contribution, we provided our view on the remaining RF requirements for NR SL inter-band concurrent operation between n78@licensed band and n46@unlicensed band UE. Based on our analysis, we propose the following.

Proposal #1: RAN4 can support the following power classes as total transmitted power for Maximum output Power of inter-band concurrent UE in Rel-18.

- 1st priority

  - PC3 Uu@Licensed + PC5 SL@Un-licensed

- RAN4 defines MOP per UE as the total transmitted power from each operating band.

Proposal #2: RAN4 needs to follow the inter-band CA principle to define the configured Tx power for inter-band con-current SL-U operation UE as follows section 2.2.

Proposal #3: RAN4 can solve the RAN1 agreement as UE implementation and the configured Tx power for inter-band con-current operation UE do not have any impact by the RAN1 agreements.

Reference

[1] RP-230077, “WID revision: NR sidelink evolution,” OPPO

[2] R4-2310313, “WF on NR_SL_enh2_UERF_part2,” LGE

[3] TR38.785

[4] TR38.886

[5] R4-2308416, “On Co-channel coexistence for LTE SL and NR SL,” LGE