3GPP TSG-RAN WG4 #108 Meeting 	R4- 2311532

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	8.30.2.1.2

Title:	TP for TR 38.786 on the remaining UE RF requirements for SL-U UE

Source: 	Meta Ireland

Document for:	Approval

Background

This contribution provides text proposal on the Maximum output power requirements for SL-U operation in unlicensed band based on the agreed WF [1].

Text Proposal

<<<<<<<<<<< Start of changes in section 6 >>>>>>>>>>

6	Transmitter characteristics for NR SL evolution

6.1	Tx requirements for NR SL single carrier operation in unlicensed band

6.1.1	Maximum output power for NR SL-U

The SL-U UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth in unlicensed bands. The period of measurement shall be at least one sub frame (1ms).

Table 6.1.1-1: SL-U UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| n102 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The SL-U UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 in TS38.101-1 when NS is signalled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

<<<<<<<<<<< End of changes in section 6 >>>>>>>>>>

References

- R4-2310314, “WF on NR_SL_enh2_UERF_part1,” OPPO

- R4-2308959, “TR38.786 v0.2.0,” OPPO

- RP-230077, “Revised WID on NR sidelink evolution,” OPPO