3GPP TSG-RAN WG4 Meeting #108 	R4-2311560

Toulouse, France, August 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of EMC core requirements

Agenda item:	8.28.3 – EMC core requirements

Document for:	Approval

Introduction

Agreements and open issues from RAN4#107 summarized in Way Forward document [1]:

Agreement 1 in [1] was to extend the scope of specification TS38.114 to include NCR type 1-H and 1-O. TS38.113 and Figures 1-3 in R4-2308626 [2] can be used as a reference and starting point. Figure 1. Conducted reference points for NCR type 1-C Figure 2. Radiated and conducted reference points for NCR type 1-H Figure 3. Radiated reference points for NCR type 1-O and 2-O

This contribution discusses the implications of updates to the NCR diagrams proposed in [3] for the EMC core requirements.

Discussion

Updates to RF diagrams for NCR

The system's main functions and characteristics must be considered in the planning of EMC testing. RF diagrams define the main elements of the NCR, so proposals from [3] to update the RF diagrams shown in Figures 1-3 [2] need to be considered from an EMC point of view. In relation to the EMC core requirements agenda item of meeting [1], it was commented that the definition needs to reflect the fact that the NCR is a system whose parts, Fwd and MT, can be placed separately. The diagrams have been updated to reflect this feedback. The updated diagrams (Figures 4-6) include interfaces between NCR-Fwd and NCR-MT. However, it is important to emphasize that the NCR is a system that requires both parts, NCR-Fwd and NCR-MT.

Figure 4. Updated NCR type 1-C diagram

Figure 5. Updated NCR type 1-H diagram

Figure 6. Updated NCR type 1-O and 2-O diagram

From EMC perspective, it is important emphasize that the NCR is a single system, one functional entity, which consists of two functional parts, Fwd and MT, which can be placed separately for implementation. The RF diagrams of the NCR system with its parts, Fwd and MT, and the interface between them to allow the option of separate placement during implementation, are shown in Figures 4-6.

Updates to the RF diagrams for NCR in Figures 4-6 have been suggested for consideration.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: From EMC perspective, it is important emphasize that the NCR is a single system, one functional entity, which consisting of two functional parts, Fwd and MT, which can be placed separately for implementation. The RF diagrams of the NCR system with its parts, Fwd and MT, and the interface between them to allow the option of separate placement during implementation, are shown in Figures 4-6.

Proposal 1: Updates to the RF diagrams for NCR in Figures 4-6 have been suggested for consideration.

- References

- R4-2309784, WF for EMC requirements, ZTE, RAN4 #107, Incheon, South Korea, May 22 – 26, 2023.

- R4-2308626, Discussion on EMC requirement for NCR, Nokia, Nokia Shanghai Bell, RAN4 #107, Incheon, South Korea, May 22 – 26, 2023.

- R4-2311561, Discussion of updating RF diagrams, Nokia, Nokia Shanghai Bell, RAN4 #108, Toulouse, France, August 21 – 25, 2023