3GPP TSG-RAN WG4 Meeting #108 	R4-2311562

Toulouse, France, August 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of Spurious Emissions requirements for NCR-Fwd

Agenda item:	8.28.2.1 – RF requirements for NCR-Fwd

Document for:	Approval

Introduction

For simultaneous and non-simultaneous transmission of NCR-MT and NCR-Fwd part in the uplink direction: - Only one limit should be defined; FFS to reuse UE spurious emission or repeater spurious emission requirement;

The WF of the RAN4#107 meeting [1] stated the following regarding the RF requirements for NCR-Fwd agenda item:

This paper discusses the open issues regarding the spurious emission requirements for NCR-Fwd.

Discussion

Spurious emission requirement

At the RAN4#107 meeting it was agreed to further study to reuse UE spurious emissions or repeater spurious emission requirements specified in in TS 38.106 [2].

Repeater core specification TS 38.106 specifies spurious emission requirements for repeater types 1-C and type 2-O. Based on initial studies, the functionalities of repeater types 1-C and type 2-O can be considered similar to the corresponding NCR (Network Controlled Repeater) types from an RF point of view. Repeater requirements can be reused for NCR type 1-C and type 2-O. IAB core specification TS38.174 [3] specifies similar requirements for spurious emissions for IAB type 1-C between 1-H, and 2-O between 1-O. New requirements for 1-H and 1-O can be applied according to IAB requirements.

Repeater requirements can be applied for NCR. Additional requirements needed for mixed type NCRs can be made according to the IAB specification.

Spurious emission requirements defined in TS 38.106 can be reused for NCR.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: Repeater requirements can be applied for NCR. Additional requirements needed for mix type NCRs can be made according to the IAB specification.

Proposal 1: Spurious emission requirements defined in TS 38.106 can be reused for NCR.

- References

- R4-2309893, WF on system parameters and RF requirements, ZTE, RAN4 #107, Incheon, South Korea, May 22 – 26, 2023.

- TS 38.106, NR; NR Repeater Radio Transmission and Reception.

- TS 38.174 NR; Integrated Access and Backhaul (IAB) radio transmission and reception.