3GPP TSG-RAN WG4 Meeting #108 	R4-2311563

Toulouse, France, August 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of RF requirement for NCR-MT

Agenda item:	8.28.2.2 - RF requirement for NCR-MT

Document for:	Approval

Introduction

Agreements and open issues from last RAN4#107 are summarized in Way Forward document [1].

Issue 3-1-2 Transmitter ON-OFF power and transition period - Agreement: Further discuss below two options: - Option 1: IAB-MT approach - Option 2: Legacy UE approach Issue 3-1-5 OBUE/SEM - Agreement: - For wide area NCR-MT: BS requirements applied - For local area NCR-MT: FFS - Proposal 3 agreed - Proposal 3 If the NCR supports simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements should be defined on the total emissions from MT and FWD. Issue 3-1-7 Transmitter intermodulation - Agreement: - Further discuss below two options for NCR-MT: - Option 1: IAB-MT approach - Option 2: Legacy UE approach

Issue 3-2-1 REFSENS requirement - Agreement: - NF is assumed as 5dB for FR1 WA NCR-MT, - FR1 LA NCR-MT: - Option1: 13dB - Option 2: legacy UE value - NF is assumed as 10dB for 30GHz and 12dB for 45GHz for FR2 NCR-MT Issue 3-2-3 ACS/IBB - Agreement: - For wide area NCR-MT: reusing IAB-MT requirements - For local are NCR-MT: FFS Issue 3-2-4 OOBB requirement - Agreement: - For wide area NCR-MT: reusing IAB-MT requirements - For local are NCR-MT: FFS Issue 3-2-6 receiver intermodulation requirement - Agreement: - For wide area NCR-MT: reusing IAB-MT requirements - For local are NCR-MT: FFS

This contribution discusses the open issues regarding the RF requirements for NCR-MT.

Discussion

At the RAN4#107 meeting it was agreed to further study the RF requirements for NCR-MT.

NR repeater specification [2] defines uplink and downlink operations, interfaces, and requirements. NR NCR has similar functionality to repeaters in its element referred to as Forwarding or NCR-Fwd. In parallel to Fwd, Control Link and NCR-MT operations have been specified for Release 18 NCR. NCR-MT operations are similar to IAB-MT operations [3]. Therefore, the main RF characteristics of IAB-MT can be applied to NCR-MT (Table 1).

Table 1. NCR-MT RF requirements and applicability notes

| RF requirements | Applicability notes |
| --- | --- |
| Transmitter ON-OFF power and transition period | Option 1: IAB-MT approach |
| OBUE/SEM: For Local area NCR-MT | Re-use same approach as for Local Area IAB-MT (TS 38.174 clause 6.6.4) |
| Transmitter intermodulation | Option 1: IAB-MT approach. |
| REFSENS requirement | Option1: 13dB |
| ACS/IBB: For Local area NCR-MT | Re-use same approach as for Local Area IAB-MT (TS 38.174 clause 7.4.1) |
| OOBB requirement: For Local area NCR-MT | Reuse same approach as for out-of-band blocking requirements of IAB-MT (TS 38.174 clause 10.6) |
| Receiver intermodulation requirement: For Local area NCR-MT | Re-use same approach as for Local Area IAB-MT (TS 38.174 clause 7.7 and 10.8) |

The operation of the NR network-controlled repeater MT part is similar to IAB-MT. The same requirements can be applied to NCR-MT as to IAB-MT.

It is proposed to reuse IAB-MT RF requirements for NCR-MT.

Conclusion

In this contribution, the following Observations and Proposals were made:

Observation 1: The operation of the NR network-controlled repeater MT part is similar to IAB-MT. The same requirements can be applied to NCR-MT as to IAB-MT.

Proposal 1: It is proposed to reusing IAB-MT RF requirements for NCR-MT.

- References

- R4-2309893, WF on system parameters and RF requirements, ZTE, RAN4 #107, Incheon, South Korea, May 22 – 26, 2023.

- TS 38.106, NR; NR Repeater Radio Transmission and Reception.

- TS 38.174 NR; Integrated Access and Backhaul (IAB) radio transmission and reception.