3GPP TSG-RAN WG4 Meeting #108                                                                                         R4-2311604

Toulouse, France, August 21 – August 25, 2023

Source: 	CATT

Title: 	Discussion on 30 MHz Channel Bandwidth for NR NTN UE requirement in FR1

Agenda item:	7.34.2

Document for:	Approval

- Introduction

In RAN#100 meeting, new WID on 30 MHz Channel Bandwidth for NR NTN in FR1 was approved. The main objectives in WID are shown as follows:

The objective of the core part is to: - Define 30 MHz channel bandwidth for n256 and n255 defined for 5G NR-NTN; - Introduce the corresponding SAN and UE RF core requirements, limited to clause 5 (Operating Bands and Channel Arrangement) requirements for these specifications. The specified requirements will be introduced in a REL-independent way starting with REL-17 Note: Other Tx and Rx requirements for UE and SAN RF specifications will not be defined in this WI. The fact that this is a limited set of requirements should be reflected explicitly in the technical specification.

In this contribution, we provide our views on how to introduce the 30MHz channel bandwidth for NR NTN UE requirement in FR1.

- Discussion

According to WID, the requirements for 30MHz was limited in clause 5 (Operating Bands and Channel Arrangement) requirements. So the following sub-clauses in clause 5 of TS 38.101-5 need to be updated to introduce 30 MHz channel bandwidth.

Transmission bandwidth configuration in sub-clause 5.3.2 of TS 38.101-5

Transmission bandwidth configuration NRB for FR1 for 30MHz in Table 5.3.2-1 of TS 38.101-1 can be reused for NTN, and a note needs to be added in Table 5.3.2-1 of TS 38.101-5 for 30MHz to indicated that the requirement for 30MHz is limited in clause 5, and other Tx and Rx requirements for 30MHz in this specification are not be defined. The corresponding updates in Table 5.3.2-1 of TS 38.101-5 are shown as follows:

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz（Note 1） |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

NOTE 1:  The requirement for 30MHz is limited in clause 5, and other Tx and Rx requirements for 30MHz in this specification are not be defined

Proposal 1: To update Table 5.3.2-1 of TS 38.101-5 for 30MHz (green shadow) as follows:

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz（Note 1） |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

NOTE 1:  The requirement for 30MHz is limited in clause 5, and other Tx and Rx requirements for 30MHz in this specification are not be defined

Minimum guardband and transmission bandwidth configuration in sub-clause 5.3.3 of TS 38.101-5

Minimum guardband (kHz) (FR1) for 30MHz in Table 5.3.3-1 of TS 38.101-1 can be reused for NTN. The corresponding updates in Table 5.3.3-1 of TS 38.101-5 are shown as follows:

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | 1010 | 990 | 1330 | 1290 |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1.

Proposal 2: To update Table 5.3.3-1 of TS 38.101-5 for 30MHz (green shadow) as follows:

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | 1010 | 990 | 1330 | 1290 |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1.

UE channel bandwidth per operating band in sub-clause 5.3.5 of TS 38.101-5

The 30MHz for 15, 30, and 60kHz SCS for operating band n256 and n255 need to be supported and updated to Table 5.3.5-1 of TS 38.101-5. The corresponding updates in 5.3.5-1 of TS 38.101-5 are shown as follows:

Table 5.3.5-1: Channel bandwidths for each NTN satellite band

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 |  |
|  | 15 | 5 | 10 | 15 | 20 | 30 |  |
| n256 | 30 |  | 10 | 15 | 20 | 30 |  |
|  | 60 |  | 10 | 15 | 20 | 30 |  |
|  | 15 | 5 | 10 | 15 | 20 | 30 |  |
| n255 | 30 |  | 10 | 15 | 20 | 30 |  |
|  | 60 |  | 10 | 15 | 20 | 30 |  |

Proposal 3: To update Table 5.3.5-1 of TS 38.101-5 for 30MHz (green shadow) as follows:

Table 5.3.5-1: Channel bandwidths for each NTN satellite band

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n256 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n255 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |

- Conclusion

This contribution provides our general consideration on 30 MHz Channel Bandwidth for NR NTN UE requirement in FR1. The following observations and proposals are concluded as follows:

Proposal 1: To update Table 5.3.2-1 of TS 38.101-5 for 30MHz (green shadow) as follows:

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz（Note 1） |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

NOTE 1:  The requirement for 30MHz is limited in clause 5, and other Tx and Rx requirements for 30MHz in this specification are not be defined

Proposal 2: To update Table 5.3.3-1 of TS 38.101-5 for 30MHz (green shadow) as follows:

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | 1010 | 990 | 1330 | 1290 |

NOTE:	The minimum guardbands have been calculated using the following equation: (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1.

Proposal 3: To update Table 5.3.5-1 of TS 38.101-5 for 30MHz (green shadow) as follows:

Table 5.3.5-1: Channel bandwidths for each NTN satellite band

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n256 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |
|  | 15 | 5 | 10 | 15 | 20 | 30 |
| n255 | 30 |  | 10 | 15 | 20 | 30 |
|  | 60 |  | 10 | 15 | 20 | 30 |

- References

- RP-231499, 30 MHz Channel Bandwidth for NR NTN in FR1, THALES, RAN#100