3GPP TSG-RAN4 Meeting #108	                                                            R4-2311614

Toulouse, France, August 21 – August 25, 2023

Title:	Discussion on the RRM impact of option A, B-1-1, B-1-2 and C for the support for bandwidth part operation without restriction

Source:	CATT

Agenda Item:	8.10.2

Document for:	Discussion

1 Introduction

In last meeting, the WI to complete the specification support for BandWidth Part operation without restriction in NR was discussed and the conclusions were captured in the approved WF [1].

This contribution further discusses the RRM requirements and specification support for the above options and presents our understandings and proposals.

2 Discussion

In last meeting, the handling of L3 measurement requirements impact was discussed, and a LS [3] was sent to RANP to decide the handling of L3 measurement. After the discussion in RAN#100, the WID [4] was updated to include the objective for L3 measurement for option B-1-1 and option C. So the L3 measurement requirements for B-1-1 and option C need to be defined.

Generally, when UE supports option B-1-1 to perform L1 measurement without gap when SSB is outside the active BWP, UE should also be able to perform the intra-frequency measurement without gap and the existing requirements apply. But based on the updated WID, UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. For such case, the existing L3 measurement requirements with gap apply.

Similarly for option C, the intra-frequency L3 measurement can be based on NCD-SSB. And the requirements defined in eRedCap for NCD-SSB can apply.

Proposal 1: For UE supporting option B-1-1, by default, the L3 measurement requirements without gap apply.

Proposal 2: For UE supporting option C, the L3 measurement requirements with NCD-SSB defined in Rel-17 can be applied.

For handover requirements, there were proposal as below to support handover to a BWP without CD-SSB directly for UE supporting option A, B-1-1 and option B-1-2 and to support handover to a BWP with NCD-SSB directly for UE supporting option C.

#### Issue 1-2: Requirements for handover for Option A, C, B-1-1 and B-1-2 - Option 1: - It is beneficial to support handover to a BWP without CD-SSB directly for UE supporting option A, B-1-1 and option B-1-2. - It is beneficial to support handover to a BWP with NCD-SSB directly for UE supporting option C. - Option 2: - Subject to RANP approval, RAN4 to support HO scenarios involving NCD-SSB and re-use the same requirements from RedCap. - Option 3: - Other options are not precluded.

For the first proposal to handover to a BWP without CD-SSB, we think some signaling procedure need to be clarified and much RAN2 work is needed. But for handover to a BWP with NCD-SSB, since the procedure has already been defined for RedCap, it would be simple to extend it to non-RedCap UE. So we think the handover based on option C can be defined in Rel-18 but the other options can be postponed to future release.

Proposal 3: RAN4 to define requirements for handover to a BWP with NCD-SSB directly for UE supporting option C.

Based on the discussion, all the options are optional capability and can be reported independently. So there were proposal on the requirements/UE behaviour for UE supporting multiple options. But we understand to support the BWP operation without restriction, one of the options is enough. And even if multiple options are supported, the measurement should be based on network configuration. We think it is not typical for NW to configuration multiple options, so there is no need to define requirements for supporting multiple options.

Proposal 4: No need to define requirements for supporting multiple options.

It was agreed in last meeting that the existing requirements for SSB-based RLM/BFD/BM can be reused for option A/B-1-1/ C. And the possible clarification is FFS. Based on our discussion in last meeting, we think no additional clarifications are needed for option A and B-1-1. For option C, it was agreed that the current SSB-based timing requirement can be also applied to non-RedCap UE with NCD-CCB support and no further clarification is needed.

Proposal 5: No additional clarification is needed for timing requirements for option A, B-1-1 and C.

For option B-1-2, the measurement is performed without gap but interruption is needed. For the measurement requirements, the same conclusions as option B-1-1 can be used. For interruption requirements, similar as that for other measurements in existing specification, the interruption for option B-1-2 can also be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate. And the existing interruption ratio in the specification for other measurements can be reused (i.e. missed ACK/NACK is up to 0.5%). And the length for each interruption should be defined based on RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

Proposal 6: For option B-1-2, existing SSB based BM/RLM/BFD measurement requirements without gap can apply.

Proposal 7: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. And the length for each interruption shall not exceed the RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

3 Summary

This contribution discusses the RRM requirements and specification support for option A, B-1-1, B-1-2 and C for the support for bandwidth part operation without restriction, and the followings are proposed:

Proposal 1: For UE supporting option B-1-1, by default, the L3 measurement requirements without gap apply.

Proposal 2: For UE supporting option C, the L3 measurement requirements with NCD-SSB defined in Rel-17 can be applied.

Proposal 3: RAN4 to define requirements for handover to a BWP with NCD-SSB directly for UE supporting option C.

Proposal 4: No need to define requirements for supporting multiple options.

Proposal 5: No additional clarification is needed for timing requirements for option A, B-1-1 and C.

Proposal 6: For option B-1-2, existing SSB based BM/RLM/BFD measurement requirements without gap can apply.

Proposal 7: For option B-1-2, the interruption requirements can be defined based on HARQ ACK/NACK loss framework with a maximum missed ACK/NACK rate up to [0.5%]. And the length for each interruption shall not exceed the RF retuning time (0.5ms for FR1 and 0.25ms for FR2).

4 References

- R4-2310176 WF on NR BWP operation without restriction, vivo, RAN4#107

- R4-2309958 Topic summary for [107][213] NR_BWP_wor, vivo, RAN4#107

- R4-2310177 LS on L3 related requirements for the options for BWP operation without restriction, vivo, Vodafone

- RP-231486 Revised WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo, RAN#100