3GPP TSG-RAN WG4 Meeting # 108				                                           R4-2311622

Toulouse, France, August 21 – August 25, 2023

Title:	Discussion on RRM requirements for NTN-TN and NTN-NTN mobility and service continuity enhancements

Source:	CATT

Agenda Item:	8.26.5.3

Document for:	Discussion

1 Introduction

In last meeting, the RRM requirements for NR NTN enhancement were discussed and the conclusions were captured in the approved WF [1]. Most of the issues are still open. In this paper, we have some further discussions on the possible RRM impact based on the agreed WF and give our views.

2 Discussion

2.1 NTN-TN and NTN-NTN cell reselection

For location-based cell reselection in earth moving cell NTN deployments, there was the following agreement in last meeting:

Issue 4-1-B: For location-based cell reselection in earth moving cell NTN deployments Agreement: - RAN4 to define UE requirements on location-based cell reselection in earth moving cell NTN deployments if the existing requirements are not applicable for earth moving system. The new requirement can be based on the existing requirement, and the updates may include “the margin for distance threshold” and “the definition of the reference Location.” - Do not further restrict DRX cycle beyond Rel-17 NR NTN. - FFS: Whether the coverage information of serving cell is (absolutely) necessary

The controversial part is whether the coverage information of serving cell is necessary. Based on the RAN2 agreements listed in the Annex, the same format of Rel-17 referenceLocation and distanceThresh is used for signaling the TN coverage area centre and radius and RAN2 would not introduce RRC dedicated signalling to provide more accurate TN coverage information. We understand RAN2 has made the conclusion that no further TN coverage information is needed. On the other hand, with the distance threshold and reference location of serving cell provided by the network, the cell selection can already be determined and performed with location-based criteria. There is no need to further provide serving cell coverage which may not bring additional gain to the cell reselection requirement.

Also RAN2 agreed that an RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for cell reselection for a TN frequency in the area, if configured, where there is no coverage of that frequency regardless of the frequency priority. The frequency coverage information is still discussed in RAN2 and we can wait for the conclusions. After defined by RAN2, the clarification on the frequency coverage of neighbor cell can be included in the cell reselection requirement.

Proposal 1: For location-based cell reselection, serving cell coverage information is not necessary.

Proposal 2: RAN4 to add the clarification on the frequency coverage of neighbor cell in the cell reselection after RAN2 concludes it.

For NTN-TN cell reselection enhancement, there is the following agreement in last RAN4 meeting.

### Issue 4-2: NTN-TN Cell reselection enhancements (to reduce UE power consumption) Agreement: - FFS: RAN4 to define RRM requirements on TN-to-NTN reselection. - RAN4 to agree to reuse same side condition of assistance information for neighbour NTN cells from R17 WF R4-2207114. - Additional side conditions can be added, as needed, and the conditions may depend on satellite types (GSO vs. non-GSO) and/or NTN cell deployments for non-GSO (earth-fixed vs. earth-moving cell). Agreement: - RAN4 to define RRM requirements on NTN-to-TN cell reselection requirement without measurement enhancement for the following cases: - both GSO and non-GSO - both earth fixed cell and earth moving cell - FFS on - whether/how to define the detailed UE behaviour when the UE is out of TN coverage. Agreement: - RAN4 to not define accuracy of TN coverage information provided by NTN cell.

For the direction of cell reselection, in last meeting, it was agreed to define NTN-to-TN cell reselection. When the TN coverage information is provided, UE is not required to perform measurement when the UE is out of TN coverage.

For the cell reselection requirements from TN to NTN, we think it can also be defined. And the assistance information of NTN cell can be provided to UE. The side condition in Rel-17 with the required target satellite information for measurement and mobility can be used as the starting point.

Proposal 3: For NTN-to-TN cell reselection, UE is not required to perform measurement when the UE is out of TN coverage.

Proposal 4: Define requirements for TN-to-NTN cell reselection. And the side condition of assistance information in Rel-17 with the listed parameters of target satellite can be used as the starting point.

2.2 NTN-NTN handover

In last meeting, the following agreement was reached for RACH-less HO.

### Issue 5-1: RACH-less (C)HO Agreement: - RAN4 to define RACH-less HO requirements for intra-/inter-satellite handover with and without gateway/gNB switch. - Known and Unknown target cell condition: - Reuse the R17 known/unknown condition applied in NTN HO requirements. - If any other new aspects to be additionally added are identified based on RAN1/2 progress, those aspects will be further discussed in RAN4. - Whether to define the requirements for both known and unknown target cells - Consider both known and unknown target cells. - Whether to include fine time tracking latency in the HO latency requirement, - Fine time tracking latency will be included even when target cell is known. - Additional latency can be added, if identified and needed, based on the detailed design of the feature to be made by RAN1 and RAN2 - The detailed formula and exact values in the requirements, e.g. T_IU, T_SI-NR, T_RACH, T_delta, T_processing, T_margin, T_search, etc. if relevant, will be defined in the next meeting accordingly.

For the delay requirements for RACH-less handover, the legacy NTN handover requirements can be used as baseline, i.e., Dhandover =TRRC + Tsearch + TIU + Tprocessing  + T∆ + Tmargin. For RACH-less handover, the ending point should be redefined as the first UL transmission on the target cell and TIU is the uncertainty for acquiring the first UL reporting resource.

Proposal 5: The delay requirement for RACH-less HO is defined as Dhandover =TRRC + Tsearch + TIU + Tprocessing  + T∆ + Tmargin, where  TIU is the uncertainty for acquiring the first UL reporting resource and the other components are same as the legacy NTN handover.

Based on RAN2 agreements in the Annex, group handover is not pursued in Rel-18 and RAN4 don’t need to further discuss.

Proposal 6: Group handover is not supported in Rel-18 based on RAN2 conclusion.

3 Summary

In this paper, we have some discussions on possible RRM impact for NTN enhancement, the following observations and proposals are given:

Proposal 1: For location-based cell reselection, serving cell coverage information is not necessary.

Proposal 2: RAN4 to add the clarification on the frequency coverage of neighbor cell in the cell reselection after RAN2 concludes it.

Proposal 3: For NTN-to-TN cell reselection, UE is not required to perform measurement when the UE is out of TN coverage.

Proposal 4: Define requirements for TN-to-NTN cell reselection. And the side condition of assistance information in Rel-17 with the listed parameters of target satellite can be used as the starting point.

Proposal 5: The delay requirement for RACH-less HO is defined as Dhandover =TRRC + Tsearch + TIU + Tprocessing  + T∆ + Tmargin, where  TIU is the uncertainty for acquiring the first UL reporting resource and the other components are same as the legacy NTN handover.

Proposal 6: Group handover is not supported in Rel-18 based on RAN2 conclusion.

4 References

- R4-2310168 WF on NR NTN enhancements RRM requirements, Qualcomm, RAN4#107

- R4-2309971 Topic summary for [107][226] NR_NTN_enh, Qualcomm, RAN4#107

Annex RAN2#122 agreements on NTN enhancement

NTN-TN cell reselection enhancement

Agreements: 1. An RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for cell reselection for a TN frequency in the area, if configured, where there is no coverage of that frequency, regardless of the frequency priority 2. Reuse the same format of Rel-17 referenceLocation and distanceThresh for signaling the TN coverage area centre and radius 3. TN coverage info is NOT included in SIB19. FFS if we use an existing SIB or a new one 4. We don’t introduce RRC dedicated signalling to provide more accurate TN coverage information

NTN-NTN cell reselection

1. Re-use epochTime-r17 in ntn-Config IE to provide the time reference for an Earth moving cell reference location. 2. Re-use t-Service-r17 format for the IE used to trigger UE neighbour cell measurements prior to cell replacement due to feeder link switch. FFS whether we reuse exactly the same IE name as in R17 (updating the field description) or a new one Agreements: 1. Location-based cell reselection criteria are not pursued in R18.

NTN-NTN handover

Agreements:  Come back to the proposal to broadcast the target cell’s servingCellConfigCommon (as common (C)HO signalling) after feedback from RAN3  Send al LS to RAN3 asking whether, in case target cell’s servingCellConfigCommon is broadcast in the source cell (as common (C)HO signalling), the target cell’s servingCellConfigCommon can be transferred to the source cell in the inter-gNB HO case in R18  Group handover related to P1~P4 from R2-2304736 is not supported in Rel-18. Agreements: In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0 Agreements: 1. From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO 2. Release pre-allocated UL grant after RACH-less HO completion 3. LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI 4. Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any) Agreements:  t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens  In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).