# 3GPP TSG-RAN4 Meeting #108	                                                            R4-2311626

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3438 | rev | - | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on R16 positioning performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2023-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The NR operating band groups and minimum Io conditions in positioning accuracy requirements are not aligned with B.2.14. - The same corrections for Rel-17 (R4-2310123, R4-2310129) and Rel-18 (R4-2307425, R4-2308654) have been agreed in RAN4#107 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the NR operating band groups and minimum Io conditions in positioning accuracy requirements to align with B.2.14. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for PRS measurements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23.2, 10.1.24.2, 10.1.25.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.1.23.2	Measurement Accuracy Requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y+Z+Δ) Tc.

X is defined in Table 10.1.23.2-1 for AWGN channel and Table 10.1.23.2-3 for fading channel for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

X is defined in Table 10.1.23.2-2 for AWGN channel and Table 10.1.23.2-4 for fading channel for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

Note: The requriements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

When UE measures RSTD on PRS resources belonging to different PFLs, then the RSTD accuracy is defined as the accuracy corresponding to the largest accuracy value among different PFLs.

When UE measures RSTD on PRS resources belonging to same PFL, Y=32 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 160 ms.

When UE measures RSTD on PRS resources belonging different PFLs, Y=256 Tc, provided that the time offset between the two PRS resource instances from the reference cell and the neighbor cell, which are used for a single RSTD estimate, is no greater than 1280 ms.

Z is defined in Table 10.1.23.2-5 for FR1 and Table 10.1.23.2-6 for FR2, respectively.

Δ is zero for single PFL, and is defined in Table 10.1.23.2-5a for FR1 and Table 10.1.23.2-6a for FR2, respectively, for dual PFL.

Table 10.1.23.2-1: RSTD absolute accuracy in FR1 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 132 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥24 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
| 98 |  |  | ≥52 | ≥1 | Note 6 | Note 6 | Note 6 |
| 42 |  |  | ≥104 | ≥1 | Note 6 | Note 6 | Note 6 |
| 75 |  | 30 | ≥24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
| 48 |  |  | ≥ 48 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 24 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 50 |  | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| 24 |  |  | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 10 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Void |  |  |  |  |  |  |  |

Table 10.1.23.2-2: RSTD absolute accuracy in FR2 for AWGN channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 35 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 24 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 11 |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 24 |  | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 13 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 6 |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Void |  |  |  |  |  |  |

Table 10.1.23.2-3: RSTD absolute accuracy in FR1 for fading channel

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| 247 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
| 140 |  |  | ≥ 52 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 86 |  |  | ≥ 104 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 118 |  | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
| 109 |  |  | ≥ 48 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 28 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 147 |  | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| 27 |  |  | ≥ 64 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| 21 |  |  | ≥ 132 | ≥ 1 | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Void |  |  |  |  |  |  |  |

Table 10.1.23.2-4:  RSTD absolute accuracy in FR2 for fading channel

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 83 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 64 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 46 |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 48 |  | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 54 |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 36 |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Void |  |  |  |  |  |  |

Table 10.1.23.2-5: Margin for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 120 |
| ≥ 52 | ≥ 24 | N/A | 72 |
| ≥ 104 | ≥ 48 | ≥ 24 | 36 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 12 |

Table 10.1.23.2-5a: Margin Δ for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 128 |
| ≥ 52 | ≥ 24 | N/A | 64 |
| ≥ 104 | ≥ 48 | ≥ 24 | 32 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 8 |

Table 10.1.23.2-6: Margin for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 72 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 16 |
| N/A | ≥ 128 | 12 |

Table 10.1.23.2-6a: Margin Δ for RSTD measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (Tc) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | 32 |
| ≥ 64 | ≥ 32 | 16 |
| ≥ 132 | ≥ 64 | 8 |
| N/A | ≥ 128 | 4 |

<End of Change 1>

<Start of Change 2>

10.1.24.2	Measurement Accuracy Requirements

10.1.24.2.1 Absolute PRS RSRP accuracy

The absolute accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.1-1 are valid under the following conditions:

Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

The absolute accuracy requirements for PRS-RSRP measurement for FR2 defined in Table 10.1.24.2.1-2 are valid under the following conditions:

Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

Table 10.1.24.2.1-1: PRS-RSRP absolute accuracy for FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±3.5 | ±8 | ≥-3dB | ≥24 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±8.5 | ±13 | ≥-13dB | 24 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±6 | ±10.5 |  | 52< BW≤ 104 | All | Note 4 |  |  |  |  |
| ±4.5 | ±9 |  | BW >104 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.24.2.1-2: PRS-RSRP absolute accuracy for FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ±5 | ±8 | ≥-3dB | ≥24 | All | Same value as PRP in Table B.2.14 -2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±8.5 | ±11.5 | ≥-13dB | 24 ≤ BW ≤ 64 | All | Note 4 |  |  |
| ±6 | ±9 |  | BW >64 | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

##### 10.1.24.2.2 Relative PRS RSRP accuracy

The relative accuracy of PRS-RSRP is defined as accuracy of the difference between two PRS-RSRP measurements.

The relative PRS-RSRP accuracy requirements apply for the cases when PRS-RSRP is measured from PRS resources in the same PRS resource set in FR1 or FR2, and measured with same Rx beam in case of FR2.

The accuracy requirements for PRS-RSRP measurement for FR1 defined in Table 10.1.24.2.2-1 are valid under the following conditions:

Conditions defined in 38.101-1 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

The accuracy requirements for PRS-RSRP measurement for FR2 defined in Table 10.1.24.2.2-2 are valid under the following conditions:

Conditions defined in 38.101-2 Clause 7.3 for reference sensitivity are fulfilled.

PRP 1,2|dBm according to Annex B.2.14 for a corresponding Band

Table 10.1.24.2.2-1: PRS-RSRP relative accuracy for FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 8 | Minimum Io Note 1 dBm / SCSPRS |  |  | Maximum Io |
| dB | dB | dB | PRB | - |  | dBm / SCSPRS |  |  | dBm/BWChannel |
|  |  |  |  |  |  | dBm/15kHz Note 6 | dBm/30kHz Note 6 | dBm/60kHz Note 6 |  |
| ±3.5 | ±5.0 | ≥-3dB | ≥24 | All | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -126 | -123 | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 | -50 |
|  |  |  |  |  | Note 4 |  |  |  |  |
|  |  |  |  |  | Note 4 |  |  |  |  |
| ±9.5 | ±11.0 | ≥-13dB | 24 ≤ BW ≤ 52 | All | Note 4 |  |  |  |  |
| ±6.5 | ±8.0 |  | 52< BW≤ 104 | All | Note 4 |  |  |  |  |
| ±5.0 | ±6.5 |  | BW >104 | All | Note 4 |  |  |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 10.1.24.2.2-2: PRS-RSRP relative accuracy for FR2

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | PRS Ês/Iot | PRS BW | Repetition factor (TrepPRS*LPRS/KcombPRS) | Io Note 7 range |  |  |
|  |  |  |  |  | Minimum Io Note 1 dBm / SCSPRS |  | Maximum Io |
| dB | dB | dB | PRB | - | dBm / SCSPRS |  | dBm/BWChannel |
|  |  |  |  |  | dBm/120kHz Note 6 | dBm/60kHz Note 6 |  |
| ±5.0 | ±8.0 | ≥-3dB | ≥24 | All | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival |  | -50 |
|  |  |  |  |  | Note 4 |  |  |
|  |  |  |  |  | Note 4 |  |  |
| ±10 | ±13 | ≥-13dB | 24 ≤ BW ≤ 64 | All | Note 4 |  |  |
| ±7.5 | ±10.5 |  | BW >64 | All | Note 4 |  |  |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: Void. NOTE 3: PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA or DL-AoD assistance data defined in [34]. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth ≥ 24 RB. NOTE 5: The serving cell, the reference cell, and the measured neighbour cell i are on the same carrier frequency. NOTE 6: The condition level is increased by ∆>0, when applicable, as described in Sections B.3.2 and B.3.3. NOTE 7: The Io is defined in PRS positioning subframes. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same subframe. NOTE 8: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

<End of Change 2>

<Start of Change 3>

10.1.25.2	Measurement Accuracy Requirements

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

- The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

If the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1.2 then:

-	UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

-	UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

Note: The requirements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

The accuracy requirements in Table 10.1.25.2-1 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25.2-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 78+ | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
| ± 59+ |  | ≥52 |  | ≥1 | Note 6 | Note 6 | Note 6 |
| ± 30+ |  | >104 |  | ≥1 | Note 6 | Note 6 | Note 6 |
| ± 57+ |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
| ± 30+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 29+ |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| ± 15+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 101+ | -13 | ≥24 | 15 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 75+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 37+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 58+ |  | ≥24 | 30 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 39+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 36+ |  | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 8+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-5. |  |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 10.1.25.2-2: UE Rx-Tx time difference measurement accuracy in FR1 in fading

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 137+ | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -126.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
| ± 96+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 62+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 87+ |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -123.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
| ± 68+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 44+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 59+ |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| ± 42+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 36+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 180+ | -13 | ≥24 | 15 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 98+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 68+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 87+ |  | ≥24 | 30 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 85+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 44+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 139+ |  | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 46+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 30+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-5. |  |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-3 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25.2-3: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 22+ | -3 | ≥24 | 60 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 12+ |  | ≥32 | 120 | ≥1 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 7+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 35+ | -13 | ≥24 | 60 | ≥1 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 14+ |  | ≥32 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ± 9+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-6. |  |  |  |  |  |  |

The accuracy requirements in Table 10.1.25.2-4 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 10.1.25.2-4: UE Rx-Tx time difference measurement accuracy in FR2 in fading

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 75+ | -3 | ≥24 | 60 | ≥4 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 72+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 57+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 61+ |  | ≥32 | 120 | ≥1 | Same value as PRP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 64+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 55+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 92+ | -13 | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 |
| ± 70+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 57+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 60+ |  | ≥32 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ± 66+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 62+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-6. |  |  |  |  |  |  |

Table 10.1.25.2-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

Table 10.1.25.2-6: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

<End of Change 3>