3GPP TSG-RAN WG4 Meeting #108                                                                                    R4-2311636

Toulouse, FR, Aug. 21 – Aug. 25, 2023

Source: 	CATT

Title: 	Further discussion on RF requirements for NCR-MT

Agenda item:	8.28.2.2

Document for:	Approval

- Introduction

In RAN4 #107 meeting, A WF [1] on NCR system parameters and RF requirements was agreed. However, there are still some remaining issues about NCR-MT need to be discussed in the next meeting.

In this contribution, we would like to share our views on remaining issues for RF requirements for NCR-MT.

- Discussion

  - RF requirements for Type 1-C and Type 1-H NCR- MT

2.1.1 Reference sensitivity level for FR1 NCR-MT

During the RAN4 #107 meeting, agreements on Reference sensitivity level for NCR-MT has already been made in WF [1]:

Issue 3-2-1 REFSENS requirement - Agreement: - NF is assumed as 5dB for FR1 WA NCR-MT, - FR1 LA NCR-MT: - Option1: 13dB - Option 2: legacy UE value - NF is assumed as 10dB for 30GHz and 12dB for 45GHz for FR2 NCR-MT

The FR1 REFSENS of IAB-MT follows the approach of BS, which the REFSENS requirement of BS (IAB-MT) is derived by equation below in TR 38.817-02 [2]:

## 7.2 Reference sensitivity level ### 7.2.1 General Reference sensitivity shall be specified according following formula:

For legacy UE, the REFSENS requirement is derived by equation below in TR 38.817-01 [3]:

## 6.1 General ## 6.2 Receiver Sensitivity For below 6GHz, the REFSENS level can be calculated by the equation below: Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

Apparently the REFSENS of UE is derived in a different manner than BS. For NCR-MT, it is obviously that the diversity gain is not needed when calculating REFSENS. Similar to IAB-MT, NCR-MT is a network device, which means that the requirements are defined for each cable connector (Type 1-C) or each TAB connector (Type 1-H), while the application of diversity gain pre-sets the number of Rx ports (2Rx or 4Rx). This will result in the REFSENS of NCR-MT cannot be calculated in UE approach.

Observation 1: The REFSENS of FR1 NCR-MT cannot be calculated in UE approach.

In addition, the NF of UE in spec is complex. As a roaming device, UE may operate on any frequency band (If supported). In TS 38.101-1, each supported operating bands will corresponds to a specific noise figure (REFSENS). Subject to the UE hardware capabilities, the noise figures in different frequency bands need to be discussed one by one and used to calculate different REFSENS. While this is an implementation-oriented issue, there are compromises in the spec. However, NCR is a network device which means that the operating band of NCR is pre-set by operators. In this scenario, we supposed that the NF of NCR-MT could be a fixed value to cover all operating bands, which is aligned with the IAB-MT approach. Therefore, we believed that the NF for FR1 LA IAB-MT i.e. 13 dB could be reused for FR1 LA NCR-MT.

Proposal 1: The 13 dB Noise figure for FR1 LA IAB-MT could be reused for FR1 LA NCR-MT.

Considering that the FRC parameters under 15 kHz SCS are not defined, we believed that the same DMRS configuration and CP-OFDM Symbols per slot configuration of 30/60 kHz SCS could be reused. In addition, allocated resource blocks for G-FR1-A1-1 and G-FR1-A1-4 from TS 38.104 could be reused for G-FR1-A1-21 and G-FR1-A1-24, respectively. Meanwhile, we noticed that the FRC channel numbering defined in the IAB-MT spec left the vacancy of channel 21 and channel 24. Therefore, the FRCs for 15kHz SCS could be reused from existing FRC Table A1-1 of TS 38.174 Annex A.1 and added to Table 2-1 for FR1 NCR-MT. It is proposed to use the FRC parameters of revised Table 2-1 below (in yellow shadow) as the FR1 reference sensitivity level for NCR-MT.

Proposal 2: It is suggested to use the revised Table 2-1 for FRC parameters below for FR1 NCR-MT REFSENS

Table 2-1: FRC parameters for FR1 reference sensitivity level for NCR-MT.

| Reference channel | G-FR1-A1-21 | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-24 | G-FR1-A1-25 | G-FR1-A1-26 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

2.1.2 ACS and IBB requirements for Type 1-C and Type 1-H NCR-MT

Since the NCR-MT will have the capability to demodulate wanted signals from carrier, the adjacent channel selectivity (ACS) requirement of NCR-MT should be defined. Similar to REFSENS, to determine ACS requirement leverages NF as well. For IAB-MT, the interfering signal mean power of ACS could be derived by the equation as below:

Interfering signal power level = Noise floor + ACS + 4.7dB = -174 dBm/Hz+10*log10(BW) + NF + ACS + 4.7 dB

For FR1 NCR-MT, we supposed that following the FR1 IAB-MT approach is reasonable, which means that the ACS value of IAB-MT i.e. 46dBc could be reused. With the NF and ACS discussed above, it is obviously that the specific values of ACS requirement could be calculated, which equals the values of IAB-MT. Therefore, the minimum requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT.

Proposal 3: For Adjacent Channel Selectivity, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT.

However, For IAB-MT ACS requirement, the 5MHz channel bandwidth related definition is missing. Therefore, the 5MHz related parameters should be defined. We listed the supplementary components below (in yellow shadow).

Proposal 4: The 5MHz channel bandwidth for ACS requirements should be supplemented in Table 2-2 and Table 2-3 below:

Table 2-2: ACS requirement for NCR type 1-C and 1-H MT

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5,10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area IAB-MT: -52 Local Area IAB-MT: -44 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the IAB-MT for that bandwidth. |  |  |

Table 2-3: NCR type 1-C and 1-H MT ACS interferer frequency offset values

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz CP-OFDM NR signal 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 40 | ±9.4675 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

For in-band blocking (IBB) requirements, similar to ACS, we believed that the minimum requirement for FR1 IAB-MT could be reused for FR1 NCR-MT, with the 5MHz channel bandwidth supplemented.

Proposal 5: For In-band blocking, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT.

Proposal 6: The 5MHz channel bandwidth for IBB requirements should be supplemented in Table 2-4, Table 2-5 and Table 2-6 below:

Table 2-4: NCR type 1-C and 1-H MT general blocking requirement

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + 6 dB | Wide Area NCR MT: -43 Local Area NCR MT: -35 | ±7.5 | 5 MHz CP-OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area NCR MT: -43 Local Area NCR MT: -35 | ±30 | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the NCR-MT channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2. |  |  |  |  |

Table 2-5: NCR type 1-C and 1-H MT narrowband blocking requirement

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area NCR MT: -49 Local Area NCR MT: -41 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the NCR-MT for that NCR-MT channel bandwidth NOTE 2: PREFSENS depends on the NCR-MT channel bandwidth as specified in tables 7.2.2-1 and 7.2.2-2. NOTE 3: 7.5 kHz shift is not applied to the wanted signal. |  |  |

Table 2-6: NCR type 1-C and 1-H MT narrowband blocking interferer frequency offsets

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 1 RB |
| 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

2.1.3 Operating band unwanted emissions for Type 1-C and Type 1-H NCR-MT

For unwanted emissions requirement, the agreements captured in WF [1]:

Issue 3-1-5 OBUE/SEM - Agreement: - For wide area NCR-MT: BS requirements applied - For local area NCR-MT: FFS - Proposal 3 agreed

However, we noticed that for LA NCR-MT, a simple application of UE requirements may not be appropriate. The  maximum offset of the band from edge i.e.ΔfOBUE is defined for OBUE in BS/IAB spec. However, the ΔfOOB is defined for SEM requirements in UE spec, which is not aligned with the BS approach. Therefore, for FR1 NCR-MT OBUE requirements, we believed that the OBUE requirements for FR1 IAB-MT could be reused. For LA class, we believed that the requirement should be defined in IAB-MT approach, but the value of UE (PC2) could be applied.

Proposal 7: For operating band unwanted emissions, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT. For LA class, it is suggested to define OBUE requirements in IAB-MT approach, but the specific UE (PC2) value could be applied.

2.1.4 Transmitter spurious emissions for Type 1-C and Type 1-H NCR-MT

RAN4 has achieved the agreement below on RAN4 #107 meeting in WF [1]:

Issue 3-1-6 transmitter spurious emission - Agreement: - Proposal 1 agreed (for NCR-MT transmitter spurious emission requirements, propose to follow Rel-17 repeater transmitter spurious emission requirements.) - If the NCR supports simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements should be defined on the total emissions from MT and FWD.

Obviously, for simultaneous and non-simultaneous transmission of NCR-MT and NCR-Fwd part in the uplink direction, the UL transmitter spurious emissions should adhere to this agreement as well. Since the frequency of spurious emissions is uncertain, the total emissions from MT and Fwd are not the sum of their respective emission. A more reasonable approach is to determine which entity dominates the spurious. For instance, BS allocates some certain radio resource (spectrum/PRBs) for NCR-MT, the radio resources for other UEs who leveraging the passband provide by same NCR will be excluded from those certain radio resource which allocated for NCR-MT (because they are under the scheduling of same BS). Therefore, the spurious emissions in the edge of the passband in fact are brought by NCR-MT. Therefore, we believed that the total emissions from FR1 NCR-MT and NCR-Fwd should still follow the FR1 BS spurious emissions requirements approach, which is aligned with Rel-17 repeater.

Proposal 8: For simultaneous and non-simultaneous transmission of NCR-MT and NCR-Fwd part in the uplink direction, it is suggested to reuse transmitter spurious emissions for FR1 repeater.

2.1.5 Receiver spurious emissions

Similar to transmitter spurious emissions, we believed that the existing IAB-MT or BS approach could be followed.

Proposal 9: For receiver spurious emissions, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT.

2.1.6 Output intermodulation

Considering that the NCR-MT will occupy only a few carriers for the control signal transmission, we supposed that the output IMD requirements could reuse the IAB-MT output IMD requirements. However, the Type 1-C does not exist in IAB-MT spec TS 38.174. Therefore, the Type 1-C output IMD requirements for NCR-MT should be reused from Type 1-C BS.

Proposal 10: For output intermodulation, the requirements for FR1 IAB-MT could be reused for Type 1-H NCR-MT; for FR1 Type 1-C NCR-MT, it is suggested to reuse the BS Type 1-C requirements

2.1.7 Transmitter ON/OFF power

For the transmitter OFF power and transmitter transient period of NCR-MT, we believed that following existing IAB-MT requirements could be reasonable.

Proposal 11: For transmitter ON/OFF power and transient period, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT

2.1.8 Out-of-band blocking

For the Out-of-band blocking requirements of NCR-MT, we believed that following existing IAB-MT requirements could be reasonable.

Proposal 12: For Out-of-band blocking, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT

2.1.9 Receiver intermodulation

For the receiver IMD requirements of NCR-MT, we believed that following existing IAB-MT requirements could be reasonable.

Proposal 13: For Receiver intermodulation, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT

2.1.10 Apply the Type 1-H requirements to Type 1-C for NCR-MT

An additional issue is that the FR1 NCR-MT could be categorized into Type 1-C, Type 1-H and Type 1-O. Therefore, some minor modifications are required when reusing the requirement of IAB-MT. We supposed that for Type 1-C NCR-MT, the requirements that apply to each TAB connector in Type 1-H scenario could be applied to each antenna connector.

Proposal 14: For Type 1-C NCR-MT, unless otherwise stated, the RF requirements that apply to each TAB connector in Type 1-H scenario should be applied to each antenna connector.

  - RF requirements for Type 1-O and Type 2-O NCR- MT

Generally, the RF requirement for IAB-MT type 1-O can be reused for NCR type 1-O MT, and RF requirement for IAB-MT type 2-O can be reused for NCR type 2-O MT.

2.2.1 OTA ACS and IBB requirements for Type 1-O and Type 2-O NCR-MT

Similar to the discussion of ACS and IBB requirement for FR1 NCR-MT, we believed that following existing FR2 IAB-MT approach could be reasonable. Likewise, the 5MHz channel bandwidth of Type 1-O NCR-MT should be supplemented in same manner of Type 1-H NCR-MT.

Proposal 15: For OTA adjacent channel sensitivity, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.  It is suggested to supplement 5MHz channel bandwidth for Type 1-O NCR-MT as the same manner of Type 1-H NCR-MT.

Proposal 16: For OTA in-band blocking, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.  It is suggested to supplement 5MHz channel bandwidth for Type 1-O NCR-MT as the same manner of Type 1-H NCR-MT.

2.2.2 OTA Operating band unwanted emissions requirement for Type 1-O and Type 2-O NCR-MT

Similar to the discussion of operating band unwanted emissions requirement for FR1 NCR-MT, we believed that following existing IAB-MT approach could be reasonable.

Proposal 17: For OTA operating band unwanted emissions, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

2.2.3 OTA transmitter spurious emissions requirement for Type 1-O and Type 2-O NCR-MT

Similar to the discussion of transmitter spurious emissions requirement for FR1 NCR-MT, we believed that following existing IAB-MT approach could be reasonable. In addition, the co-location with BS and repeater requirement is not applicable for FR2.

Proposal 18: For OTA transmitter spurious emissions, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

2.2.4 OTA receiver spurious emissions requirement for Type 1-O and Type 2-O NCR-MT

Similar to the discussion of receiver spurious emissions requirement for FR1 NCR-MT, we believed that following existing IAB-MT approach could be reasonable.

Proposal 19: For OTA receiver spurious emissions, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

2.2.5 OTA Output intermodulation for Type 1-O NCR-MT

Similar to the discussion of output intermodulation requirement for FR1 NCR-MT, we believed that following existing IAB-MT approach could be reasonable. In addition, the Output IMD is not applicable for FR2.

Proposal 20: For OTA output intermodulation, the requirement for Type 1-O IAB-MT could be reused for Type 1-O NCR-MT

2.2.6 OTA transmitter ON/OFF power for Type 1-O and Type 2-O NCR-MT

Similar to the discussion of transmitter ON/OFF power requirement for FR1 NCR-MT, we believed that following existing IAB-MT approach could be reasonable.

Proposal 21: For OTA transmitter ON/OFF power, the requirement for FR2 IAB-MT could be reused for FR2 NCR-MT.

2.2.7 OTA out-of-band blocking

Similar to the discussion of out-of-band blocking requirement for FR1 NCR-MT, we believed that following existing IAB-MT approach could be reasonable. In addition, the co-location requirement is not applicable for FR2.

Proposal 22: For OTA out-of-band blocking, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

2.2.8 OTA receiver intermodulation

Similar to the discussion of receiver intermodulation requirement for FR1 NCR-MT, we believed that following existing IAB-MT approach could be reasonable. In addition, the receiver intermodulation requirement is not applicable for FR2.

Proposal 23: For OTA receiver intermodulation, the requirement for Type 1-O IAB-MT could be reused for Type 1-O NCR-MT

- Conclusion

This contribution provides our consideration on RF requirements for NCR-MT. The following proposals are concluded as follows:

Observation 1: The REFSENS of FR1 NCR-MT cannot be calculated in UE approach.

Proposal 1: The 13 dB Noise figure for FR1 LA IAB-MT could be reused for FR1 LA NCR-MT.

Proposal 2: It is suggested to use the revised Table 2-1 for FRC parameters below for FR1 NCR-MT REFSENS

Table 2-1: FRC parameters for FR1 reference sensitivity level for NCR-MT.

| Reference channel | G-FR1-A1-21 | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-24 | G-FR1-A1-25 | G-FR1-A1-26 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

Proposal 3: For Adjacent Channel Selectivity, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT.

Proposal 4: The 5MHz channel bandwidth for ACS requirements should be supplemented in Table 2-2 and Table 2-3 below:

Table 2-2: ACS requirement for NCR type 1-C and 1-H MT

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5,10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area IAB-MT: -52 Local Area IAB-MT: -44 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the IAB-MT for that bandwidth. |  |  |

Table 2-3: NCR type 1-C and 1-H MT ACS interferer frequency offset values

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz CP-OFDM NR signal 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 40 | ±9.4675 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

Proposal 5: For In-band blocking, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT.

Proposal 6: The 5MHz channel bandwidth for IBB requirements should be supplemented in Table 2-4, Table 2-5 and Table 2-6 below:

Table 2-4: NCR type 1-C and 1-H MT general blocking requirement

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + 6 dB | Wide Area NCR MT: -43 Local Area NCR MT: -35 | ±7.5 | 5 MHz CP-OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area NCR MT: -43 Local Area NCR MT: -35 | ±30 | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the NCR-MT channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2. |  |  |  |  |

Table 2-5: NCR type 1-C and 1-H MT narrowband blocking requirement

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area NCR MT: -49 Local Area NCR MT: -41 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the NCR-MT for that NCR-MT channel bandwidth NOTE 2: PREFSENS depends on the NCR-MT channel bandwidth as specified in tables 7.2.2-1 and 7.2.2-2. NOTE 3: 7.5 kHz shift is not applied to the wanted signal. |  |  |

Table 2-6: NCR type 1-C and 1-H MT narrowband blocking interferer frequency offsets

| NCR-MT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 1 RB |
| 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz CP-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper NCR-MT RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

Proposal 7: For operating band unwanted emissions, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT. For LA class, it is suggested to define OBUE requirements in IAB-MT approach, but the specific UE (PC2) value could be applied.

Proposal 8: For simultaneous and non-simultaneous transmission of NCR-MT and NCR-Fwd part in the uplink direction, it is suggested to reuse transmitter spurious emissions for FR1 repeater.

Proposal 9: For receiver spurious emissions, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT.

Proposal 10: For output intermodulation, the requirements for FR1 IAB-MT could be reused for Type 1-H NCR-MT; for FR1 Type 1-C NCR-MT, it is suggested to reuse the BS Type 1-C requirements

Proposal 11: For transmitter ON/OFF power and transient period, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT

Proposal 12: For Out-of-band blocking, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT

Proposal 13: For Receiver intermodulation, the requirement for FR1 IAB-MT could be reused for Type 1-C and Type 1-H NCR-MT

Proposal 14: For Type 1-C NCR-MT, unless otherwise stated, the RF requirements that apply to each TAB connector in Type 1-H scenario should be applied to each antenna connector.

Proposal 15: For OTA adjacent channel sensitivity, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.  It is suggested to supplement 5MHz channel bandwidth for Type 1-O NCR-MT as the same manner of Type 1-H NCR-MT.

Proposal 16: For OTA in-band blocking, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.  It is suggested to supplement 5MHz channel bandwidth for Type 1-O NCR-MT as the same manner of Type 1-H NCR-MT.

Proposal 17: For OTA operating band unwanted emissions, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

Proposal 18: For OTA transmitter spurious emissions, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

Proposal 19: For OTA receiver spurious emissions, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

Proposal 20: For OTA output intermodulation, the requirement for Type 1-O IAB-MT could be reused for Type 1-O NCR-MT

Proposal 21: For OTA transmitter ON/OFF power, the requirement for FR2 IAB-MT could be reused for FR2 NCR-MT.

Proposal 22: For OTA out-of-band blocking, the requirement for Type 1-O and Type 2-O IAB-MT could be reused for Type 1-O and Type 2-O NCR-MT, respectively.

Proposal 23: For OTA receiver intermodulation, the requirement for Type 1-O IAB-MT could be reused for Type 1-O NCR-MT

- References

- R4-2309783, WF on system parameters and RF requirements, ZTE, RAN4#107

- TR 38.817-02, General aspects for Base Station (BS) Radio Frequency (RF) for NR, V15.10.0, 3GPP TR

- TR 38.817-01, General aspects for User Equipment (UE) Radio Frequency (RF) for NR, V16.4.0 ,3GPP TR