3GPP TSG-RAN WG4 Meeting # 108                                                       R4- 2311637

# Toulouse, FR, August 21 – August 25, 2023

Title: 	TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects”

Source: 	CATT

Agenda item:	8.19.2.2.1

Document for:	Approval

Background

This TP provides the TP for FR1 LA BS feasibility from editor point of view. It was agreed in [1] that WA analysis can be broke down to capture several companies’ analysis. Considering the TP inputs for LA may be less than WA. We think the analysis may be combined if there’s not much difference from companies.

The following agreement was agreed in RAN4#107, but is missing in the WF [3],

Issue 1-1-5: MR and LA-BS SBFD with 1dB sensitivity degradation

- Agreement:

  - For implementation feasibility study, RAN4 confirm FR1 MR and LA SBFD gNB with 1dB sensitivity degradation due to self-interference is achievable.

    - FFS on co-site inter-sector interference impact.

So for LA SBFD BS, the conclusion is feasible.

Reference

[1] R4-2306006, “WF on implementation feasibility of SBFD”, RAN4#106bis-e, Samsung

[2] R4-2309790, “WF for RF feasibility study and BS requirements impact on SBFD operation”, RAN4#107, Samsung

[3] R4-2307388, TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects”, RAN4#107, CATT

TP for TR 38.858: Feasibility of FR1 local area BS aspects

9.4	Feasibility of FR1 local area BS aspects

9.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 9.4.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

Table 9.4.1.1-1: SI feasibility study for FR1 Local Area BS

| Parameters |  |  |  | CATT | Company name |
| --- | --- | --- | --- | --- | --- |
| BS TX Power dB(P_TX ) = ① dBm |  |  |  | 24 |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc |  | 45 |  |
|  |  | Frequency isolation techniques used |  | CFR |  |
|  |  | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc |  | 70 |  |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation、 isolation structures、isolation material、cross polarization |  |
|  | TX Beam nulling /isolation in TX sub-band dB(α_(SI-beam) ) = ④ dBc |  |  | 0 |  |
|  |  |  |  | TX Beam nulling is not assumed due to the array size |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBm(P_(SI_leakage) ) (Note 1) |  |  | -104.01 |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band sub-band dB(α_(SI-RFIC) ) = ⑤ dBc |  | 0 |  |
|  |  | RF IC capability and other tech. in RX sub-band sub-band dB(α_(SI-RFIC) ) = ⑧ dBc |  | 0 |  |
|  |  | RF IC techniques and other tech.(before LNA) |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBm(P_(SI_TXSB) ) (Note 1) |  |  | -46 |  |
|  | Blocker Suppression at RX | Frequency isolation capability dB(α_(CSSI-frequency,RX) )=⑥dBc |  | 55 |  |
|  |  | Frequency isolation techniques |  | Digital filter for ACS |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -16 |  |
|  |  |  | Rx IM3 contribution (dBm) | -106 |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No other significant impacts |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBm(P_(SI_Selectivity) )(Note 1, 2) |  |  | -101 |  |
|  | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc |  |  | 0 |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |
|  | Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  |  | 10 |  |
| Overall RSIC capability dB(RSIC) (Note 1) |  |  |  | 122.33 |  |
| Noise floor ⑩dBm |  |  |  | -87.99 |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93.99 |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117.99 |  |
| SBFD configuration |  |  |  | 40-20-40 |  |
| Guardband assumption (if exist) |  |  |  |  |  |
| Bandwidth over which suppression is achieved |  |  |  |  |  |

### 9.4.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

In SI capability analysis, the following techniques are used,

- CFR is used to improve equipment efficiency, DPD is used for high power equipment to optimize ACLR.

- Tx antennas panel and Rx antennas panel are separate, there are some isolation material between them, and cross polarization is also used.

- Rx RF filter is assumed in the analysis.

- Digital filter is used to resolve the adjacent sub-band (i.e. TX subband) interference issue.

- Digital IC is used to reduce interference in the UL sub-bands.

From the analysis in Table 9.4.1.1-1, the following capabilities are needed,

- Frequency isolation capability of frequency isolation at TX is 45dB

- Spatial isolation is 70dB

- ACS is 55dBc

- IIP3 is -16dBm

- Digital IC is 10dB

The above capabilities except frequency isolation are improved compared with legacy LA BS, but it’s feasible from implementation point of view.

### 9.4.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

From the analysis in 9.4.1.1, it can be concluded that SBFD LA BS can achieve the 1dB sensitivity degradation target.

9.4.2	Co-channel inter-sub-band inter-site interference analysis

This section captures the typical assumption of RF requirements and analysis results

9.4.3	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

Based on the analysis for self-interference capability and the RF capability for co-channel inter-sub-band inter-site interference analysis, it can be concluded that Rel-18 SBFD function is feasible for local area gNB.