3GPP TSG-RAN WG4 Meeting # 108                                                       R4- 2311638

# Toulouse, FR, August 21 – August 25, 2023

Title: 	TP for TR 38.858:“Feasibility of FR1 Wide Area BS aspects”

Source: 	CATT

Agenda item:	8.19.2.2.1

Document for:	Approval

Background

In WF [1], The TR section “9.2 Feasibility of FR1 Wide Area BS aspects” has been further broken-down. We have provided some analyses in [3] , this TP provides our company’s further analyses based on WF [2].

Reference

[1] R4-2306006, “WF on implementation feasibility of SBFD”, RAN4#106bis-e, Samsung

[2] R4-2309790, “WF for RF feasibility study and BS requirements impact on SBFD operation”, RAN4#107, Samsung

[3] R4-2307389, TP for TR 38.858:“Feasibility of FR1 Wide Area BS aspects”, RAN4#107, CATT

TP for TR 38.858

## 9.2	Feasibility of FR1 wide area BS aspects

9.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 9.2.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

SI feasibility study for Wide area BS can be made as following.

Table 9.2.1.1-1: SI feasibility study for Wide Area BS

| FR1 |  |  |  | Calculation formula | CATT |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  |  | WA |
| BS TX Power dB(P_TX ) = ① dBm |  |  |  |  | 49 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc |  |  | 45 |
|  |  | Frequency isolation techniques used |  |  | CFR、DPD |
|  |  | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc |  |  | 70 |
|  |  | Spatial isolation techniques used |  |  | TX/RX panel separation、 isolation structures、isolation material、cross polarization |
|  | TX Beam nulling /isolation in TX sub-band dB(α_(SI-beam) ) = ④ dBc |  |  |  | 10 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBm(P_(SI_leakage) ) (Note 1) |  |  | ①-②- ③- ⑨-⑦-10*log10(40/20) | -89.01 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band sub-band dB(α_(SI-RFIC) ) = ⑤ dBc |  |  | 0 |
|  |  | RF IC capability and other tech. in RX sub-band sub-band dB(α_(SI-RFIC) ) = ⑧ dBc |  |  | 0 |
|  |  | RF IC techniques and other tech.(before LNA) |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  |  | 0 |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBm(P_(SI_TXSB) ) (Note 1) |  |  | ①-③-④-⑤ | -31 Rx is blocked |
|  | Blocker Suppression at RX | Frequency isolation capability dB(α_(CSSI-frequency,RX) )=⑥dBc |  |  | 65 |
|  |  | Frequency isolation techniques |  |  | Digital filter for ACS |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  | -10 |
|  |  |  | Rx IM3 contribution (dBm) | (①-③-④-⑤)-2*(IIP3-(①-③-④-⑤)) | -73 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  | -116 |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalizeddB(P_(SI_Selectivity) )(Note 1, 2) |  |  | ①-③-④-⑤-⑥ | -96 |
|  | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc |  |  |  | 10 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |
|  | Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  |  |  | 10 |
| Overall RSIC capability dB(RSIC) (Note 1) |  |  |  | ①-(P_(SI_leakage)+Rx IM3+Other RX +P_(SI_Selectivity)) Calculate with power linear value | 121.86 |
| Noise floor ⑩dBm |  |  |  | -174+10*log10(20*106)+NF | -95.99 |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  | -101.99 |
| Required RSIC budget (①-⑪dBc) |  |  |  |  | 150.99 |
| SBFD configuration |  |  |  |  | 40-20-40 |
| Guardband assumption (if exist) |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. |  |  |  |  |  |
| Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. |  |  |  |  |  |
| Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

### 9.2.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

### 9.2.1.2.1	[CATT]

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 9.2.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

For FR1 Wide Area BS SI analysis, the following assumptions are used.

Table 9.2.1.2.1-1: SI analysis assumptions for Wide area BS

| Parameters | Wide Area BS |
| --- | --- |
| Reference sensitivity level degradation due to SI | 1dB |
| Channel bandwidth | 100MHz |
| Subband configuration | {DUD} |
| DL subband width | 40MHz*2 |
| UL subband width | 20MHz |
| Tx output power over whole channel | 49dBm |
| Adjacent subband Leakage Power Ratio | 45dB |
| Noise Figure | 5dB |

In SI capability analysis, the following techniques are used,

- CFR is used to improve component efficiency, DPD is used for high power equipment to optimize ACLR.

- Tx antennas panel and Rx antennas panel are separate, there are also some isolation material between them, and cross polarization is also used.

- RF subband RF filter is assumed for Tx and Rx path.

- Digital filter is used to resolve the adjacent subband (i.e. Tx subband) interference issue.

- Beam nulling is used to improve isolation between Tx and Rx.

- Digital IC is used to reduce interference in the UL sub-bands. The interference is leakage from the transmitter, similar as DPD, digital domain need to capture and sample the interference signals, then subtract the interference at receiver.

From the analysis in Table 9.2.1.1-1, the receiver may be blocked. Some specific techniques need to be used to improve the spatial ISO. Requirement of IIP3 is relatively high and the ACS requirement is improved largely compared with the legacy BS. It seems wide area SBFD BS design may need much improvement from several aspects to make it feasible.