3GPP TSG-RAN WG4 Meeting # 108                                                         R4- 2311640			R4-22xxxxx

# Toulouse, FR, August 21 – August 25, 2023

Title: 	SBFD adjacent channel co-existence simulation results

Source: 	CATT

Agenda item:	8.19.2.1

Document for:	Discussion

- Background

This contribution provides the SBFD adjacent channel co-existence simulation results according to the update of the simulation scenarios specified in WF [1] of lasting meeting and template to collect simulation results which has been used during offline between #107 and #108.

- Discussion

  - SBFD adjacent channel co-existence simulation scenarios

SBFD adjacent channel co-existence simulation scenarios summarized in table 2.1-1, relevant case are specified in table 2.1-2. SBFD subband configuration is {DU}.

Table 2.1-1: SBFD adjacent channel co-existence simulation scenarios

| FR | Scenario No. | Deployment Scenario1 | Priority |
| --- | --- | --- | --- |
|  |  | (Aggressor -> Victim) |  |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 3 | Indoor -> Indoor | Low |
|  | 4 | CableLabs：UMa-to-UMi | Note 5 |
|  | 5 | UMi-to-UMi | Note 6 |
| FR2 (30GHz) | 6 | Urban Macro -> Urban Macro | High |
|  | 7 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 8 | Urban Micro -> Urban Micro | Low |
|  | 9 | Indoor -> Indoor | Low |
| Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios. |  |  |  |
| Note 2: The Urban Hotspot uses the same assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE. Both random dropping and cluster-based dropping for calibration. |  |  |  |
| Note 3: Consider Urban Macro scenario first for calibration purpose. |  |  |  |
| Note 4: Companies are encouraged to provide simulation results for Urban Hotspot scenario as 2nd priority. [Editor’s Note: Agreement 2.2.1 of R4-2302888] |  |  |  |
| Note 5: Companies also encouraged to simulate Uma-to-UMi co-existence scenario as 2nd priority. [Editor’s Note: Agreement 2.2.3 of R4-2302888] |  |  |  |
| Note 6: Use UMi simulation assumption R4-2305922 as starting point. Consider Tx power refer to 3GPP UMi output power. Detailed simulation assumptions will be discussed after RAN4#107 meeting. |  |  |  |

Table 2.1-2: SBFD adjacent channel co-existence simulation case

| Case | Victim | Aggressor | Figures: | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Aggressor(left) and Victim(right) |  |  |
| 1 | NR TDD DL | SBFD (DU) |  | NR TDD DL | High |
| 2 | NR TDD UL | SBFD (DU) |  | NR TDD UL | Low |
| 3 | SBFD (DU) | NR TDD DL |  | No system in adj channel | High |
| 4 | SBFD(DU) | NR TDD UL |  |  | Low |

  - Simulation calibration results

Scenario 1: Urban Macro -> Urban Macro

Table 2.2.1-1:  Victim with aggressor ACI / victim with baseline ACI compared to victim without ACI (Scenario 1)

Table 2.2.1-2:  Victim with aggressor ACI degradation compared to victim with baseline ACI (Scenario 1)

Table 2.2.1-3:  TP degradation corresponding to ACIR (Scenario 1)

| Case | Victim | TP degradation corresponding to ACIR |  |
| --- | --- | --- | --- |
|  |  | TP degradation @5% | TP degradation @Mean |
| 1 | NR TDD DL | Relative ACIR | Relative ACIR |
| 2 | NR TDD UL | / | Relative ACIR+8dB |
| 3 | SBFD (UL) | / | Relative ACIR+6dB |
|  | SBFD (DL) | Relative ACIR+2dB | Relative ACIR |
| 4 | SBFD (UL) | Relative ACIR+4dB | Relative ACIR |
|  | SBFD (DL) | Relative | Relative ACIR |

The simulation results of victim with aggressor ACI compared to victim without ACI situation show,

- 50% throughput degradations corresponding to ACIR of case1, case 3 when SBFD DL is victim and case 4 are relative values; this situation shows that the impact of adopting the relative ACIR can meet the performance degradation requirements of the victim system. ACIR of case3 when SBFD UL is victim is relative ACIR plus 6dB; it’s too high and can’t be achieved even if SBFD UL ACS capability is 65dB.

- 5% throughput degradation corresponding to ACIR of case1 and case4 when SBFD DL are both relative values, this situation shows that the impact of adopting the relative ACIR can meet the performance degradation requirements of the victim system. The ACIR of case 3 when SBFD DL is victim may be achieved if ACS capability of UE can be improved to 36dB, but the ACIR of case 4 when SBFD UL is victim can’t be achieved.

Observation 1: For scenarios 1,

- 50% throughput degradations corresponding to ACIR of case1, case 3 when SBFD DL is victim and case 4 can reuse the relative values. ACIR of case2 and case3 when SBFD UL is victim can’t be achieved.

- 5% throughput degradation corresponding to ACIR of case1 and case4 when SBFD DL is victim can reuse the relative values.  The ACIR of case 3 when SBFD DL is victim may be achieved if ACS capability of UE can be improved to 36dB, but the ACIR of case 4 when SBFD UL is victim can’t be achieved.

Scenario 6: Urban Macro -> Urban Macro

Table 2.2.2-1:  Victim with aggressor ACI / victim with baseline ACI compared to victim without ACI (Scenario 6)

Table 2.2.2-2:  Victim with aggressor ACI degradation compared to victim with baseline ACI (Scenario 6)

Table 2.2.2-3:  TP degradation corresponding to ACIR (Scenario 6)

| Case | Victim | TP degradation corresponding to ACIR |  |
| --- | --- | --- | --- |
|  |  | TP degradation @5% | TP degradation @Mean |
| 1 | NR TDD DL | Relative ACIR | Relative ACIR |
| 2 | NR TDD UL | Relative ACIR | Relative ACIR |
| 3 | SBFD (UL) | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR+2dB | Relative ACIR |
| 4 | SBFD (UL) | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR |

The simulation results of victim with aggressor ACI compared to victim without ACI situation show,

- 50% throughput degradations corresponding to ACIR of case1 to case 4 are all relative value; this situation shows that the impact of adopting the relative ACIR can meet the performance degradation requirements of the victim system.

- 5% throughput degradations corresponding to ACIR except case 3 when SBFD DL is victim are all relative value, this situation shows that the impact of adopting the relative ACIR can meet the performance degradation requirements of the victim system. Relative ACIR plus 2dB of case 3 may be achieved if ACS of UE can be improved to 36dB.

Observation 2: For scenarios 6,

- 50% throughput degradations corresponding to ACIR of case1 to case4 can reuse the relative values.

- 5% throughput degradation corresponding to ACIR of case1 to case4 except case 3 when SBFD DL is victim can reuse the relative values.  The excepted case may be achieved if ACS capability of UE can be improved to 36dB.

  - Co-existence simulation results

Table 2.3-1:  TP degradation corresponding to ACIR

|  |  | TP degradation corresponding to ACIR |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Case | Victim | Scenario 1 |  | Scenario 6 |  |
|  |  | TP degradation @5% | TP degradation @Mean | TP degradation @5% | TP degradation @Mean |
| 1 | NR TDD DL | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR |
| 2 | NR TDD UL | / | Relative ACIR+8dB | Relative ACIR | Relative ACIR |
| 3 | SBFD (UL) | / | Relative ACIR+6dB | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR+2dB | Relative ACIR | Relative ACIR+2dB | Relative ACIR |
| 4 | SBFD (UL) | Relative ACIR+4dB | Relative ACIR | Relative ACIR | Relative ACIR |
|  | SBFD (DL) | Relative ACIR | Relative ACIR | Relative ACIR | Relative ACIR |

Table 2.3-2:  Realizability of every case

| Case | Victim | Scenario 1 |  | Scenario 6 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TP degradation @5% | TP degradation @Mean | TP degradation @5% | TP degradation @Mean |
| 1 | NR TDD DL | Yes | Yes | Yes | Yes |
| 2 | NR TDD UL | No | No | Yes | Yes |
| 3 | SBFD (UL) | No | No | Yes | Yes |
|  | SBFD (DL) | ACS of UE need to be 36dB | Yes | ACS of UE need to be 36dB | Yes |
| 4 | SBFD (UL) | No | Yes | Yes | Yes |
|  | SBFD (DL) | Yes | Yes | Yes | Yes |
| Note 1：“Yes”: the performance degradation is acceptable when relative ACIR is used. Note 2：“No”: the performance degradation is not acceptable when relative ACIR is used. |  |  |  |  |  |

Victim with aggressor ACI compared to victim without ACI,

- For scenarios 1,

- 50% throughput degradations corresponding to ACIR of case1, case 3 when SBFD DL is victim and case 4 can reuse the relative values. ACIR of case3 when SBFD UL is victim can’t be achieved.

- 5% throughput degradation corresponding to ACIR of case1 and case4 when SBFD DL is victim can reuse the relative values.  The ACIR of case 3 when SBFD DL is victim may be achieved if ACS capability of UE can be improved to 36dB, but the ACIR of case 4 when SBFD UL is victim can’t be achieved.

- For scenarios 6,

- 50% throughput degradations corresponding to ACIR of case1 to case4 can reuse the relative values.

- 5% throughput degradation corresponding to ACIR of case1 to case4 except case 3 when SBFD DL is victim can reuse the relative values.  The excepted case may be achieved if ACS capability of UE can be improved to 36dB.

- According to assumption of SBFD system, ACLR and ACS of UE, also of NR TDD BS are fixed value, only ACS of SBFD BS can be improved, the improved ACS capability of UE need to be considered.

- Summary

This contribution summarizes the simulation results from our company; the realizability of every case is specified in table 2.3-2.

Observation 1: For scenarios 1,

- 50% throughput degradations corresponding to ACIR of case1, case 3 when SBFD DL is victim and case 4 can reuse the relative values. ACIR of case2 and case3 when SBFD UL is victim can’t be achieved.

- 5% throughput degradation corresponding to ACIR of case1 and case4 when SBFD DL is victim can reuse the relative values.  The ACIR of case 3 when SBFD DL is victim may be achieved if ACS capability of UE can be improved to 36dB, but the ACIR of case 4 when SBFD UL is victim can’t be achieved.

Observation 2: For scenarios 6,

- 50% throughput degradations corresponding to ACIR of case1 to case4 can reuse the relative values.

- 5% throughput degradation corresponding to ACIR of case1 to case4 except case 3 when SBFD DL is victim can reuse the relative values.  The excepted case may be achieved if ACS capability of UE can be improved to 36dB.

Reference

[1] R4-2309793, “WF for SBFD co-existence study”, RAN4#107