3GPP TSG-RAN WG4 Meeting # 108	            R4-2311653

Toulouse, FR, Aug 21th – Aug 25th, 2023

Agenda Item:	8.24.2.3

Source: 	CATT

Title: 	Discussion on L1/L2 inter-cell mobility delay requirements

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on NR mobility enhancements (part 1) was approved [1]. In this contribution, some open issues on L1/L2 inter-cell mobility delay requirements are further discussed.

2. Discussion

Detail of cell switch delay requirements for Pcell/PSCell

Regarding the detail of cell switch delay requirements for Pcell/PSCell, there was still some issues left that have not yet reached a consensus in the last meeting. In this paper, we will further provide our views and proposals:

Processing time: Tprocessing,2 /TLTM_processing

In the last meeting, RAN4 discussed the value of Tprocessing,2 /T LTM_processing, and the related WF is duplicated below [1]:

<Way Forward> FFS the following option: - Option 1 (Huawei): Tprocessing,2 is 20ms for the same FR and 40ms for different FR. - Option 2 (MTK): - Categorize all the scenarios into at most four groups depending on if L2/L3 reconfiguration or L1 reconfiguration is needed and further discuss which group(s) to define requirements for. - Tprocessing,2=20ms for intra-FR cell switch and Tprocessing,2=40ms for inter-FR cell switch when software processing for L2/L3 reconfiguration and L1 reconfiguration is needed. - Option 3 (Ericsson): RAN4 to define 4 groups of UE processing delays and the delay values for different groups is [0ms, 5ms, 10ms, 20ms/40ms] - In which scenarios which delay being applied is FFS. - Recommended WF - Recommend agree on: - As a baseline, Tprocessing,2 /T LTM_processing is 20ms for the intra-FR cell switch and 40ms for inter-FR cell switch. - FFS: in which scenarios Tprocessing,2 /T LTM_processing can be reduced and the detailed value.

RAN4 has agreed to use legacy value of Tprocessing,2 as a baseline (20ms for the intra-FR cell switch and 40ms), and in which scenarios Tprocessing, 2/T LTM_ Processing can be reduced for LTM and the detailed value are still under discussion .

Considering that beam application time (BAT) mentioned in LS R1-2306259 [2], we believe that the legacy application time of MAC-CE may has been included in the current mobility latency model defined in 38.300 running CR, as 3Nslotsubframe,μ and BeamAppTime-r17 are already defined in Rel-17. And the values of BAT need to be defined in RAN1 according to different scenarios.

For Tprocessing,2 /T LTM_processing, it is suggested to consider whether R18 BAT is included in it. If the legacy application time of MAC-CE has not been included in Tprocessing,2, additional time for beam application will be introduced for LTM cell switch time. Otherwise, RAN4 should consider the impact of enhancement of BAT compared with the legacy application time of MAC-CE on Tprocessing,2 /T LTM_processing.

Proposal 1: RAN4 to discuss whether the legacy application time of MAC-CE has been included in the current mobility latency model defined in 38.300 running CR.

- The legacy application time of MAC-CE has been included in the current mobility latency model, as 3Nslotsubframe,μ and BeamAppTime-r17 have been already defined in Rel-17.

Proposal 2: If the legacy application time of MAC-CE has not been included in Tprocessing,2, additional time for beam application will be introduced for LTM cell switching time. Otherwise, RAN4 should consider the impact of enhancement of BAT compared with the legacy application time of MAC-CE on Tprocessing,2 /T LTM_processing.

T/F fine tracking: TΔ and Tmargin

In the last meeting, RAN4 discussed the value of TΔ and Tmargin, and the related WF is duplicated below:

<Way Forward> FFS the following options: - Option 1 (CTC, Nokia, Ericsson, Apple, CATT, MTK,): The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms. FFS: whether TΔ and Tmargin can be 0 under certain conditions. - Option 1a (CATT, MTK): the baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms - TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms. - Option 1b (CTC, Ericsson): the baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms - TΔ and Tmargin can be 0 under the condition that pre-Sync for candidate cell(s) has been performed before cell switch command. - Option 2 (CMCC,): TΔ = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command or target cell is current active SCell. - Option 3 (vivo, Huawei, ZTE): TΔ = 0 for the case that SSB based fine synchronization for candidate cell(s) is performed before cell switch command.

We support that the baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms. For the certain conditions where TΔ and Tmargin can be 0, we support Option 1a. For Option 1b, we don’t believe that pre-Sync means that UE has performed T/F fine tracking for candidate cell(s).

In the last meeting, regarding the requirements of TCI state activation of neighbour cell before cell switch command, RAN4 has agreed that at least one SSB is available at the UE during the last 160ms to satisfy transmit timing requirements [1], so we believe that it is necessary for the UE to meet the Te requirement for an initial transmission. The certain conditions where TΔ and Tmargin can be 0 should consider the requirements.

Therefore, if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms, then the UE does not need to perform T/F fine tracking on subsequent initial transmission, that is, TΔ and Tmargin can be 0. Otherwise, TΔ=1 Tfirst-RS.

Proposal 3: The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms

- TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms.

Whether to define PCell/PSCell switch delay requirements for unknown TCI state case when TCI state is indicated together with cell switch command

In the last meeting, RAN4 discussed whether to define PCell/PSCell switch delay requirements for unknown TCI state case when TCI state is indicated together with cell switch command, and the related WF is duplicated below:

< Way Forward>: - When TCI state is indicated together with cell switch command, only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

It is not reasonable for NW to indicate an unknown TCI state to UE. Though the TCI state is unknown at UE due to poor channel state, it is better to perform RACH-based cell switch [3].

Proposal 4: When TCI state is indicated together with cell switch command, only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

TCI state switching time

In the last meeting, RAN4 discussed whether the TCI state switching time is needed, and the related WF is duplicated below:

<Way Forward> FFS the following options: - Option 1 (Huawei): If SSB based fine synchronization is performed before cell switch, TCI state switch delay is needed. - Option 2 (Nokia, Ericsson): wait for more progress in RAN1. - Option 3 (vivo): add extra switching time, while the detailed value equals to beam application time, which will be further specified by RAN1.

For Option 1 and Option 3, considering that RAN1 has agreed that TCI is indicated together with cell switch command, if SSB based fine synchronization is performed before cell switch, TCI state switch delay is needed, which may include the beam application time (BAT) mentioned in RAN1 LS R1-2306259 [2].

However, as analyzed above, the legacy application time of MAC-CE has been defined in Rel-17, how to consider the relationship between legacy application time of MAC-CE and the current mobility latency model defined in 38.300 running CR should be first determined.

Observation 1: The legacy application time of MAC-CE has been defined in Rel-17, how to consider the relationship between legacy application time of MAC-CE and the current mobility latency model defined in 38.300 running CR should be first determined.

Proposal 5: If SSB based fine synchronization is performed before cell switch, TCI state switch delay is needed, which may include the beam application time (BAT) mentioned in RAN1.

Tinterruption

In the last meeting, RAN4 discussed Tinterruption, and the related WF is duplicated below:

<Way Forward> FFS the following proposals: - Proposals - Proposal 1 (CATT, CTC, CMCC, ZTE, OPPO): The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd - Proposal 2 (Nokia): Interruption caused by LTM handover shall not exceed the interruption time caused by L3 handover - Proposal 3 (Huawei): There is almost no interruption during cell switch procedure when target Pcell/SCell is current SCell/PCell. - Proposal 4 (CATT, vivo): - For RACH-based cell switch, T_interruption at least include the time of Tprocessing,2 and T_IU. - For RACH-less cell switch, T_interruption at least include T_processing,2

We support Proposal 1. For Proposal 4, considering Tsearch, TΔ , Tmargin can be 0 under certain conditions, so for RACH-based cell switch, Tinterruption at least include the time of Tprocessing,2 and TIU, and for RACH-less cell switch, Tinterruption at least include Tprocessing,2.

Proposal 6: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 7:

- For RACH-based cell switch, Tinterruption at least include the time of Tprocessing,2 and TIU.

- For RACH-less cell switch, Tinterruption at least include Tprocessing,2

3. Conclusion

In this paper, we provide our views on L1/L2 inter-cell mobility delay requirements. From this discussion we have derived the following observation and proposals:

Proposal 1: RAN4 to discuss whether the legacy application time of MAC-CE has been included in the current mobility latency model defined in 38.300 running CR.

- The legacy application time of MAC-CE has been included in the current mobility latency model, as 3Nslotsubframe,μ and BeamAppTime-r17 have been already defined in Rel-17.

Proposal 2: If the legacy application time of MAC-CE has not been included in Tprocessing,2, additional time for beam application will be introduced for LTM cell switching time. Otherwise, RAN4 should consider the impact of enhancement of BAT compared with the legacy application time of MAC-CE on Tprocessing,2 /T LTM_processing.

Proposal 3: The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms

- TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms.

Proposal 4: When TCI state is indicated together with cell switch command, only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

Observation 1: The legacy application time of MAC-CE has been defined in Rel-17, how to consider the relationship between legacy application time of MAC-CE and the current mobility latency model defined in 38.300 running CR should be first determined.

Proposal 5: If SSB based fine synchronization is performed before cell switch, TCI state switch delay is needed, which may include the beam application time (BAT) mentioned in RAN1.

Proposal 6: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd.

Proposal 7:

- For RACH-based cell switch, Tinterruption at least include the time of Tprocessing,2 and TIU.

- For RACH-less cell switch, Tinterruption at least include Tprocessing,2.

4. Reference

- R4-2310160 WF on NR mobility enhancements (part 1), MediaTek Inc.

- R1-2306259 LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM, RAN1

- R4-2307400 Discussion on L1/L2 inter-cell mobility delay requirements, CATT