3GPP TSG-RAN WG4 Meeting # 108	           R4-2311654

Toulouse, FR, Aug 21th – Aug 25th, 2023

Title:	Reply LS on time gap between a PDCCH order and the corresponding PRACH transmission for LTM

Response to:	R1-2304276

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	RAN WG 4

To:	RAN WG 1

Cc:

Contact person:

Name:	Lingyu Gao

E-mail Address:	gaolingyu@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1 Overall description

RAN4 thanks RAN1 for the LS sent in R1-2304276 asking about the time gap between a PDCCH order and the corresponding PRACH transmission for LTM.

In last and this meeting, RAN4 has discussed the existing time components in RAN1 spec and the additional latency. RAN4 confirmed that final delay requirements of PDCCH ordered RACH will be captured in RAN1 spec as legacy, and RAN4 sincerely asks RAN1 to consider potential spec update based on RAN4’s following feedback.

RAN4 feedback:

- For the need for any update is required to ΔBWPSwitching, ΔDelay,

- For ΔDelay: RAN4 has agreed to not change the component.

- For ∆BWPSwitching: RAN4 has agreed to remove the component.

- For the need for additional latency, the specific components and the corresponding value will be further discussed in RAN4,

- 1 SSB occasion for T/F tracking if the total delay before UL transmission > 160ms or tracking is not configured before PDCCH order.

- The time for RF retuning and baseband processing. FFS: the specific value.

- 1 SSB occasion for T/F tracking provided that:

- The total delay (the tracking period and other time components before UL transmission) is larger than 160ms or tracking, or

- Tracking is not configured before PDCCH order.

- The time for RF retuning and baseband processing.

- RACH occasion (RO) is outside active BWP.

- For any impact/interruption on UL Tx of serving cell due to the PRACH Tx on a candidate cell that is not a current serving cell with PUCCH/PUSCH,

- If RACH occasion (RO) is outside active BWP, there will be interruption on UL Tx of all the serving cells due to RF retuning.

- There will be interruption on UL Tx of all the serving cells after RACH transmission due to returning to serving cell.

- It is suggested to introduce the scheduling restriction for UL of all the serving cells before and after RACH transmission and the relevant requirements will be captured in RAN4 spec.

Note: If RAN1 makes progress on the issue, RAN4 should further capture it in the spec.

2 Actions

To RAN WG1 group

ACTION: 	RAN4 sincerely asks RAN1 to take into account the above feedback into consideration.

3 Dates of Next TSG RAN WG4 Meetings

TSG-RAN WG4 Meeting #108-bis	9 Oct – 13 Oct 2023		     Xiamen , CN

TSG-RAN WG4 Meeting #109	13 Nov –17 Nov 2023	     Chicago , US