3GPP TSG-RAN WG4 Meeting # 108	            R4-2311657

Toulouse, FR, Aug 21th – Aug 25th, 2023

Agenda Item:	8.24.5

Source: 	CATT

Title: 	Discussion on enhanced CHO configurations

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, WF on NR Mobility Enhancements RRM requirements (part 2) was approved [1]. In this contribution, some open issues on enhanced CHO configurations are further discussed.

2. Discussion

CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1: (CATT, ZTE)

    - consider DCHOwithCPAC = Max [DCHO,  DCPAC] as baseline. TEvent_DU needs to be updated.

      - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

      - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

      - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

      - TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change

  - Option 1a: (Ericsson)

    - DCHOwithCPAC = Max [DCHO,  DCPAC].

      - DCHO = TRRC + TEvent_DU + Tcell_index_identify + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - DCPAC = TRRC_delay + TEvent_DU + Tcell_index_identify + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow legacy except from update of TEvent_DU and  Tcell_index_identify

  - Option 2: (CMCC)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution if

      - CPC/CPA and CHO conditions are satisfied simultaneously

      - CPC/CPA condition is fulfilled firstly

  - Option 3: (MTK, Apple, HW, QC)

    - similar as obj.3, i.e., DCHOwithPCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

      - Tprocessing = 25 ms

      - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

  - Option 4: (vivo)

    - if CHO and CPC conditions are met simultaneously or CPC conditions is met firstly and CHO condition is met later, DCHOwithCPC_PCell = TRRC_delay + max (TEvent_DU_PCell + Tmeasure_PCell, TEvent_DU_PSCell + Tmeasure_PSCell) + Tinterrupt + TCHO_executio

PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1: (CATT, ZTE)

    - consider DCHOwithCPAC = Max [DCHO,  DCPAC] as baseline. TEvent_DU needs to be updated.

      - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

      - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

      - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

  - Option 1a: (E///)

    - DCHOwithCPAC = Max [DCHO,  DCPAC].

      - DCHO = TRRC + TEvent_DU + Tcell_index_identify + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - DCPAC = TRRC_delay + TEvent_DU + Tcell_index_identify + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow legacy except from update of TEvent_DU and  Tcell_index_identify

  - Option 2: (MTK)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

      - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

      - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

  - Option 2a: (Apple)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, TEvent_DU needs to be updated

  - Option 3: (vivo)

    - DCHOwithCPC_PSCell = TRRC_delay + max (TEvent_DU_PCell + Tmeasure_PCell, TEvent_DU_PSCell + Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

  - Option 4: (HW)

    - Additional Tmeasure_PCell is supposed to be added:

    - DCHOwithcandidatePSCell_PCell = = TRRC + TEvent_DU + Tmeasure_PCell+ Tmeasure_PSCell + Tprocessing + TPSCell_ DU + T∆ + Tmargin + TCHO_execution.

  - Option 5: (QC)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

    - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

    - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

It was agreed that UE needs to wait to execute conditional PCell/PScell handover until both CHO and CPC conditions are met, so DCHOwithCPAC will depend on the larger handover delay of the CHO and CPC .Therfore,  Option 1 can be a baseline. In addition, since UE needs to wait until conditions for two events are met, the case will become more complex, so TEvent_DU may need to be updated [2].

Proposal 1: DCHOwithCPAC for PCell /PScell in Option 1 can be a baseline.

- TEvent_DU may need to be updated.

TEvent_DU

In the last meeting, the candidate solutions for TEvent_DU is duplicated below:

- Candidate solutions:

  - Option 1: (Apple)

    - TEvent_DU in PCell and PSCell handover delay requirement in FR1+FR2 NR-DC (obj. 4) is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC/CPA until conditions exist at the measurement reference point which will trigger the CHO and CPC/CPA.

  - Option 1a: (HW)

    - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until conditions fulfilled for PCell and associated candidate PSCell at the measurement reference point which will trigger the conditional handover

  - Option 1c: (vivo)

    - TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change

  - Option 2: (E///)

    - Resue the TS38.133 clause6.1.4

    - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CHO+CPAC command until a condition exists at the measurement reference point which will trigger the conditional handover (CHO).

In current spec, TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists at the measurement reference point which will trigger the conditional handover [3]. For FR1+FR2 NR-DC(obj. 4), considering that UE needs to wait until conditions for two events are met, the case will become more complex, so TEvent_DU may need to be updated. We believe that Option 1 is ok for us.

Proposal 2: TEvent_DU in PCell and PSCell handover delay requirement in FR1+FR2 NR-DC (obj. 4) is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC/CPA until conditions exist at the measurement reference point which will trigger the CHO and CPC/CPA.

3. Conclusion

In this paper, we provide our views on enhanced CHO configurations. From this discussion we have derived the following proposals:

Proposal 1: DCHOwithCPAC for PCell /PScell in Option 1 can be a baseline.

- TEvent_DU may need to be updated.

Proposal 2: TEvent_DU in PCell and PSCell handover delay requirement in FR1+FR2 NR-DC (obj. 4) is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC/CPA until conditions exist at the measurement reference point which will trigger the CHO and CPC/CPA.

4. Reference

- R4-2310064 WF on NR Mobility Enhancements RRM requirements (part 2), Apple

- R4-2307403 Discussion on enhanced CHO configurations, CATT

- 3GPP TS 38.133, Requirements for support of radio resource management