3GPP TSG-RAN WG4 Meeting # 108	R4-2311665

Toulouse, France, 21 – 25 August 2023

Agenda Item:	8.6.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	MPR simulation results for FR2-1 UL 256QAM

Document for:	Information

# 1.	Introduction

The revised work item on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved at TSG RAN#96 [1]. One of the objectives of this work item is to investigate and enable UL 256QAM for FR2-1.

One of the main tasks to investigate and enable UL 256QAM for FR2-1 is the MRP simulations to study the required MPR for the target signal quality and emission requirements. We proposed simulation assumptions and provided initial simulation results in [2]. This topic was further discussed at subsequent RAN4 meetings and the MRP simulation assumptions were included in the agreed WFs [3-6]. This contribution provides MPR simulation results for FR2-1 UL 256QAM according to the agreed simulation assumptions.

# 2.	Simulator assumptions

The following MPR simulation assumptions were agreed in [3]:

##### Issue 2-2-1: Antenna configuration and PA calibration point for MPR simulation - The MPR evaluation was performed by using 32 PAs, 16 for each polarizations within an antenna array for PC1/2/5 keeping align with the antenna configuration agreed in system level simulation. - PA calibration point should follow current definition in Spec 38.101-2: - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition. - Calculate MPR as total backoff needed for 256QAM from this calibration point. ##### Issue 2-2-2: Emission requirements for MPR simulation Follow the current Spec 38.101-2, otherwise specified - Occupied bandwidth (Table 6.5.1-1 in TS 38.101-2) - General NR spectrum emission mask for FR2-1 (Table 6.5.2.1-1 in TS 38.101-2) - NR ACLR1 for FR2-1 (Table 6.5.2.3-1 in TS 38.101-2) - General in-band emissions limit for FR2-1 (Tables 6.4.2.3.2-1 for PC1, 6.4.2.3.3-1 for PC2, 6.4.2.3.6-1 for PC5 in TS 38.101-2) - General NR spurious emission limits for FR2-1 (Table 6.5.3-2 in TS 38.101-2) - Maximum error vector magnitude (EVM) 3.5% for 29GHz and 39GHz as agreed in RAN4 #104-bis-e meeting. ##### Issue 2-2-3: Priority of PTRS correction method and MPR simulations - Option 1: PTRS based correction method is agreed prior to MPR simulations.

The following MPR simulation assumptions were agreed in [4]:

Issue 2-1-4: Whether consider other UE implementation-based methods to confine the MPR values for 256QAM Agreement: - The advanced UE implementation technologies are not precluded for MPR evaluation. - Companies should clarify what advanced UE implementation technologies are used in the simulation. - For MPR evaluation, either simulation results or measurement data or both can be provided Issue 3-2-2: PTRS correction methods Agreement: - For CP-OFDM - PTRS correction is implemented by de-rotation of each sub carrier in an OFDM symbol. The de-rotation angle is estimated as the frequency domain average of the phase rotation of all the PTRS tones in the allocation. - For DFT-s-OFDM: - PTRS correction is implemented by de-rotation of each time-domain symbol by the estimated instantaneous phase deviation. - The instantaneous phase deviation impacting a data symbol due to DUT phase noise is estimated by linearly interpolating between the phase deviations determined for the nearest neighbouring PTRS groups. The phase deviation for each PTRS group is determined as the time domain arithmetic mean phase deviation of all PTRS symbols in the group.

The following MPR simulation assumptions were agreed in [5]:

Issue 1-1-4 How to apply the phase noise profiles in MPR simulation Agreement: - For 29GHz: - Example 1 in TR38.803 for 29 GHz. - Example 2 in TR38.803. - New phase noise model using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from Qualcomm PSD0; 32 dB. 1; 3e3; 2.37; 1; 3.3. 2; 6e5; 4.7; 8e5; 5.3. 3; 8e5; 2.0; 1.1e6; 2.5. - Parameters from MTK - 39GHz - Example 2 in TR38.803. - Example 1-based using following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from vivo - Parameters from Anritsu - New phase noise model using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from MTK - Adopt min(example1, example2) as the phase noise profile for UL256QAM, where ‘example2’ refers to the example phase noise profiles in TR38.803 and ‘example1’ refers to Example 1-based for 39GHz. Issue 1-2-3 Simulation assumption for MPR Agreement: - Companies are expected to clarify EVM budge, PA model, Phase noise profile, PTRS configuration for CPE correction. Issue 2-2-2: the PTRS configuration for MPR requirement - FFS, companies are encouraged to evaluate the different PTRS configurations for narrow RB allocations by simulation based on the simulation assumptions in issue 1-1-3 and issue 1-1-4 in next meeting, especially for DFT-s-OFDM.

The following MPR simulation assumptions were agreed in [6]:

MPR Issue 1-1-1: Phase noise profiles evaluation for 29GHz - Proposals - Option 1: Both of new phase noise profiles from Qualcomm and MTK for 29GHz are feasible for MPR simulation. (Anritsu, MTK, Xiaomi, ZTE, vivo, Sony) - New phase noise profiles using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from Qualcomm PSD0; 32 dB. 1; 3e3; 2.37; 1; 3.3. 2; 6e5; 4.7; 8e5; 5.3. 3; 8e5; 2.0; 1.1e6; 2.5. - Parameters from MTK - Option 2: Example 1 in TR38.803 for 29GHz and new phase noise profiles from Qualcomm and MTK for 29GHz are feasible for MPR simulation. (LGE) Agreement: - Agreed on Option 1. Issue 1-1-2: Phase noise profiles evaluation for 39GHz - Proposals - Option 1: The new phase noise profile from MTK for 39GHz is feasible for MPR simulation. (Xiaomi, ZTE) - New phase noise profile using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from MTK - Option 2：The new phase noise profiles from MTK and adopting min(example1-based, example2) for 39GHz are feasible for MPR simulation, where ‘example1-based’ refers to the example phase noise profiles from vivo and Anritsu, ‘example2’ refers to the example phase noise profile in TR38.803. (Anritsu) - Option 3: Further discuss feasible phase noise profile for 39GHz. (Vivo, LGE) Agreement: - Take Option 1 as the starting point. MPR simulation Issue 2-2-1a: PTRS configuration in MRP simulation for CP-OFDM - Proposals - Adopt L-PTRS = 1 K-PTRS =2 as PTRS configuration. Issue 2-2-2a: PTRS configuration in MPR requirement for DFT-S-OFDM - Proposals - No PTRS configuration for wider RB allocations as starting point - Companies are encouraged to simulate the difference between with/without PTRS configuration under narrow RBs (<20 RB) allocations. Note: The above PTRS configurations for CP-OFDM and DFT-s-OFDM are just for MPR simulation, FFS whether the PTRS configurations are mandatory or optional in EVM test.

# 3.	Simulation results and observations

To simulate the MPR we used the maximum allocation placed at the center of the channel. The phase noise is expected to increase with increasing size of the allocation, and the effect of the IQ imbalance is maximized at the center of the channel.

In the simulations we use the multi-zero/pole model with Qualcomm’s parameters.

In all cases in the tables below we see the transmit power is limited by the EVM. This explains why the MPR is very similar for all the simulated power classes.

The MPR simulation results with different PCs are provided in Tables 1-3, and the EVM budget is provided in Table 4.

Table 1. MPR simulation results for 256QAM with PC1.

|  |  |  | Required back-off [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform |  | SCS [kHz] | Channel bandwidth [MHz] |  |  |  | Max |
|  |  |  | 50 | 100 | 200 | 400 |  |
| DFT-S-OFDM |  | 60 | 7.1 | 7.1 | 7.2 |  | 7.2 |
|  |  | 120 | 7.1 | 7.0 | 7.1 | 7.2 |  |

Table 2. MPR simulation results for 256QAM with PC2.

|  |  |  | Required back-off [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform |  | SCS [kHz] | Channel bandwidth [MHz] |  |  |  | Max |
|  |  |  | 50 | 100 | 200 | 400 |  |
| DFT-S-OFDM |  | 60 | 7.0 | 7.0 | 7.1 |  | 7.1 |
|  |  | 120 | 7.0 | 6.9 | 7.0 | 7.0 |  |

Table 3. MPR simulation results for 256QAM with PC5.

|  |  |  | Required back-off [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform |  | SCS [kHz] | Channel bandwidth [MHz] |  |  |  | Max |
|  |  |  | 50 | 100 | 200 | 400 |  |
| DFT-S-OFDM |  | 60 | 7.1 | 7.3 | 7.3 |  | 7.3 |
|  |  | 120 | 7.0 | 7.0 | 7.1 | 7.1 |  |

Table 4. EVM budget for DFT-s-OFDM at 29 GHz. RB start position 0, number of RBs 64, SCS 120 kHz.

| Tx EVM contributor | EVM (%) |
| --- | --- |
| Phase Noise+IQ Imbalance | 2.79 |
| PA Non-linearity & Transmitter | 2.11 |
| Total | 3.50 |

# 4.	Conclusion

This contribution has provided MPR simulation results for FR2-1 UL 256QAM according to the agreed simulation assumptions.

# References

[1]	RP-221862, “Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi.

[2]	R4-2211813, “Proposals on simulation assumptions for FR2-1 UL 256QAM MPR simulations”, Nokia, Nokia Shanghai Bell.

[3]	R4-2220810, “WF on FR2 UL 256QAM”, Xiaomi.

[4]	R4-2303709, “WF on FR2-1 UL 256QAM”, Xiaomi.

[5]	R4-2306603, “WF on FR2-1 UL 256 QAM”, Xiaomi.

[6]	R4-2310260, “WF on UL 256QAM requirements”, Xiaomi.