3GPP TSG-RAN WG4 Meeting #108	R4-2311677

Toulouse, France, 21st - 25th August 2023

Agenda item:			8.26.5.1

Source:	MediaTek Inc., Airbus, Eutelsat, SES, THALES

Title:	Discussion on RRM requirements for NTN above 10 GHz bands

Document for:	Discussion

# Introduction

Per RAN’s guidance,

“TSG RAN tasks RAN4 to scope the work needed to define the RRM requirements (i.e. beam steering related) necessary for NR-NTN UEs operating in the NTN example band (i.e. Ka band) based on associated assumptions of the UE architecture (including beam steering capabilities).”

As the rational provided in RP-230878 [1], it is expected that UE needs to use RX beamforming, similar to FR2.

This contribution is to scope the RRM requirement for NTN above 10GHz.

In this paper, our view on scope of RRM requirement for NTN above 10GHz and UL timing accuracy requirements are provided.

# Scoping on RRM requirement for NTN above 10GHz

In pratical, there could be various UE architectures for VSAT, depending on

- Whether UE can only form 1 Rx beam at a time or 2 Rx beams (for 2 different directions) at a time

- Mechanically-steered or electronically-steered UE beam

Agreement:

- At least consider the differentiation between mechanical and electronic steering;

  - FFS on phased array or parabolic;

  - Encourage satellite companies to provide the data to show the beam switching delay.

- As the baseline, assume that UE has the single beam towards one single satellite at a given time.

In RAN4#107, the following agreement has been reached in RF discussion [2]

In the spirit of defining minumiun (baseline) requirement, we think it makes sense to simplify the UE architecture by the following 2 categories, assuming that UE has the single beam towards one single satellite at a given time.

Option A) Single beam & Mechanically-steered UE, which may require several seconds for beam steering

Option B) Single beam & electronically-steered UE, which can sweep beams as fast as current smart phone.

In other words, UE with capability of forming 2 Rx beams at a time can be treated as a more advanced UE which would still fulfil the baseline requirements.

Proposal 1: Introduce 2 single RX beam UE architectures regarding the beam steering speed: Mechanically-steered and electronically-steered. Requirements are to be discussed separately for these 2 architectures. TBD whether to introduce UE capability.

In the following, we discuss the requirements under following cases:

- Delay requirements

  - These include IDLE mode requirements, handover, RRC re-establishment, RRC connection release with redirection, radio link monitoring, link recovery procedure (BFD/CBD), TCI-state switch (DL/UL), Measurement Procedure (L1/L3).

  - For smart phone, the FR2 requirements are extended from FR1 delays with a RX beam scaling factor (e.g., 8).

  - For NTN UE with Option A, since the beam steering time is typically longer than the delay with beam sweeping factor 8. It is not useful to really introduce the corresponding requirements.

  - For NTN UE with Option B, the requirements can be defined with probably a different beam sweeping factor which is subjected to the working phase discussion. E.g., Whether the beam sweeping factor can be reduced or be 1, assuming that UE has the ephemerics data to predict the best receiption direction for a targer cell.

- Scheduling/measurement restriction

  - This includes radio link monitoring, link recovery procedure (BFD/CBD), measurement procedure (L1/L3)

  - For smart phone, the requirements were defined in FR2 due to the limitation of Rx beam.

    - The scheduling restriction in FR2 was introduced because RAN4 allows UE to form fine beam for L1 measurement / PDSCH reception and rough beam for L3 SSB measurement. However, UE is not expected to form fine beam and rough beam at the same time. Besides, the beam direction of PDSCH could be different from the beam direction of neighbor cell’s SSB. Therefore, when PDSCH and L3 SSB collide on the same OFDM symbol, RAN4 decided to prioritize L3 SSB measurement, implying the scheduling restriction to PDSCH on the same OFDM symbol.

    - The measurement restriction in FR2 was introduced when 2 RS for L1 measurements (RLM, BFD, CBD, L1-RSRP) collided on the same OFDM symbol. UE may encounter a conflict when trying to form different Rx beams toward different directions to receive different RS. RAN4 did not explicitly specify the measurement delay requirements in this case.

  - For NTN UE with Option A, as we do not specify the measurement delay requirements, there is no need to further discuss the corresponding scheduling/measurement restriction.

  - For NTN UE with Option B, we expect scheduling/measurement restrictions are needed.

- UL Tx timing accuracy

  - There is already an ongoing discussion right now in RAN4. In our understanding, the UL Tx timing difference has nothing to do with the UE beam sweeping architecture. The main causes to the timing error are the DL timing estimation accuracy, RF circuit uncertainty, GNSS estimation accuracy, …, etc. In general, we believe the same requirement can be shared for both Option A and Option B.

- Measurement accuracy

  - the measurement accuracy for the new band needs to be specified, as performance part discussion. The FR2 requirements maybe a good starting point.

As a summary, the potential impact on RRM requirements are analysed as the following:

| TS 38.133 requirements | Option A (Mechanically-steered UE) | Option B (electronically-steered UE) | Description |
| --- | --- | --- | --- |
| RRC IDLE mobility | n.a. | Need requirement | To consider RX beam scaling factor |
| Handover | n.a. | Need requirement | To consider FR3 – FR3 HO To consider RX beam scaling factor |
| RRC Re-establishment | n.a. | Need requirement | To consider RX beam scaling factor |
| RRC Connection Release with Redirection | n.a. | Need requirement | To consider RX beam scaling factor |
| Timing requirement | Need requirement | Need requirement | To consider 60kHz/120kHz SCS |
| Radio Link Monitoring | n.a. | Need requirement | To consider RX beam scaling factor |
| Link Recovery procedure | n.a. | Need requirement | To consider RX beam scaling factor |
| DL TCI switch | n.a. | Need requirement | Modification for T L1-RSRP |
| Measurement Procedure | n.a. | Need requirement | Modify for RX beam scaling factor and scheduling restriction |
| Measurement Performance | Need requirement | Need requirement | RSRP, RSRQ, SINR, L1-RSRP |

Proposal 2: The potential scope on RRM requirement for NTN above 10GHz:

- RRC IDLE mobility

- Handover, RRC Re-establishment, RRC Connection Release with Redirection

- Radio Link Monitoring, Link Recovery procedure, DL TCI switch

- Measurement Procedure

- Measurement Performance

# UL timing accuracy

The UL SCS has be discussed in RAN#106 in RF [4], and it waits for RRM conclusion on the feasibility of using sub-carrier spacing values 60 and 120 kHz for NTN above 10 GHz.

RAN4 discussions are still ongoing on how to define the maximum timing errors (Te) for 60 kHz and 120 kHz sub-carrier spacing, which had not been defined for NTN FR1 in Release-17. Moreover, maximum timing errors (Te) defined in Release-17 may not be compatible with 60 kHz and 120 kHz sub-carrier spacing configurations due to lower CP length. RAN4 RRM is currently investigating maximum timing errors (Te_NTN) limits, maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN, for 60 kHz and 120 kHz sub-carrier spacing for R18 NTN Ka band.

Besides, the RAN4 discussions on timing requirement are still ongoing, as in the agreed RAN4 LS to RAN1/RAN2 [5]:

In RRM WF [6], the UL Timing Accuracy Requirements for UL SCSs of 60kHz and 120kHz has been discussed

### Issue 2-1: UE UL Timing Accuracy Requirements for UL SCSs of 60kHz and 120kHz

Agreement:

- The assumption of the maximum total positioning error due to UE location and Satellite position estimation error shall be tightened as below:

  - For UL SCS of 60kHz, [X] meters.

  - For UL SCS of 120kHz, [Y] meters

  - The above is applicable only when SSB SCS is equal to or higher than 120kHz

  - FFS on whether applicable to UE in mobile platform.

  - FFS on whether and how to connect the tightened UE positioning error to the advanced GNSS capability or UE type.

  - FFS whether to use different requirements for different types of devices defined in the RF session and/or different satellite types

  - FFS on different UE UL Timing Accuracy Requirements for different physical signals/channels, e.g. relaxed requirements for PRACH for certain PRACH formats, compared to other signal/channels.

as follows:

In the discussion on R17 NR NTN, the UE UL timing accuracy requirements can be formulated as:

Te_NTN = Te_TN + Te_GNSS + Te_SAT , where

- Te_TN: Legacy TN UE UL Timing Accuracy

- Te_GNSS: UE GNSS accuracy.

- Te_SAT : UE ephemeris est. error.

Besides, additional timing error can be introduced by the following factors:

- TA command resolution error (section 4.2, TS 38.213): 4*64 Tc

- TA adjustment accuracy (section 7.3.2, TS 38.133): 4*64 Tc

It can be observed that, for 120kHz SCS, where normal CP is 18*64 Tc, the timing error introduced by legacy Te_TN (3.5*64 Tc), TA command resolution error (4*64 Tc), TA adjustment accuracy (4*64 Tc) is already larger than 0.5 normal CP (9*64 Tc) and compromise the UL performance.

Observation 1: For 120kHz SCS, the timing error exceeds 0.5 normal CP, regardless the positioning error due to UE location.

Proposal 3: For 120kHz SCS with normal CP, no timing accuracy requirement for NTN above 10GHz.

Proposal 4: Consider Extended CP of 60kHz SCS for NTN above 10GHz.

# Summary

Proposal 1: Introduce 2 single RX beam UE architectures regarding the beam steering speed: Mechanically-steered and electronically-steered. Requirements are to be discussed separately for these 2 architectures. TBD whether to introduce UE capability.

Proposal 2: The potential scope on RRM requirement for NTN above 10GHz:

- RRC IDLE mobility

- Handover, RRC Re-establishment, RRC Connection Release with Redirection

- Radio Link Monitoring, Link Recovery procedure, DL TCI switch

- Measurement Procedure

- Measurement Performance

Observation 1: For 120kHz SCS, the timing error exceeds 0.5 normal CP, regardless the positioning error due to UE location.

Proposal 3: For 120kHz SCS with normal CP, no timing accuracy requirement for NTN above 10GHz.

Proposal 4: Consider Extended CP of 60kHz SCS for NTN above 10GHz.

# Reference

- RP-230878, Motivation to revise the Rel-18 WID NR_NTN_enh, Thales, 3GPP TSG RAN #99, June 2023.

- R4-2310483, WF for NR_NTN_enh_UERF, THALES, ZTE (Moderator), RAN4#107, May 2023.

- R4-2309717, Updates for NTN UE terminal requirements discussion in above 10 GHz, Thales, RAN4#107, May 2023.

- R4-2302998, WF for system parameters of above 10 GHz NTN band, THALES(Moderator), RAN4#106, Feb. 2023.

- LS on the system parameters for NTN above 10 GHz, RAN4#106, Feb. 2023.

- R4-2310168, WF on NR NTN enhancements RRM requirements, Qualcomm, RAN4#107, May 2023.