3GPP TSG-RAN WG4 Meeting #108	R4-2311681

Toulouse, France, 21st - 25th August 2023

Agenda item:			6.8.3

Source:	MediaTek Inc.

Title:	RRM core requirements maintenance for LTE NB-IoT/eMTC over NTN

Document for:	Discussion

# Introduction

In this contribution, our views are provided on the topics on RRM requirements RRM core requirements maintenance for LTE NB-IoT/eMTC over NTN.

# RRM core requirements maintenance

For NGSO, serving cell measurement - relaxation factor N

One issue is whether to revise the relaxation factor N, as Issue 1-2 in WF [1]. As the relaxation factor N is derived by n, which is configured by network, we do not see the strong need to introduce new tables for relaxation factor N, because the current specification already allows network parameters to differentiate the relaxation requirements.

Observation 1: The current specification already allows network parameters to differentiate the relaxation requirements.

Proposal 1: Do not introduce a scaling factor for the relaxation factor for NGSO compared to GSO. The relaxation factor can already be modulated by the RRC parameter numDRX-cyclesRelaxed specified by RAN2. (Option 1)

# Summary

In this contribution, the RRM requirements for NB-IoT/eMTC over NTN has been discussed and views on the open issues are provided. The observations and proposals are summarized below:

Observation 1: The current specification already allows network parameters to differentiate the relaxation requirements.

Proposal 1: Do not introduce a scaling factor for the relaxation factor for NGSO compared to GSO. The relaxation factor can already be modulated by the RRC parameter numDRX-cyclesRelaxed specified by RAN2. (Option 1)

# Reference

- R4-2310098, WF on RRM requirements for NTN NB-IoT/eMTC, MediaTek Inc., RAN4#107, May 2023.