3GPP TSG-RAN WG4 Meeting #108	R4-2311694

Toulouse, France, 21st - 25th August, 2023

Source:	Qualcomm Incorporated

Title:	RF requirements NR_FDD_ULn28_DLn75

Agenda item:	7.32.3

Document for:	Approval

# 1.	Introduction

In RAN4#107 several agreements were made on the RF requirements for NR FDD Band nA: UL_n28 and DL_n75. In this paper we further present our views on some of the remaining unresolved issues.

# 2. 	Discussion

In RAN4#106bis-e the agreed WF [1] indicates that there are several issues that needed further discussion. In this paper we present our views on some of the unresolved issues.

We think that the MSD for band combination FDD_ULn28_DLn75 can be based on the MSD for CA_n28-n75 which is captured in section 7.3A.4 of [2] and is given below:

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n28 | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

If there is agreement on extending the DL to support to 40 MHz for FDD_ULn28_DLn75 then the above table can be updated using the corresponding MSD value for CA_n28-n75.

Proposal 1: MSDs for band combination FDD_ULn28_DLn75 can be based on the values for CA_n28-n75 and can be specified as:

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n28 | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Also, if there is agreement on extending DL support to 40 MHz for FDD_ULn28_DLn75 then the above table can be updated for 40 MHz using the corresponding MSD value from CA_n28-n75.

Furthermore, we support UL coverage up to 30 MHz and DL support up to 40 MHz as proposed in option 1 for issue 2.3.1 in reference [1] as given below:

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 733 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL |

Proposal 2: FDD_ULn28_DLn75 can support UL coverage up to 30 MHz and DL coverage up to 40 MHz as given in the table below::

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 733 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL |

We think that it is reasonable for FDD_ULn28_DLn75 band to support the following band asymmetric bandwidth combinations in addition to the symmetric bandwidths:

| NR Operating Band | Channel BWs for UL (MHz) | Channel BWs for DL (MHz) |
| --- | --- | --- |
| Band nA: n75_DL / n28_UL | 5 | 10, 15, 20, 25, 30, 40 |
|  | 10 | 15, 20, 25, 30, 40 |
|  | 15 | 20, 25, 30, 40 |
|  | 20 | 25, 30, 40 |
|  | 25 | 30, 40 |
|  | 30 | 40 |

As can be seen these BW combinations limits the UL to a maximum of 30 MHz and the DL to a maximum of 40 MHz.

Proposal 3: In addition to the symmetric BWs enable FDD_ULn28_DLn75 to support the following asymmetric bandwidth combinations:

| NR Operating Band | Channel BWs for UL (MHz) | Channel BWs for DL (MHz) |
| --- | --- | --- |
| Band nA: 1432 – 1517 MHz DL / 703 – 733 MHz UL | 5 | 10, 15, 20, 25, 30, 40 |
|  | 10 | 15, 20, 25, 30, 40 |
|  | 15 | 20, 25, 30, 40 |
|  | 20 | 25, 30, 40 |
|  | 25 | 30, 40 |
|  | 30 | 40 |

# Conclusion

In this paper we further discuss our views on the RF requirements for NR FDD Band nA: UL_n28 and DL_n75. The following proposals are made:

Proposal 1: MSDs for band combination FDD_ULn28_DLn75 can be based on the values for CA_n28-n75 and can be specified as:

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n28 | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Also, if there is agreement on extending DL support to 40 MHz for FDD_ULn28_DLn75 then the above table can be updated for 40 MHz using the corresponding MSD value from CA_n28-n75.

Proposal 2: FDD_ULn28_DLn75 can support UL coverage up to 30 MHz and DL coverage up to 40 MHz as given in the table below:

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 733 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL |

Proposal 3: In addition to the symmetric BWs enable FDD_ULn28_DLn75 to support the following asymmetric bandwidth combinations:

| NR Operating Band | Channel BWs for UL (MHz) | Channel BWs for DL (MHz) |
| --- | --- | --- |
| Band nA: 1432 – 1517 MHz DL / 703 – 733 MHz UL | 5 | 10, 15, 20, 25, 30, 40 |
|  | 10 | 15, 20, 25, 30, 40 |
|  | 15 | 20, 25, 30, 40 |
|  | 20 | 25, 30, 40 |
|  | 25 | 30, 40 |
|  | 30 | 40 |

# References

[1] R4-2310262, WF on RF requirements NR_FDD_ULn28_DLn75_n76, RAN4#107, May 22nd – May 26th , 2023

[2] TS 38.101-1, NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.2.0 (2023-06-30)