3GPP TSG-RAN WG4 Meeting #108	R4-2311702

# Toulouse, France,  August 21 - August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | 0007 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.181: Applicaiton of unwanted emissions requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2023-8-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Applicable points for unwanted emissions requirements are not specified in the current text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add following sentence in unwanted emissions general section. “For SAN type 1-H the unwanted emission requirements are applied to sum of power over all TAB connectors for all the configurations supported by the SAN unless otherwise stated.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicable points for unwanted emissions requirements are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.108 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

6.6	Unwanted emissions

6.6.1	General

Unwanted emissions consist of out-of-band emissions and spurious emissions according to ITU definitions [4]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the SAN channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The out-of-band emissions requirement for the SAN transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE).

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

The values of ΔfOBUE are defined in table 6.6.1-1 for the SAN operating bands.

Table 6.6.1-1: Maximum offset of OBUE outside the downlink operating band

| SAN type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| SAN type 1-H | FDL,high – FDL,low < 100 MHz | 2×BWChannel |

For SAN type 1-H the unwanted emission requirements are applied to sum of power over all TAB connectors for all the configurations supported by the SAN unless otherwise stated.

There is in addition a requirement for occupied bandwidth.

6.6.2	Occupied bandwidth

6.6.2.1	Definition and applicability

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage b/2 of the total mean transmitted power. See also Recommendation ITU-R SM.328 [5].

The value of b/2 shall be taken as 0.5%.

The minimum requirement below may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in the present clause.

For SAN type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

--------------End of change-------------