3GPP TSG-RAN WG4 Meeting #108	R4-2311713

# Toulouse, France,  August 21 - August 25, 2023

Source: 	NEC

Title: 	Discussion on RF requirements for NCR-MT

Agenda Item:	8.28.2.2

Document for:	Approval

Introduction

This contribution discusses the RF requirements for NCR-MT and provide proposals on them.

Discussion

In the last meetings, system parameters and RF requirements for NCR were discussed and a WF was agreed [1]. In the WF, agreements were summarized, however, we can still see FFSs and options in it.

In this contribution, we further discuss RF requirements for NCR-MT and provide proposals.

Agreements on the TX requirements for NCR-MT can be summarized as below.

Table 1: Agreements on TX requirements for NCR-MT in RAN4#107

|  | WA NCR-MT | LA NCR-MT | note |
| --- | --- | --- | --- |
| Output power dynamics: | [reuse IAB-MT requirements] | reuse legacy UE requirements |  |
| Transmit ON-OFF power and transition period | IAB-MT approach or legacy UE approach | IAB-MT approach or legacy UE approach |  |
| Occupied bandwidth | follow ITU-R SM.328 | follow ITU-R SM.328 |  |
| ACLR | apply BS requirements | apply UE (PC2) requirements |  |
| OBUE/SEM | apply BS requirements | FFS | If the NCR supports simultaneous MT and FWD transmission, then the UL TX emissions requirements should be defined on the total emissions from MT and FWD. |
| Transmitter spurious emissions | follow Rel-17 spurious emission requirements | follow Rel-17 spurious emission requirements | If the NCR supports simultaneous MT and FWD transmission, then the UL TX emissions requirements should be defined on the total emissions from MT and FWD. |
| Transmitter intermodulation | IMT-MT approach or legacy UE approach | IMT-MT approach or legacy UE approach |  |

For Tx requirements for NCR-MT, following requirements remain open.

- Output power dynamics

- Transmit ON-OFF power and transition period

- OBUE/SEM

- Transmitter intermodulation

[Output power dynamics]

Requirement for WA NCR-MT was agreed with square bracket. As we do not have concern on this agreement, we propose to confirm to reuse IAB-MT requirements.

Proposal 1:

For WA NCR-MT output power dynamics requirements, confirm to reuse IAB-MT requirements.

[Transmit ON-OFF power and transition period]

Two options (option 1: IAB-MT approach, option 2: legacy UE approach) were provided for both WA NCR-MT and LA NCR-MT. For WA NCR-MT, we think it would be difficult to follow the legacy UE approach when the WA NCR-MT output power is large. On the other hand, we think LA NCR-MT shall follow the legacy normal UE approach.

Proposal 2:

For transmit ON-OFF power and transition period requirements, follow IAB-MT approach for WA NCR-MT and follow legacy UE approach for LA NCR-MT.

[OBUE/SEM]

For WA NCR-MT, it was agreed that BS requirements are applied. It means OBUE requirements are applied to WA NCR-MT.  For consistency, OBUE requirements shall be applied to LA NCR-MT, too. Adopting OBUE requirements looks reasonable when we consider the agreement for the case when the NCR supports simultaneous MT and FWD transmission. OBUE is applied to NCR-Fwd. It is proposed to follow Rel-17 repeater approach for LA NCR-MT.

Proposal 3:

For LA NCR-MT OBUE/SEM requirements, follow Rel-17 repeater approach.

[Transmitter intermodulation]

Transmitter intermodulation requirements are to satisfy ACLR, OBUE/SEM, and transmitter spurious emissions requirements in the presence of an interfering signal. Therefore, it is difficult to decide to adopt one of the options provided in the WF, IAB-MT approach or legacy UE approach. We propose to adopt the corresponding approaches for ACLR, OBUE/SEM, and transmitter spurious emissions requirements. That is,

For WA NCR-MT: BS approach for ACLR and OBUE/SEM, rel-17 repeater approach for transmitter spurious emissions requirements.

For LA NCR-MT: legacy UE approach for ACLR, rel-17 repeater approach for OBUE/SEM and transmitter spurious emission requirements.

Proposal 4:

For transmitter intermodulation requirements, follow corresponding approaches for ACLR, OBUE/SEM, and transmitter spurious emission requirements.

Agreements on the RX requirements for NCR-MT can be summarized as below.

Table 2: Agreements on RX requirements for NCR-MT in RAN4#107

|  | WA NCR-MT | LA NCR-MT | note |
| --- | --- | --- | --- |
| REFSENS (NF) FR1 | 5 dB | 13 dB or legacy UE value |  |
| REFSENS (NF) FR2 | 10 dB (30 GHz) 12 dB (45 GHz) | 10 dB (30 GHz) 12 dB (45 GHz) |  |
| Dynamic range | Not to define | Not to define |  |
| ACS/IBB | Reuse IAB-MT requirements | FFS |  |
| OOBB | Reuse IAB-MT requirements | FFS |  |
| receiver intermodulation | Reuse IAB-MT requirements | FFS |  |
| In-channel selectivity | Not to define | Not to define |  |

For Rx requirements for NCR-MT, following requirements remain open for LA NCR-MT.

- NF assumption for REFSENS for FR1

- ACS/IBB

- OOBB

- Receiver intermodulation

LA NCR-MT receiver should support the Rx functionality as legacy UE. We do not see any special issues to be considered for LA NCR-MT RX. Therefore, we propose to follow the legacy UE approach for all remaining requirements.

Proposal 5:

For LA NCR-MT NF assumption for REFSENS for FR1, adopt legacy UE value

Proposal 6:

For LA NCR-MT ACS/IBB requirements, follow legacy UE approach

Proposal 7:

For LA NCR-MT OOBB requirements, follow legacy UE approach

Proposal 8:

For LA NCR-MT receiver intermodulation requirements, follow legacy UE approach

Conclusions

We have discussed the open issues on RF requirements for NCR-MT and provided the following proposals.

Proposal 1:

For WA NCR-MT output power dynamics requirements, confirm to reuse IAB-MT requirements.

Proposal 2:

For transmit ON-OFF power and transition period requirements, follow IAB-MT approach for WA NCR-MT and follow legacy UE approach for LA NCR-MT.

Proposal 3:

For LA NCR-MT OBUE/SEM requirements, follow Rel-17 repeater approach.

Proposal 4:

For transmitter intermodulation requirements, follow corresponding approaches for ACLR, OBUE/SEM, and transmitter spurious emission requirements.

Proposal 5:

For LA NCR-MT NF assumption for REFSENS for FR1, adopt legacy UE value

Proposal 6:

For LA NCR-MT ACS/IBB requirements, follow legacy UE approach

Proposal 7:

For LA NCR-MT OOBB requirements, follow legacy UE approach

Proposal 8:

For LA NCR-MT receiver intermodulation requirements, follow legacy UE approach

References

[1] R4-2309893, WF for system parameter and NCR RF requirements, ZTE