3GPP TSG-RAN WG4 Meeting # 108	R4-2311735

Toulouse, August 21st – August 25th, 2023

Title:	3Tx MSD Analysis for CA_n41A-n71A

Source:	Huawei, HiSilicon

Agenda item:	7.29.3.2

Document for:	Approval

# 1	Introduction

In the WID[1], the band combination CA_n41A-n71A is considered for 3Tx with CA power class 1.5. In the last meeting [2] we showed that since the second PA of a dual Tx band is placed on the diversity path, its contribution on the receiver is limited. Hence, we can consider the MSD analysis with 1Tx per band. Moreover the MSD analysis for 2Tx with PC1.5 CA_n41A-n71A is not already on the specification (in contrast to PC3 and PC2) so in this contribution the MSD analysis is provided.

# 2	MSD analysis of CA_n41A-n71A for CA PC1.5

The dual UL transmission on CA_n41A-n71A could cause an IMD4 (3fn71-fn41) on n71 receiver, hence an MSD analysis is needed. In WF [3] two options were considered for MSD test configurations. As shown below. With option 1, the UE could reach 29 dBm on n41 with 2Tx. However with option 2, the UE can only reach to 26 dBm on n41 with 2Tx. Therefore option 1 is more practical and it can reach to the maximum power level for PC1.5. As previously explained the second PA of n41 can be neglected in our MSD analysis and consider one PA per band.

Table 1 Maximum output power of the PAs per band for the MSD test configuration

| MSD test configuration | PC3 FDD band (n71) | PC1.5 TDD band (n41) |
| --- | --- | --- |
| option 1 | 23 dBm | 26 dBm |
| option 2 | 23 dBm | 23 dBm |

Proposal 1: For MSD test configuration, option 1 is chosen, as it can reach to PC1.5 UE power level by 2Tx on n41.

Observation 1: As the interference from the Tx PA on the diversity path is low, the 2Tx per band can be simplified to 1Tx per band.

The analysis was performed on the architecture depicted in Fig 1:

Figure 1 CA_n41A-n71A UE architecture

Table 2 and 3 summarize the analysis for the main and the diversity paths of the receiver and Table 4 provide with the MSD due to both paths. The output powers of the transmitters are considered as 23 dBm@n77 and 26dBm@n41 (with 1 Tx).

Table 2: IMD4 level to n41 Rx main path (PC1.5)

| Main Path | received interference, IMD4 (dBm) |
| --- | --- |
| Diplexer (L-H) | -68.0 |
| Antenna switch | -66.5 |
| Diplexer (H-H or L-L) | -60.5 |
| n71 SAW/Duplexer | -113.0 |
| n71 PA Forward mixing | -51.2 |
| n41 PA Forward mixing | -84.5 |
| n71 PA Reverse mixing | -99.7 |
| n41 PA Reverse mixing | -206.0 |
| Total | -50.5 |
| B71 LNA via filters | -79.2 |
| B71 LNA via PCB coupling | -105.2 |
| Total main IMD | -50.5 |

Table 3: IMD4 level to n41 Rx diversity path

| Diversity Path | received interference, IMD4 (dBm) |
| --- | --- |
| From main path | -65.5 |
| Diplexer | -129.5 |
| Antenna switch | -130.5 |
| Duplexer | -131.5 |
| n71 LNA via antenna ISO | -164.2 |
| n71 LNA via PCB coupling | -119.7 |
| Total div IMD | -65.5 |

Table 4: CA_n3-n78 MSD for n41 Rx due to IMD4 (PC1.5)

| MSD calculation parameters | received interference, IMD4 (dB) |
| --- | --- |
| MSD | 35.9 |
| SNR | -29.7 |
| MSD at SNR=-1 | 28.7 |

Proposal 2: Based on the above analysis the following MSD test point is proposed:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 28.7 | FDD | IMD4 |

# 3	Conclusion

Observation: As the interference from the Tx PA on the diversity path is low, the 2Tx per band can be simplified to 1Tx per band.

Proposal 1: For MSD test configuration, option 1 is chosen, as it can reach to PC1.5 UE power level by 2Tx on n41.

Proposal 2: Based on the above analysis the following MSD test point is proposed:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 28.7 | FDD | IMD4 |

# 4	Reference

[1]	“RP-231013, Revised WID for Low NR band 4Rx and 3Tx, OPPO.”

[2]	“R4-2307146, On UE MSD requirements for FWA UE supporting 3T for two bands, Huawei, HiSilicon.”

[3]	“R4-2310409, WF on UE RF requirements for 3Tx and 4Rx, OPPO.”