3GPP TSG-RAN WG4 Meeting #108 	R4-2311740

Toulouse, France, August 21-25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Advanced Receivers - Simulation results

Agenda item:	8.18.1.2

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenario. Updated agreements were made in RAN4#107 [1] for facilitating companies to provide simulation results for Phase I.

Simulation assumptions for Phase I

Table 1 provides an overview of the already agreed simulation assumptions for Phase I taken from the WF [1]:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver assumption for E-MMSE-IRC | WRX,1k,l=H1H(k,l)R-1 R= P1H1k,lH1Hk,l+Pjj=2NUEHjk,lHjHk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H, rk,l=rk, l-j=1NUEHjk,ldjk,l . |
| --- | --- |
| Reference receiver for phase I simulation | Consider both R-ML and E-IRC in initial evaluation stage |
| Rank allocation for the target and co-scheduled UEs, with 1 co-scheduled UE | 2Rx UE: 1+1 4Rx UE: 2+2 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for rank 1+1 |
| Antenna configuration | For rank 1+1: cover 2T2R For rank 2+2 |
| Channel model | Use TDLC300-100 when the rank of the target UE is 1 Use TDLA30-10 when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium Rank 2+2: ULA Low |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 For rank 1+1: Random PMI selection For rank 2+2: Orthogonal PMI selection |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | Only consider 1 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | For initial simulation in phase I, assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| MCS for the target UE | For rank 1: MCS 4 and MCS 13 For rank 2: MCS 13 and MCS 17 |
| PDSCH resource allocation for the target UE | Maximum transmission bandwidth configuration. Full OFDM symbol allocation. |

Based on the information in Table 1, we provide the simulation results on the cases of interest for this meeting, listed in Table 2.

Table 2 - Simulation Overview – study cases for genie-aided knowledge of all parameters + blind detection

| Case | # Co-scheduled UE | Rank target UE | Rank Co-scheduled UE | MCS target UE | Modulation order co-scheduled UE | MIMO | Channel model | FDRA allocation of Co-scheduled UE | Precoder selection for the Co-scheduled UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | MCS 4 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Full CHBW (52 PBRs) | random |
| 2 |  |  |  | MCS 13 |  |  |  |  |  |
| 3 |  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 |  | orthogonal |
| 4 |  |  |  |  | 16QAM |  |  |  |  |
| 5 |  |  |  |  | QPSK |  |  |  |  |
| 6 |  |  |  | MCS 17 | 64QAM |  |  |  |  |
| 7 |  |  |  |  | 16QAM |  |  |  |  |
| 8 |  |  |  |  | QPSK |  |  |  |  |
| 9 | 1 | 1 | 1 | MCS 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Full CHBW (52 PBRs) | random |
| 10 |  |  |  |  |  |  |  | Partial CHBW (0~25 PBRs) |  |
| 11 | 1 | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 | Full CHBW (52 PBRs) | orthogonal |
| 12 |  |  |  |  |  |  |  | Partial CHBW (0~25 PBRs) |  |
| 13 | 1 | 1 | 1 | MCS13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Full CHBW (52 PBRs) | random |
| 14 | 1 | 2 | 2 | MCS17 | 16QAM | 4Tx 4Rx ULA low | TDLA30-10 |  | orthogonal |
| 15 | 1 | 1 | 1 | MCS13 | 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 |  | random |
| Note1: All tests consider the use of MCS index Table 1. Note2: Cases 1-8 assume genie knowledge of all information while Cases 9-15 consider blind detection of the co-scheduled UE’s FDRA and DMRS port allocation. Cases 13-15 consider blind detection of modulation order of co-scheduled UE for R-ML receiver. |  |  |  |  |  |  |  |  |  |

Simulation Results

This section displays our current simulation results. Normalized throughput curves are presented below along with a black reference line to highlight the SNR @ 70% of maximum throughput for each receiver. Cases 1-8 assume genie knowledge of all the co-scheduled UE information and results are presented for a baseline receiver MMSE-IRC (IRC), E-MMSE-IRC (E-IRC) and R-ML receivers, summarized in Table 3. Cases 9-15 assume blind detection of the co-scheduled UE’s FDRA and DMRS port allocation, while cases 13-15 also assume blind detection of modulation order for R-ML receiver, are summarized in Table 4.

Some observations follow below:

Results show that both E-IRC and R-ML offer improved receiver performance over the IRC baseline for all the tested cases.

R-ML generally outperforms E-IRC but the improvement margin is reduced for higher modulation orders of the co-scheduled UE.

Blind detection of the co-scheduled UE’s FDRA and DMRS port allocation is feasible, offering a close to negligeable performance degradation when compared to a genie knowledge scenario.

Table 3 – Summary of Receiver performance with genie-aided knowledge (Cases 1-8).

| Case | # Co-Sch UE* | Rank target UE | Rank Co-Sch UE | MCS Target UE | MO* Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-scheduled UE | Precoder Co-Sch UE | Nokia |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | SNR@70%maxTP (dB) |  |  | Gain of R-ML | Gain of E-IRC | Gain of R-ML over E-IRC |
|  |  |  |  |  |  |  |  |  |  | R-ML | E-IRC | MMSE-IRC (baseline) |  |  |  |
| 1 | 1 | 1 | 1 | MCS4 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Full CHBW (52 PBRs) | random | 5.4 | 7.5 | 12.7 | 7.3 | 5.2 | 2.1 |
| 2 |  |  |  | MCS13 |  |  |  |  |  | 14.2 | 22.5 | 25.7 | 11.5 | 3.2 | 8.3 |
| 3 |  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 |  | orthogonal | 12.5 | 13.2 | 14.2 | 1.5 | 1 | 0.5 |
| 4 |  |  |  |  | 16QAM |  |  |  |  | 12.1 | 13.2 | 14.2 | 2.1 | 1 | 1.1 |
| 5 |  |  |  |  | QPSK |  |  |  |  | 10 | 13.2 | 14.2 | 4.2 | 1 | 3.2 |
| 6 |  |  |  | MCS17 | 64QAM |  |  |  |  | 18 | 17.5 | 18.3 | 0.3 | 0.8 | -0.5 |
| 7 |  |  |  |  | 16QAM |  |  |  |  | 15.4 | 17.4 | 18.3 | 2.7 | 0.9 | 1.8 |
| 8 |  |  |  |  | QPSK |  |  |  |  | 12.9 | 17.5 | 18.3 | 5.4 | 0.8 | 4.6 |
| Note1: All tests consider the use of MCS index Table 1. Note2: Cases 1-8 assume genie knowledge of all information. Note*: Co-scheduled UE (Co-Sch UE) and Modulation order (MO). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4 – Summary of receiver performance with blind parameter detection (Cases 9-15).

| Case | # Co-Sch UE* | Rank target UE | Rank Co-Sch UE | MCS Target UE | MO* Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-scheduled UE | Precoder Co-Sch UE | Nokia |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | SNR@70%maxTP (dB) |  |  |  | Gain of R-ML | Gain of E-IRC | Gain of R-ML over E-IRC |
|  |  |  |  |  |  |  |  |  |  | R-ML BD | Aided BD3 R-ML | E-IRC BD | MMSE-IRC Baseline |  |  |  |
| 9 | 1 | 1 | 1 | MCS13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Full CHBW (52 PBRs) | random | 14.2 | NA | 22.5 | 25.7 | 11.5 | 3.2 | 7.8 |
| 10 |  |  |  |  |  |  |  | Partial CHBW (0~25 PBRs) |  | 12.4 | NA | 14.7 | 17.6 | 5.2 | 2.9 | 2.3 |
| 11 | 1 | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 | Full CHBW (52 PBRs) | orthogonal | 12.5 | NA | 13.2 | 14.2 | 1.7 | 1 | 0.7 |
| 12 |  |  |  |  |  |  |  | Partial CHBW (0~25 PBRs) |  | 10.3 | NA | 10.4 | 11.1 | 0.8 | 0.7 | 0.1 |
| 13 | 1 | 1 | 1 | MCS13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Full CHBW (52 PBRs) | random | 21.3 | 18.4 | 22.4 | 25.7 | 4.4 | 3.3 | 1.1 |
| 14 | 1 | 2 | 2 | MCS17 | 16QAM | 4Tx 4Rx ULA low | TDLA30-10 |  | orthogonal | TBA | TBA | 17.3 | 18.1 | TBA | 0.8 | TBA |
| 15 | 1 | 1 | 1 | MCS13 | 16QAM | 2Tx 2Rx ULA medium | TDLC300-100 |  | random | 19.6 | 14.6 | 22.5 | 25.6 | 6 | 3.1 | 3.1 |
| Note1: All tests consider the use of MCS index Table 1. Note2: Cases 9-15 consider blind detection of co-scheduled UEs FDRA, DMRS ports. Cases 13-15 consider blind detection of modulation order of co-scheduled UE for R-ML receiver. Note3: Aided BD R-ML relies on ZP-CSI-RS being scheduled in MU-MIMO slot for blind detection of modulation order. Note*: Co-scheduled UE (Co-Sch UE) and Modulation order (MO). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Case 1:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | MCS4 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random |

Figure 1 - Case 1: 2T2R Rank 1+1 TDLC300-100, medium correlation, MCS4-QPSK, random precoder. An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 2:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 1 | 1 | MCS13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random |

Figure 2 -Case 2: 2T2R Rank 1+1 TDLC300-100, medium correlation, MCS13-QPSK, random precoder. An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 3:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 1 | 2 | 2 | MCS13 | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 3 - Case 3: 4T4R Rank 2+2 TDLA30-10, low correlation, MCS13-64QAM, orthogonal precoder. An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 4:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 1 | 2 | 2 | MCS13 | 16QAM | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 4 - Case 4: 4T4R Rank 2+2 TDLA30-10, low correlation, MCS13-16QAM, orthogonal precoder. An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 5:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 1 | 2 | 2 | MCS13 | QPSK | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 5 - Case 5: 4T4R Rank 2+2 TDLA30-10, low correlation, MCS13-QPSK, orthogonal precoder. An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 6:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 1 | 2 | 2 | MCS17 | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 6 – Case 6: 4T4R Rank 2+2 TDLA30-10, low correlation, MCS17-64QAM, orthogonal precoder. An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 7:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 1 | 2 | 2 | MCS17 | 16QAM | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 7 - Case 7: 4T4R Rank 2+2 TDLA30-10, low correlation, MCS17-16QAM, orthogonal precoder. An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 8:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 1 | 2 | 2 | MCS17 | QPSK | 4Tx 4Rx ULA low | TDLA30-10 | orthogonal |

Figure 8 - Case 8 (4T4R Rank 2+2 TDLA30-10, low correlation, MCS17-QPSK, orthogonal precoder). An additional light blue curve is added to showcase the blind detection (FDRA+DMRS port) performance of the R-ML receiver in this scenario.

Case 9:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-Sch UE | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 1 | 1 | 1 | MCS13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Full CHBW (52 PRBs) | random |

Figure 9 - Case 9: 2T2R Rank 1+1 TDLC300-100, medium correlation, MCS13-QPSK, random precoder, full FDRA CHBW allocation.

Case 10:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-Sch UE | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 1 | 1 | 1 | MCS13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | Partial CHBW (0 ~25 PRBs) | random |

Figure 10 - Case 10: 2T2R Rank 1+1 TDLC300-100, medium correlation, MCS13-QPSK, random precoder, partial FDRA CHBW allocation.

Case 11:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-Sch UE | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | 1 | 2 | 2 | MCS13 | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 | Full CHBW (52 PRBs) | orthogonal |

Figure 11 - Case 11: 4T4R Rank 2+2 TDLA30-10, low correlation, MCS13-64QAM, orthogonal precoder).

Case 12:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-Sch UE | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 1 | 2 | 2 | MCS13 | 64QAM | 4Tx 4Rx ULA low | TDLA30-10 | Partial CHBW (0 ~25 PRBs) | orthogonal |

Figure 12 - Case 12 (4T4R Rank 2+2 TDLA30-10, low correlation, MCS13-64QAM, orthogonal precoder).

Case 13:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-Sch UE | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 1 | 1 | 1 | MCS13 | 16QAM | 2Tx 2Rx ULA Medium | TDLC300-100 | Full CHBW (52 PRBs) | random |

Figure 13 - Case 13 (2T2R Rank 1+1 TDLC300-100, medium correlation, MCS13-16QAM, random precoder).

Case 14:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-Sch UE | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 1 | 2 | 2 | MCS17 | 16QAM | 4Tx 4Rx ULA Low | TDLA30-10 | Full CHBW (52 PRBs) | orthogonal |

Figure 14 - Case 14 (4T4R Rank 2+2 TDLA30-10, low correlation, MCS17-16QAM, orthogonal precoder).

Case 15:

| Case | # Co-Sch UE | Rank target UE | Rank Co-Sch UE | Modulation order Target UE1 | Modulation order Co-Sch UE1 | MIMO | Channel model | FDRA allocation of Co-Sch UE | Precoder selection for the Co-Sch UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 1 | 1 | 1 | MCS13 | QPSK | 2Tx 2Rx ULA Medium | TDLC300-100 | Full CHBW (52 PRBs) | random |

Figure 15 - Case 15 (2T2R Rank 1+1 TDLC300-100, medium correlation, MCS13-QPSK, random precoder).

Conclusion

This paper presents Nokia's current simulation results on the WID for advanced receivers for MU-MIMO scenario. It includes the configurations agreed in RAN#107 for the all the candidate receivers.

In the paper, the following Observations were made:

- Results show that both E-IRC and R-ML offer improved receiver performance over the IRC baseline for all the tested cases.

R-ML generally outperforms E-IRC but the improvement margin is reduced for higher modulation orders of the co-scheduled UE.

Blind detection of the co-scheduled UE’s FDRA and DMRS port allocation is feasible, offering a close to negligeable performance degradation when compared to a genie knowledge scenario.

- References

- R4-2309892 WF on advanced receiver for MU-MIMO scenario