3GPP TSG-RAN WG4 Meeting #108 	R4-2311744

Toulouse, France, Aug 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On MultiRx Demodulation performance and CSI requirements - PDSCH

Agenda item:	8.7.4.2

Document for:	Discussion

Introduction

In RAN4#107 good progress was made on the topic of MultiRx performance and CSI requirements for PDSCH.

In the following we will provide Nokia’s view on the remaining open issues and provide new proposals when needed.

Discussion

Channel model

Issue 2-1-1: Channel model Agreement - At least consider TDLA30-75 for 100 MHz/120 kHz - FFS TDLA30-300 for 100 MHz/120 kHz.

Initial alignment can be done using the already agreed channel mode TDLA30-75. Extending further with additional channel model of TDLA30-300 can be decided later. We also see cases of high Doppler to be less important for MultiRx scenarios.

Initial simulation alignment can be done using the agreed channel model (TDLA30-75). High doppler is less important for MultiRx scenarios, hence it can be decided later if TDLA30-300 is to be included.

Make simulation initial alignment using TDLA30-75. Keep TDLA30-300 FFS for now.

PTRS Port for sDCI schemes

In RAN4#107 it was discussed how many PT-RS ports would be required for mDCI and sDCI. RAN4#107 decided on the mDCI case, however the sDCI case was still open (see [1]):

Issue 2-1-2: PTRS Port for sDCI schemes <Way forward > - Proposals - Option 1: One PTRS port per TRP for sDCI schemes - Option 2: One PTRS port for sDCI SDM based scheme. - Recommended WF: Encourage comments if any

We see it critical especially for scenarios with joint demodulation (i.e. sDCI) to transmit PT-RS on each TRP as it cannot be assumed that each TRP will be received with the same phase difference. We also acknowledge the information provided in RAN4#107 contribution, that the capability parameter that ‘supportTwoPortDL-PTRS-r16’ is optional hence selecting “option1: One PTRS port per TRP for sDCI schemes” will indeed result in requirements only being relevant for UEs supporting two PT-RS ports.

It cannot be assumed that each TRP will be received with the same phase difference, hence there is likely a need for transmitting PT-RS on each TRP.

UE support of two PT-RS ports for sDCI is optional.

RAN4 to define requirements with one PTRS port pre TRP for sDCI SDM. RAN4 to additionally define requirements using one PT-RS port for sDCI SDM, if found to be feasible.

Time offset between TRPs

In RAN4#107 it was agreed to use time offset between the two TRP transmission points, but it was not yet decided which time offset to use for specific configurations (see WF [1]).

Issue 2-1-4: Time offset between TRPs Agreement: - Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}. - Further discuss which time offset to be used with certain configurations.

As the configurations to be used have not been decided yet and simulation alignment is pending, we do not see it feasible to decide on which time offset to be used with certain configurations in this meeting.

It is not feasible to discuss which time offsets to use for certain configurations before initial simulation alignment is finalized.

Make decision on which time offset to be used with certain testcases based on the initial simulation alignment and agreements concerning which test cases to define.

PDSCH rate matching in mTRP transmission

In RAN4#107 it was brought up, there the current specifications do not fully consider the case of mTRP transmission with relation to PT-RS allocation (see [3]). Based on the observations in [3] the following open issue was added to the WF (see [1]):

Issue 2-1-6: PDSCH rate matching in mTRP transmission <Way forward > - Proposals - Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP. - Option 2: PT-RS allocation does not overlap with any PDSCH allocation. - Recommended WF: Encourage comments if any

We have checked the specifications and based on our findings the specifications do not clearly cover the restrictions of PT-RS allocation between the two TRPs in case of mDCI PDSCH transmissions. The latest specifications were written already for Rel15 when there were no mDCI defined yet. Normally we expect that for mDCI it will be seen as two single (individual) TRP transmissions, hence it cannot be assumed, that PT-RS allocation does not overlap with any PDSCH allocation, as such “Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP” should be assumed unless we can get confirmation from RAN1 that “Option 2: PT-RS allocation does not overlap with any PDSCH allocation.” is how RAN1 understands the specification.

Based on our evaluation, the current specification only considers single TRP setup with relation to PT-RS configurations.

For mDCI configurations it can be assumed that each TRP is seen as an individual TRP, which means PT-RS allocation for TRP1 can overlap with PDSCH allocation for TRP2.

Use the following as baseline: “Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP.”. The baseline assumption can be re-evaluated if decided to ask RAN1 for clarification and RAN1 response does not match the baseline assumption.

Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous)

In RAN4#107 it was brought up (see [3]), that there might be the need for send an LS to RAN1 for clarification on PT-RS rate matching behavior for mTRP transmission if deemed ambiguous. The following was added to the WF (see [1]):

Issue 2-1-7: Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous) <Way forward > - Proposals - Option 1: Yes - Recommended WF: Encourage comments if any

In section 2.4 we have identified, that RAN1 likely has not considered the case of m-TRP with respect to PT-RS rate matching between the two TRPs. In case companies agree that RAN4 would need to ask RAN1 if the assumption can be made, that “PT-RS allocation does not overlap with any PDSCH allocation” we will support RAN4 sending an LS to RAN1 to clarify the RT-RS rate matching behavior for mTRP transmission.

RAN4 to send LS to RAN1 for specification clarification on PTRS rate matching behavior for mTRP transmission in case it cannot be agreed in RAN4 that “PT-RS allocation does not overlap with PDSCH allocation per TRP“.

Test cases and simulation parameters

The detailed simulation parameters for sDCI and mDCI were not discussed in RAN4#107. The WF therefore carry forward the proposals, which were provided before the meeting (see [1]):

Issue 2-1-8: Test cases and simulation parameters for sDCI SDM <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for sDCI based SDM scheme as presented in Tables 1 and 2. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any Issue 2-1-9: Test cases and simulation parameters for mDCI fully overlapping. <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based SDM scheme as presented in Tables 3 and 4. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any Issue 2-1-10: Test cases and simulation parameters for mDCI non-overlapping <Way forward > - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based non-overlapping scheme as presented in Tables 5 and 6. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any

For the simulation parameters the proposals are not fully aligned with the agreements done during RAN4#107 with relation to at least the following:

- frequency offset between TRPs has been agreed to be 600Hz

- Agreed correlation model should be used. Decision on which cross-talk values to use, can be decided based on aligned simulation results.

- No agreement is made yet on the modulation order to be used for requirement definition. This should be decided based on aligned simulation results.

- Since RAN4#107 decided to use XPOL channel model, two layers are required from each TRP.

In addition, as simulation alignment is still pending we do not see it relevant to decide on the test cases in this meeting.

Final test cases should be decided based on simulation alignment results.

RAN4 to select the testcases among aligned simulation cases during this meeting or once enough results are available.

Simulation parameters for sDCI SDM

Based on the existing proposal for sDCI SDM (see [1]) including taking into account the agreements in RAN4#107 we propose the following as simulation parameters for sDCI SDM case:

Use the following simulation parameters for sDCI SDM (changes from RAN4#107 proposals highlighted):

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000, 1001 | 1002, 1003 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Channel model |  |  |  | TDLA30-75 |  |
| Correlation/cross-talk model (as decided in RAN4#107) |  |  |  | XPOL γ=0.125 Cross-talk=-15dB,-12dB,-9db,-6dB[,-3dB, 0dB] |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Simulation parameters for mDCI fully overlapping

Based on the existing proposal for mDCI fully overlapping (see [1]) including taking into account the agreements in RAN4#107 we propose the following as simulation parameters for mDCI fully overlapping case:

Use the following simulation parameters for mDCI fully overlapping (changes from RAN4#107 proposals highlighted):

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000,1001 | 1002,1003 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Channel model |  |  |  | TDLA30-75 |  |
| Correlation/cross-talk model (as decided in RAN4#107) |  |  |  | XPOL γ=0.125 Cross-talk=-15dB,-12dB,-9db,-6dB[,-3dB, 0dB] |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Simulation parameters for mDCI non-overlapping

Non-overlapping setup can be simulated by providing half the available RBs to each TRP to simulate non-overlapping frequency bands. Alternatively, a simulation configuration which splits in time can be used.
In previous defined requirements for non-overlapping configuration in [2] section 5.2.3.1.12 for FR1 it was selected to split the available RBs to be used per TRP. Specific reference channels were defined to accommodate the setup.

In 38.101-4 section 5.2.3.1.12 FR1 mDCI non-overlapping configuration has been implemented by splitting the available RBs so each TRP is assigned half number of available RBs.

Define simulation parameters where the mDCI non-overlapping case is configured by splitting the available RBs equally between the two TRPs.

Conclusion

In this paper we have presented Nokia's view on the open issues with relation definition of PDSCH requirements for MultiRx Demodulation performance.

In the paper, the following Observations and Proposals were made:

Channel Model

- Initial simulation alignment can be done using the agreed channel model (TDLA30-75). High doppler is less important for MultiRx scenarios, hence it can be decided later if TDLA30-300 is to be included.

- Make simulation initial alignment using TDLA30-75. Keep TDLA30-300 FFS for now.

PTRS Port for sDCI schemes

It cannot be assumed that each TRP will be received with the same phase difference, hence there is likely a need for transmitting PT-RS on each TRP.

UE support of two PT-RS ports for sDCI is optional.

RAN4 to define requirements with one PTRS port per TRP for sDCI SDM. RAN4 to additionally define requirements using one PT-RS port for sDCI SDM, if found to be feasible.

Time offset between TRPs

It is not feasible to discuss which time offsets to use for certain configurations before initial simulation alignment is finalized.

Make decision on which time offset to be used with certain testcases based on the initial simulation alignment and agreements concerning which test cases to define.

PDSCH rate matching in mTRP transmission

Based on our evaluation, the current specification only consider single TRP setup with relation to PT-RS configurations.

For mDCI configurations it can be assumed that each TRP is seen as an individual TRP, which means PT-RS allocation for TRP1 can overlap with PDSCH allocation for TRP2.

Use the following as baseline: “Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP.”. The baseline assumption can be re-evaluated if decided to ask RAN1 for clarification and RAN1 response does not match the baseline assumption.

Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous)

RAN4 to send LS to RAN1 for specification clarification on PTRS rate matching behavior for mTRP transmission in case it cannot be agreed in RAN4 that “PT-RS allocation does not overlap with PDSCH allocation per TRP“.

Test cases and simulation parameters

Final test cases should be decided based on simulation alignment results.

RAN4 to select the testcases among aligned simulation cases during this meeting or once enough results are available.

Simulation parameters for sDCI SDM

Use the following simulation parameters for sDCI SDM (changes from RAN4#107 proposals highlighted):

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000, 1001 | 1002, 1003 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Channel model |  |  |  | TDLA30-75 |  |
| Correlation/cross-talk model (as decided in RAN4#107) |  |  |  | XPOL γ=0.125 Cross-talk=-15dB,-12dB,-9db,-6dB[,-3dB, 0dB] |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Simulation parameters for mDCI fully overlapping

Use the following simulation parameters for mDCI fully overlapping (changes from RAN4#107 proposals highlighted):

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000,1001 | 1002,1003 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | {-0.0625, 0.25} |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | {0, 600} |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Channel model |  |  |  | TDLA30-75 |  |
| Correlation/cross-talk model (as decided in RAN4#107) |  |  |  | XPOL γ=0.125 Cross-talk=-15dB,-12dB,-9db,-6dB[,-3dB, 0dB] |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Simulation parameters for mDCI non-overlapping

In 38.101-4 section 5.2.3.1.12 FR1 mDCI non-overlapping configuration has been implemented by splitting the available RBs so each TRP is assigned half number of available RBs.

Define simulation parameters where the mDCI non-overlapping case is configured by splitting the available RBs equally between the two TRPs.

- References

- R4-2309824 - WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

- 38.101-4 User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

(Release 18)

- R4-2307852 - Discussion on PDSCH requirements of FR2 Multi-Rx DL Demod (Mediatek)