3GPP TSG-RAN WG4 Meeting # 108	R4-2311755

# Toulouse, France, August 21 – August 25, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	requirements for LBLBLB CA_n5-n28-n105

Agenda Item:	8.3.2.1	CA configuration of CA_n5A-n28A-n105A	[FS_NR_sub1GHz_combo_enh]

Document for:	Approval

# Introduction

In RAN#100, the FS_NR_sub1GHz_combo_enh was extended by one quarter and the SID revised accordingly in [1], the main open case is the LBLBLB case CA_n5A-n28A-n105A. To ensure completion in one quarter, the UL configuration has been simplified by removing the critical 2UL CA_n28-n105 similar to what was done for the two-band fallback. In this contribution we provide our input on the architecture and evaluate potential MSDs.

# Discussion

## Architecture for LBLBLB combinations

With the CA_n8-n20-n28 combination being the first LBLBLB case where requirements were captured for 38.101-1 in the endorsed CR [1], CA_n5-n28-n105 is only the second of this kind. Some of the architectural assumptions can be reused but it should be clear that this new case brings further complexity compared to CA_n8-n20-n28, the differences are illustrated in Figure 1:

- CA_n8-n20-n28:

- Covers 257MHz + 202MHz for a two-antenna design at the top of LB making one of the antennas designs critical.

- Only used the lower 30MHz of band n28:

- Enabling the merge of n28 and n20 DL

- Keeping the n28A UL to n28A DL to a reasonable duplex gap

- Could reuse the n28A-n20 triplexer from the related ENDC combination

- n8 has significant distance to n28 allowing an easier multiplexing

- Only 5 filters for main: n28AUL + merged n28A-n20DL + n20UL + n8DL + n8UL

- Only 2 filter for diversity: merged n28A-n20DL+n8DL

- Although 3 antenna solution was used to derive requirements, a two-antenna solution is feasible (as shown in Figure 1 at the top):

- Antenna 1: n28AUL+(n28A+n20DL)+n20UL+n8DL

- Antenna 2: (n20A+n20DL)+n8UL+n8DL

- The only two critical duplex gaps are n20 and n8 but no additional or more critical duplex gap than the single band case.

- Instead, CA_n5-n28-n105, has a number of added complexities:

- Covers 282MHz at the bottom of LB for both antennas for a two-antenna design, making their designs even more critical

- Uses the lower the entire band n28

- Need the two most critical low bands n28 and n105 in terms on BW and duplex gap

- No gap between n105 and n28

- No option for co-banding

- n5 very close to n28 (even more if implemented with a band 26 filter)

- there are 6 filters for main + 3 for diversity

- there are 4 critical duplex gaps.

- Two antenna solution although potentially feasible (as shown in Figure 1 at the bottom) is unlikely but should be enabled by the delta T delta R requirements.

Figure 1: CA_n8-n20-n28 and CA_n5_n28-n105 spectrums across two antennas and CA_n5-n28 UL IMD3

## Delta T and Delta R

For Delta T and Delta R, as discussed in the above chapter, the specification must take into account:

- Co-banding of n5 with n26

- The critical design of the band n105, n28 and n26 duplexers to be further multiplexed in a LBLBLB configuration

- A two-antenna architecture in the future

- Potential split approach for band n28

- Existing Delta T and Delta R from 2 band fallbacks as shown in Table 1.

Table 1: existing LBLBLB case 2 band fallbacks ΔTIB,c and ΔRIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB) |  |  | ΔRIB,c for NR bands (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |  |  |  |
| CA_n8-n20-n28 | 0.8 | 0.7 | 0.7 | 0.3 | 0.2 | 0.2 |
| CA_n71-n28=CA_n105-n28 | 1.1 | 1.1 |  | 0.7 | 0.7 |  |
| CA_n5-n28 | 0.5 | 0.5 |  | 0.3 | 0.2 |  |
| CA_n5-n105 | 0.5 | 0.5 |  | - | - |  |

If the values for CA_n105-n28 copied from CA_n71-n28 already account for the complexity of multiplexing n105 and n28, the further addition of n5 and its support via an n26 filter is not accounted for and requires an additional 0.2dB to account from further loading and rejection requirement in band 28 which leads to the following proposal

Proposal for CA_ n5-n28-n105 ΔTIB,c and ΔRIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB) |  |  | ΔRIB,c for NR bands (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |  |  |  |
| CA_ n5-n28-n105 | 0.7 | 1.1 | 1.1 | 0.5 | 0.7 | 0.7 |

## MSD to n105 due to IMD3 of CA_n5-n28 UL

For this 3band case, the potential MSD to the third band due to IMDs should be checked for the two 2UL configurations:

- CA_n28-n105 UL is not supported but on top of the IMD3 in band n28 and n105, would generate IMD5 in band n5 DL too.

- CA_n5-n105 UL is such that band n28 DL is between the two bands ULs and thus there is no IMD issue, and the dual UL cross-band MSD is low enough compared to CA_n5-n28 1UL and 2UL cases that it can be ignored.

- However, CA_n5-n28 UL generates IMD3 into band n105 DL, as shown at the bottom of Figure 1, and high MSD should be anticipated.

It should be noted that this CA_n5-n28 UL IMD3 into n105 DL case is very similar to the CA_n20-n28 UL IMD3 into n8 shown in table 2 other than now the lower IMD3 is involved rather than the upper one.

Table 2: existing LBLBLB case 2 band fallbacks ΔTIB,c and ΔRIB,c

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20-n28 | n8 | N/A | 5 | 25 | 951.5 | 24.3 | FDD | IMD3 |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 717.5 | 5 | 25 | 772.5 | N/A | FDD | N/A |

The fact it is the lower IMD3 and band n28 is involved, will increase the MSD due to a few reasons:

- n28 UL distance to n105 is smaller than n20 to n8

- n28 UL filter is more critical than band n20 and the lower side is more critical resulting in lower cross-band isolation

- The lower IMD3 is often higher since the PA has higher gain at lower frequencies.

In order to account for this, we suggest increasing the MSD level by a few dBs: 2dB for higher level lower IMD3 vs upper and 4dB for lower isolation.

Proposal on n105 MSD from CA_n5-n28 UL IMD3:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n28 | 738.0 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | 25 | 649.5 | 30.3 | FDD | IMD3 |

# Conclusions

In this contribution, we have discussed the LBLBLB architecture and requirements for CA_n5-n28-n105 in comparison to its fallbacks and the LBLBLB example combination CA_n8-n20-n28 and make the following proposal.

Proposal for CA_ n5-n28-n105 ΔTIB,c and ΔRIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB) |  |  | ΔRIB,c for NR bands (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |  |  |  |
| CA_ n5-n28-n105 | 0.7 | 1.1 | 1.1 | 0.5 | 0.7 | 0.7 |

Proposal on n105 MSD from CA_n5-n28 UL IMD3:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n28 | 738.0 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n105 | N/A | 5 | 25 | 649.5 | 30.3 | FDD | IMD3 |

# References

- RP-231141 SID revision: Study on enhancement for sub-1GHz NR band combinations, Huawei, HiSilicon, RAN#100

- R4-2308574 CR for TS 38.101-1 to introduce CA_n8-n20-n28 and corresponding fallback configurations Huawei, HiSilicon, Skyworks Solutions Inc., Xiaomi, RAN4#107