3GPP TSG-RAN WG4 Meeting #108	R4-2311768

Toulouse, France, 21 – 25 August 2023

Title:	Ambiguities with the case {1,1,0,0} to {0,0,1,1}

Source:	Qualcomm Incorporated

Agenda item:	8.23.2.1

Release:	Rel-18

WI: 	NR_MC_enh-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4 agreed new capabilities [37, 39] to declare the switching time in special events that may occur. These capabilities can be used to explicitly declare the switching time in the so-called ambiguous case {1,1,0,0} to {0,0,1,1} where max of all four possible switching periods was agreed to be used as baseline earlier [21]. RAN4 also agreed a WF [40] in last RAN4 meeting where third solution for defining the switching period based on scheduling order for this case would be worked on. This paper discusses aspects related to the

# 2. 	Discussion

The switch between cases {1,1,0,0} to {0,0,1,1} means that the US switches between two band pairs. The capability [uplinkTxSwitchingPeriod1T1Tto1T1T] can be used to explicitly state what is the switching period applicable to this event, in case the UE is congested of grants on all four bands. In case there is more time between granted transmissions than the switching period, then as discussed also in the rel-16 context, there is no need to worry about the switching time.

Observation: There is no ambiguity for switching time and UE can declare the shortest switching period for case {1,1,0,0} to {0,0,1,1} switch with [uplinkTxSwitchingPeriod1T1Tto1T1T].

The grant order based switching in [40] would clarify which two of the four switching periods are applicable and then maximum of the two apply to this event similarly as above.  The WF states that the switching period is

min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}

Which we find erroneous interpretation. What if the grant order means the switching happens A-D (140 usec) and B-C (35 usec), and switching periods are A-C(35 usec) and B-D(35 usec). The formula would give:

min {max(35, 35), max(140, 35)} = 35 usec

But UE performed A-D switch that required 140 usec. If this formula is the agreement (and not the grant order based method), then the it means that UE needs to support min(all switching times).

WF also states that there are two new capabilities defined for this:

    - Introduce new per Tx chain-based Tx switching capability

    - Introduce optional UE capability on supporting the band ordering based approach to resolve ambiguity issue

It is not clear what these capabilities are.

Unfortunately we did not notice the online treatment of this WF in RAN4#107. we would have asked these questions and requested clarifications before agreeing this WF.

Proposal: Before progressing to specification work based on this WF, clarifications what these agreements mean is needed.

It should warrant a conversation if multiple ways to resolve the same ambiguity is needed.

# Conclusion

Observation: There is no ambiguity for switching time and UE can declare the shortest switching period for case {1,1,0,0} to {0,0,1,1} switch with [uplinkTxSwitchingPeriod1T1Tto1T1T].

Proposal: Before progressing to specification work based on this WF, clarifications what these agreements mean is needed.

# Reference

[1]	RP-222251, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #97e, Electronic meeting, September 12 - 16, 2022

[2] R4-2211508 (R1-2205502), “LS on UL Tx switching across 3 or 4 bands in Rel-18”, RAN1, Ran4#104, Aug 2022

[3]	R4-2204605, “Introduction of TX switching for non-collocated UL CA”, Ericsson, 3GPP TSG-RAN WG4 Meeting#102-e, Electronic meeting, 21 February – 3 March 2022

[5]	R1-2204724, “On multi-carrier UL Tx switching scheme”, MediaTek Inc.

[6]	RP-221880, “Discussion on target scenarios for Rel-18 UL Tx switching in NR Multi-carrier enhancements WI”, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022

[7]	R1-2205052, “Discussion on Rel-18 UL Tx switching”, Qualcomm, 3GPP TSG RAN WG1 #109-emeeting, e-Meeting, May 9th – May 20th, 2022

[8]	R4-2214044, “UL TX switching requirements for two TAGs”, RAN4#104e, Online, Aug 2022

[9]	R4-2214043, “UL TX switching requirements for multiple TAGs”, RAN4#104e, Online, Aug 2022

[10]	RP-222251, “Revised WID on Multi-carrier enhancements”, NTT DOCOMO, INC., 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[11]	RP-222672, “Moderator's summary for discussion [97e-19-R18-Multicarrier]”, Moderator (NTT DOCOMO, INC.), 3GPP TSG RAN Meeting #97-e, Electronic meeting, September 12 - 16, 2022

[12]	R4-2214463, “WF on UL Tx switching across 3/4 bands with single TAG”, China Telecom, RAN4#104e, Online, Aug 2022

[13]	R4-2214464, “Reply LS on UL Tx switching across 3 or 4 bands”, China Telecom, RAN4#104e, Online, Aug 2022

[14]	R4-2215163, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104e, Online, Aug 2022

[15]	R4-2215306, “LS on switching option capability for UL 2Tx-2Tx switching”, RAN2, RAN4#104Bis-e, Online, Oct 2022

[16]	R4-2217740, “WF on UL Tx switching across 3/4 bands with single TAG, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[17]	R4-2217741, “LS on Rel-18 UL Tx switching”, China Telecom, RAN4#104Bis-e, Online, Oct 2022

[18]	R4-2217742, “WF on UL Tx switching with multiple TAGs”, Ericsson, RAN4#104Bis-e, Online, Oct 2022

[19]	R4-2220546, “WF on UE Tx switching requirement for single TAG”, China Telecom, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[20]	R4-2220547, “WF on UE Tx swithing requirement for multiple TAG”, Ericsson, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[21]	R4-2220548, “LS on Rel-18 Tx switching”, China Telecom, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[22]	R4-2220118, “Topic summary for [105][138] NR_MC_enh_UERF”, Moderator(China Telecom), 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, November 14th - November 18th, 2022

[23]	R4-2219876, “UE behaviour in case of non-equal switching time”, Qualcomm, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[24]	R4-2219877, “UE requirements during overlaps due to NZ-MTTD”, Qualcomm, 3GPP TSG-RAN WG4 Meeting#105, Toulouse, France, 14th - 18th  November 2022

[25]	R4-2215569, “UL Outage time considerations for 2TAG”, Qualcomm, 3GPP TSG-RAN4 Meeting #104Bis-e, Oct 2022

[26] R4-2303693, “WF on Multi-carrier enhancements for NR”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[27]	R4-2303507, “LS on Rel-18 Multi-carrier enhancement for NR”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[28]	R4-2303719, “CR for 38.101-1: Time mask for switching across three or four uplink bands”, China Telecom, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[29]	R4-2215569, “UL Outage time considerations for 2TAG”, 3GPP TSG-RAN4 Meeting #104Bis-e, Oct 2022, Online

[30]	R4-2300822, “UE switching time in more complicated scenarios”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[31]	R4-2304127, “Switching period ambiguity”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-Bis-e, Online,, 17th April – 26th April 2023

[32]	R4-2302751, “UE ON-OFF time masks for non-equal switching time cases”, Qualcomm Incorporated,  3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[33]	R4-2305789, “3 and 4 band TX switching open issues and requirements, Qualcomm Incorporated,  3GPP TSG-RAN4 Meeting #106, Athens, Greece, 27th Feb – 3rd March 2023

[34]	R4-2305808, “draft CR on TX Switching when UE is configured for 3 or 4 bands UL”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-bis-e, Online,, 17th April – 26th April 2023

[35]	R4-2306621, “WF on Rel-18 Tx switching across 3/4 bands”, China Telecom, 3GPP TSG-RAN WG4 Meeting #106-bis-e, Electronic Meeting, 17 April - 26 April, 2023

[36]	R4-2304162, “Simultaneous switching”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #106-bis-e, Online,, 17th April – 26th April 2023

[37]	R4-2310270, “CR for 38.101-1: Time mask for switching across three or four uplink bands”, China Telecom,3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, 22 - 26 May, 2023

[38]	R4-2310503, “CR for 38.101-1 to clarify the time mask for switching with multiple TAGs”, Huawei, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[39]	R4-2310495, “LS on multi-carrier enhancement”, vivo, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023

[40]	R4-2310496, “WF on ambiguity issues for Tx switching accorss four bands”, Mediatek, 3GPP TSG-RAN WG4 Meeting #107, Incheon, KR, May 22 – May 26, 2023