3GPP TSG-RAN WG4 Meeting # 108	R4-2311777

# Toulouse, France, August 21 – August 25, 2023

Title: 	TP on 38.878: Introduction of analysis on the required information

Source: 	Qualcomm, Inc.

Agenda item:	8.18.1.2

Document for:	Approval

# Background

As per [1], many simulation assumptions for link level evaluation have been agreed. It’s time to submit the section on parameters for link level evaluation.

# Advanced receiver to cancel inter-user interference for MU-MIMO

## Scenario and interference modelling

## Receiver structure

## Analysis on the required information

The agreed list of required information is below:

- Information signaled by RRC

| Information | RAN4 Default assumption (If N/A, how could be obtained by the UE) | Signalling if RAN4 default assumption not valid |
| --- | --- | --- |
| The DMRS port information for the co-scheduled UE | N/A (Obtained by UE blind detection) | N/A |
| PRB bundling size for the co-scheduled UE Frequency domain resource allocation for the co-UE within each PRG of the target UE | UE assume in each its PRG, the resource allocation and precoding of the potential DMRS sequence aligned co-scheduled UE(s) in other DM-RS ports of different CDM group are aligned with PRG=2 or 4. | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not |
| DMRS power boosting for the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not |
| Time domain resource allocation information of the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not |
| Frequency domain resource allocation for the co-UE across different PRGs of the target UE: | N/A (Obtained by UE blind detection) | N/A |
| CSI-RS location of co-scheduled UE (Only required for R-ML) | UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE | Down-select to one of the below options in the next meeting: Option 1: No RRC signalling is needed Option 2: 1-bit RRC signaling |

- Information signaled by DCI

Based on RAN4’s evaluation, RAN4 observes that R-ML receiver can achieve better performance in most scenarios. To enable the implementation of R-ML receiver within feasible complexity, RAN4 has agreed that it is beneficial to have DCI based network assistant signalling to know the essential information related to the interfering layers associated with the co-scheduled UE(s).

| Bit field mapped to index | Content |
| --- | --- |
| 0 | No co-scheduled UE(s) which has same DMRS sequence as target UE exists |
| 1 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have QPSK scheduled |
| 2 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 16QAM scheduled |
| 3 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 64QAM scheduled |
| 4 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 256QAM scheduled |
| 5 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 1024QAM scheduled |
| 6 | Not covered by cases corresponding to index 0~5. In each individual PRB allocated to the target UE, the following condition is satisfied: Only single modulation order is allocated for the co-scheduled UE(s) which has the same DMRS sequence as the target UE, if the co-scheduled UE(s) exist |
| 7 | Others |

- The existence of MU-MIMO DCI signalling is configured by RRC signalling.

- The field is intended to be included in a DCI which can be based on the format 1_1.

## Link performance characterization

### 4.4.1	 Parameters for link level evaluation

### 4.4.2	 Link level simulation results

### 4.4.3	 Summary of link level evaluation

# Conclusions

# Reference

- R4-2305914 WF on advanced receiver for MU-MIMO scenario. China Telecom.