3GPP TSG-RAN WG4 Meeting # 108	R4-2311781

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.32.2

Source:	Qualcomm Inc.

Title:	On NR SL relay

Document for:	Discussion

# Introduction

In this contribution, we present our view SL relay RRM requirements.

# Discussion

Observation: Upon checking the R16 RRC reconfiguration and R17 DRx interruption requirements, the following changes can cover the interruption requirements for R18 SL relay:

- For interruptions due to DRx transitions, we can revise the text to include SL relay in addition to V2X in the current spec:

### 12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX

Interruption on PCell/serving cell if configured due to V2X transitions between active and non-active during SL-DRX of V2X or other sidelink communications are allowed with up to 1% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured SL-DRX cycle is 640 ms or longer. When multiple SL-DRX cycles are configured, the shortest SL-DRX cycle is applied. Each interruption shall not exceed X slot as defined in table 12.7.4-1.

- For interruptions due to RRC reconfiguration, since SL relay is configured by SL-ConfigDedicatedNR according to 38.331, which is the same as V2X, we can revise the text to include SL relay in addition to V2X in the current spec:

### 12.7.1	Interruptions to WAN due to V2X Sidelink Communication

This clause contains the requirements related to the interruptions on the PCell/serving cell due to V2X and other sidelink communication.

A UE capable of V2X sidelink communication may indicate its interest (initiation or termination) in V2X and other sidelink communication to the connected gNodeB using IE SidelinkUEInformationNR in TS38.331[2].

The UE is allowed an interruption of up to the duration shown in table 12.7.1-1 on the PCell/serving cell during the RRC reconfiguration procedure that includes the V2X sidelink communication configuration message SL-ConfigDedicatedNR in TS 38.331[2] (setup and release). This interruption is for both uplink and downlink of the PCell/serving cell.

Table 12.7.1-1: Interruption length at V2X RRC reconfiguration

|  | NR Slot length (ms) | Interruption length (number of slots) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 3 |
| 2 | 0.25 | 5 |
| 3 | 0.125 | 9 |

Proposal: Revise the specification as above to include SL relay in RRC reconfiguration and DRx interruptions.