3GPP TSG-RAN WG4 Meeting # 108  		R4-2311790

Toulouse, France, Aug 21 - Aug 25, 2023

Agenda Item:	8.13.4.1

Source: 	CMCC

Title: 	TP to TR 38.876: RRM requirements for ATG network

Document for:	Approval

- Introduction

After discussion in RAN4#107 meeting, some agreements have been achieved. This TP provides the update of RRM requirements impact for ATG network.

- Reference

[1] R4-2309106, 3GPP TR 38.876 ATG v0.4.0

[2] R4-2310159, WF on NR ATG RRM requirements, CMCC

- TP to TR 38.876

The following TP to TR 38.876 is proposed for approval.

# 8	RRM requirements

<<  Start of text proposal >>

### 8.1 General

In R18, the WI on ATG doesn’t consider FR2, CA/DC and inter-RAT measurement scenario, the corresponding requirements are not applicable for R18 ATG. In light of ATG characteristics (e.g. ISD assumption of [14]-200km, maximum UE speed of 1200km/h, maximum distance between UE and BS is greater than [200]km), some RRM requirements are different from legacy ground-based network requirements, as listed in table 8-1.

<Editor’s Note: In table 8-1, the RRM requirements with square brackets are still under discussion, which may be updated according to the latest agreements. >

Table 8-1: RRM requirements for R18 ATG which are different from legacy ground-based network requirements

| Requirement | Item | Comments |
| --- | --- | --- |
| [Cell re-selection] | [Cell re-selection mechanism] | TBD |
|  | [Cell re-selection measurement requirements for intra-frequency measurement] | - [TBD for the impact of phase antenna array] |
|  | Cell re-selection measurement requirements for inter-frequency measurement | Define two set of cell detection requirements for ATG - Set 1: legacy R15 cell-reselection requirement - Set 2: R17 HST cell-reselection requirement |
| [Conditional handover] | [Conditional handover mechanism] | TBD |
|  | [Conditional handover requirements] | TBD |
| [RRC Connection Mobility Control] | - [RRC re-establishment requirement] | [TBD for the impact of phase antenna array] |
|  | - [RRC Connection Release with Redirection] | [TBD for the impact of phase antenna array] |
| UE transmit timing | Initial transmit timing requirements Te | Involve UE pre-compensation timing error, GNSS error=40m is assumed |
|  | Timing advance adjustment delay requirement | Introduce the mechanism of Koffset |
| [Signalling characteristics] | [Radio link monitoring] | [TBD for the impact of phase antenna array] |
|  | - [Link recovery procedures] | [TBD for the impact of phase antenna array] |
| Measurement procedure and requirements | [NR intra-frequency measurements] | For ATG TDD deployment, ‘deriveSSB-IndexFromCell’ is not always applicable for NR intra-frequency measurement. [TBD for the impact of phase antenna array] |
|  | [Inter-frequency measurements without measurement gap] | [TBD for the impact of phase antenna array] |
|  | [CSI-RS based intra-frequency measurements] | For ATG TDD deployment, ‘deriveSSB-IndexFromCell’ is not always applicable for CSI-RS based intra-frequency measurement. [TBD for the impact of phase antenna array] |
|  | [L1-RSRP/SINR measurements for Reporting] | [TBD for the impact of phase antenna array] |
|  | [Scheduling restrictions of UE performing measurements] | TBD |

For other RRM requirements, no new ATG specific requirements will be defined in R18.

<Other text to be added>

<< End of text proposal>>