3GPP TSG-RAN WG4 Meeting # 108		R4-2311794

Toulouse, France, Aug 21- Aug 25, 2023

Agenda Item:	8.13.5.1

Source: 	CMCC

Title: 			 Discussion on general issues for ATG

Document for:	Discussion

- Introduction

In last meeting, the agreement for channel model and doppler shift assumption have been achieved, the WF was approved in [1]. The agreements are as follows for information.

Issue 1-1: Channel model: Agreement: - Only consider single path AWGN channel with Doppler for ATG incremental requirements Issue 1-2: Doppler shift assumption: Agreement: - For FDD: Set Doppler as 200Hz for UL, 220Hz for DL - For TDD: Set Doppler as 500Hz

In this meeting, we further discuss the impact of new TDD pattern for UE and BS demodulation requirements and specification impact.

- Discussion

Issue 1-3: TDD pattern

Agreement:

- New TDD pattern together with the features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ can be considered as one of possible solution to mitigate the guard period impact for Rel-18 ATG scenario.

- Other solutions not precluded including the UE specific TA reporting as adopted in RRM

- For UE demodulation:

  - New UE demodulation requirement can be specified for the static TDD pattern (if introduced)

    - FFS on exact test case(s) which may be configured with new TDD pattern.

    - FFS on test applicability rules.

  - FFS on the impact of UE demodulation for all candidate options

- For BS demodulation: Further discuss test applicability rules and impact with new TDD pattern.

For ATG UE demodulation requirements, if RAN1 confirms that the features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ could be introduced, then new incremental test cases should be introduced with new TDD pattern, which including following test cases as we proposed in our series paper [2]

| TDD 40MHz 30kHz SCS | new TDD pattern: 30D4S6U | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |
|  |  |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

In order to reduce the test effort, we are fine to introduce the test applicability rule between TDD patterns. For ATG UE support the features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’,  if UE has passed the test cases with new TDD pattern, UE is allowed to skip the corresponding test cases with legacy TDD pattern.

Proposal 1: For ATG UE demodulation requirements, if UE has passed the test cases with new TDD pattern, UE is allowed to skip the corresponding test cases with legacy TDD pattern.

For ATG BS demodulation requirements, new TDD pattern should also be introduced for 30kHz SCS. The new incremental TDD test cases are as following:

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & FFS on new TDD pattern: 30D4S6U | 1T2R | 16QAM (MCS [16] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [28] in table 1) |
|  |  |  | 256QAM (MCS [22] in table 2) |

In order to reduce the test effort, we are fine to introduce the test applicability rule between TDD patterns. For BS supports multiple TDD UL-DL patterns including new TDD pattern 30D4S6U, only the supported TDD UL-DL pattern 30D4S6U shall be used for all tests.

Proposal 2: For ATG BS demodulation requirements, if BS supports multiple TDD UL-DL patterns including new TDD pattern 30D4S6U, only the supported TDD UL-DL pattern 30D4S6U shall be used for all tests.

Issue 1-4: Specification impact

- FFS in next meeting

For UE demodulation requirements:

New clause for ATG applicability of requirements should be introduced, for instance:

#### 5.1.1.x Applicability of requirements for ATG UE Type/capability; Test type; Test list; Applicability notes. ATG UE with 2RX; FR1 FDD; PDSCH; All tests in Clause 5.2.1.1.x. PDCCH; Test number 2, 3 and 4 in Clause 5.3.2.1.1, and All tests in Clause 5.3.2.1.2. CQI; All tests in Clause 6.2.2.1.1.1. FR1 TDD; PDSCH; Tests in Clause 5.2.1.2.x except TDD pattern 30D4S6U; If the Tests in Clause 5.2.1.2.x with TDD pattern 30D4S6U are passed, the test coverage can be considered fulfilled without executing Tests in Clause 5.2.1.2.x except TDD pattern 30D4S6U.. PDCCH; Test number 1 and 2 in Clause 5.3.2.2.1, and All tests in Clause 5.3.2.2.2. CQI; All tests in Clause 6.2.2.2.1.1. ATG UE with 4RX; FR1 FDD; PDSCH; Tests in Clause 5.2.2.1.x except TDD pattern 30D4S6U. PDCCH; Test number 2, 3 and 4 in Clause 5.3.3.1.1, and All tests in Clause 5.3.3.1.2. CQI; All tests in Clause 6.2.3.1.1.1. FR1 TDD; PDSCH; Tests in Clause 5.2.2.2.x except TDD pattern 30D4S6U; If the Tests in Clause 5.2.2.2.x with TDD pattern 30D4S6U are passed, the test coverage can be considered fulfilled without executing Tests in Clause 5.2.2.2.x except TDD pattern 30D4S6U.. PDCCH; Test number 1 and 2 in Clause 5.3.3.2.1, and All tests in Clause 5.3.3.2.2. CQI; All tests in Clause 6.2.3.2.1.1. ATG UE support ATG feature (K1 and HARQ process extension introduced by RAN1) with 2Rx; FR1 TDD; PDSCH; Tests in Clause 5.2.1.2.x with TDD pattern 30D4S6U. ATG UE support ATG feature (K1 and HARQ process extension introduced by RAN1) with 4Rx; FR1 TDD; PDSCH; Tests in Clause 5.2.2.2.x with TDD pattern 30D4S6U.

New clauses for ATG new incremental PDSCH requirements should be introduced, for instance:

5.2.1.1.x Minimum requirements for ATG (FR1 FDD 2Rx) 5.2.1.2.x Minimum requirements for ATG (FR1 TDD 2Rx) 5.2.2.1.x Minimum requirements for ATG (FR1 FDD 4Rx) 5.2.2.2.x Minimum requirements for ATG (FR1 TDD 4Rx)

For BS demodulation requirements:

New clause for ATG new incremental PUSCH requirements should be introduced in 38.104, for instance:

8.2.x Requirements for PUSCH with transform precoding disabled for ATG 11.2.1.x Requirements for PUSCH with transform precoding disabled for ATG

New clause for ATG applicability of PUSCH requirements should be introduced in 38.141-1 and 38.141-2, for instance:

8.1.2.x1 Applicability of PUSCH performance requirements for ATG Unless otherwise stated, for BS (type 1-O) which declaring support ATG, the requirements for PUSCH with transform precoding disabled for ATG in 8.2.x of 38.104, requirements for PUSCH with transform precoding disabled demodulation in Clause 8.2.1 (11.2.1.1) and the requirements for UCI multiplexing on PUSCH in Clause 8.2.3 (11.2.1.3) shall be mandatory tested. 8.1.2.x1.1 Applicability of requirements for TDD with different UL-DL patterns Unless otherwise stated, for 30kHz declared to be supported, if BS (type 1-O) supports multiple TDD UL-DL patterns including TDD pattern 30D4S6U, only the supported TDD UL-DL pattern 30D4S6U shall be used for all tests.

New clause for ATG applicability of PUCCH requirements should be introduced in 38.141-1 and 38.141-2, for instance:

8.1.2.x2 Applicability of PUCCH performance requirements for ATG Unless otherwise stated, for BS (type 1-O) which declaring support ATG, the performance requirements for PUCCH in 8.3 (11.3.1) are applied.

New clause for ATG applicability of PRACH requirements should be introduced in 38.141-1 and 38.141-2, for instance:

8.1.2.x3 Applicability of PRACH performance requirements for ATG Unless otherwise stated, for BS (type 1-O) which declaring support ATG, the performance requirements for PRACH in 8.4 (11.4.1) are applied.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: For ATG UE demodulation requirements, if UE has passed the test cases with new TDD pattern, UE is allowed to skip the corresponding test cases with legacy TDD pattern.

Proposal 2: For ATG BS demodulation requirements, if BS supports multiple TDD UL-DL patterns including new TDD pattern 30D4S6U, only the supported TDD UL-DL pattern 30D4S6U shall be used for all tests.

- Reference

[1] R4-2309796, WF for ATG demodulation requirements, CMCC.