3GPP TSG-RAN WG4 Meeting # 108		R4-2311798

Toulouse, France, Aug 21 - Aug 25, 2023

Agenda Item:	8.13.5.3

Source: 	CMCC

Title: 			 Discussion on BS demodulation requirements for ATG scenario

Document for:	Discussion

- Introduction

In last meeting, RAN4 has agreed the channel model, doppler shift assumption, test scope and simulation assumptions for UL demodulation requirements, we list the agreements for information.

Issue 1-1: Channel model: Agreement: - Only consider single path AWGN channel with Doppler for ATG incremental requirements Issue 1-2: Doppler shift assumption: Agreement: - For FDD: Set Doppler as 200Hz for UL, 220Hz for DL - For TDD: Set Doppler as 500Hz Issue 3-1-2: Bandwidth & SCS Agreement: - For the test case which reusing existing requirements, reuse all channel bandwidth requirements in TS38.104 8.2.1 and 8.2.3 for ATG PUSCH demodulation. - For the new test case with new dedicated ATG requirement, use 5MHz for FDD and 10MHz for TDD. Issue 3-1-3: Antenna Configuration Agreement: - For the new test case with new dedicated ATG requirement, 1T2R for 1-C/1-H and 1T2R for 1-O. Issue 3-1-5: Transform precoding Agreement: - Only consider CP-OFDM. Issue 3-1-6: PUSCH mapping type Agreement: - Type A and B for legacy requirements - Type A for new defined requirements - Test applicable rules for Type A and Type B should be reused. Issue 3-1-8: Other parameters Agreement: parameters; Value. HARQ; Maximum number of HARQ transmissions; 4. RV sequence; 0, 2, 3, 1. DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; pos 1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port; {0}. DM-RS sequence generation; NID0=0, nSCID =0. Time domain resource assignment; Start symbol; 0. Allocation length; 14. Frequency domain resource assignment; RB assignment; Full applicable test bandwidth. Frequency hopping; Disabled. Code block group based PUSCH transmission; Disabled. Issue 3-2-1: Test scope Agreement: - Reuse the existing TN BS requirement and applicable rules for ATG BS - No need to define incremental requirement. - PRACH requirements Issue 3-3-1: Test scope Agreement: - Reuse the existing normal PRACH requirements and same applicability rules for ATG BS - No need to define incremental requirement

In this contribution, we further provide our views on test setup and requirements.

- PUSCH requirements

Issue 3-1-4: Rank and MCS

Agreement:

- For the test case which reusing existing requirements, cover 16QAM, 64QAM at least

  - FFS for 256QAM supporting pending on UE RF session conclusion on the supporting UL 256QAM Tx

- For the new test case with new dedicated ATG requirement, cover 16QAM and 64QAM at least

  - FFS for 256QAM supporting pending on UE RF session conclusion on the supporting UL 256QAM Tx

In last meeting, RAN4 UE RF session has achieved the agreement as follows:

<Agreement>: UL 256QAM should be supported for ATG UE as optional feature for ATG.

Since UL 256QAM is an optional feature for ATG, the test cases should cover 256QAM.

For the new test case with new dedicated ATG requirements, based on our simulation results, the SNR at 70%TP of MCS20 (256QAM, R=682.5/1024) and MCS22(256QAM, R=754/1024) are both in the testable range. MCS 22 is preferred.

Proposal 1: For the test case which reusing existing requirements, cover 256QAM.

Proposal 2: For the new test case with new dedicated ATG requirement, cover 256QAM, MCS 22 in table 2 is preferred.

Issue 3-1-7: Test metric

- Option 1: Consider 70% and 30% throughput requirements for ATG PUSCH demodulation. (Ericsson, HW)

- Option 2: 70% throughput requirements (CMCC, ZTE, Ericsson (for new incremental requirements), Samsung)

The SNR at 30% TP is to test the HARQ performance. For test cases which reusing the existing requirements, we are fine to cover 30% TP. For new test cases with new dedicated ATG requirements, we prefer only consider 70% TP, since the channel quality is usually quite good with LOS propagation.

Proposal 3: For the test case which reusing existing requirements, the test metric cover SNR at 70% and 30% TP.

Proposal 4: For the new test case with new dedicated ATG requirements, the test metric only cover SNR at 70% TP.

Issue 3-1-1: Test scope for PUSCH

Agreement:

- Reuse the existing TN BS requirement and companies discuss which requirements can be taken as mandatory for ATG BS demodulation. Other features should be optional for the ATG BS and which cases can be tested should be based on BS manufacturer declaration

  - Candidate mandatory requirements

    - Normal PUSCH demodulation

    - UCI multiplexing on PUSCH

- Introduce new dedicated requirement with limited test cases for PUSCH, which only applied for BS declared to support ATG scenario.

We propose to introduce following new dedicated requirements for PUSCH, which only applied for BS declared to support ATG scenario. Due to the channel model is different for FDD and TDD, we propose to define separate requirements for FDD and TDD cases.

- New PUSCH test cases for ATG with FDD can be as follows.

| FDD 5 MHz 15kHz SCS AWGN+200Hz doppler | 1T2R | 16QAM (MCS [16] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [28] in table 1) |
|  |  | 256QAM (MCS [22] in table 2) |

- New PUSCH test cases with static TDD slot pattern can be as follows (if introduced):

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & FFS on new TDD pattern: 30D4S6U | 1T2R | 16QAM (MCS [16] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [28] in table 1) |
|  |  |  | 256QAM (MCS [22] in table 2) |

Proposal 5: Introduce following new dedicated requirements for PUSCH, which are separately for FDD and TDD cases.

| FDD 5 MHz 15kHz SCS AWGN+200Hz doppler | 1T2R | 16QAM (MCS [16] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [28] in table 1) |
|  |  | 256QAM (MCS [22] in table 2) |

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & FFS on new TDD pattern: 30D4S6U | 1T2R | 16QAM (MCS [16] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [28] in table 1) |
|  |  |  | 256QAM (MCS [22] in table 2) |

For the test case which reusing exiting requirement, which requirements should be taken as mandatory for ATG BS demodulation are FFS.

In our view, the normal PUSCH with transform precoding disabled demodulation requirements in Clause 8.2.1 and the requirements for UCI multiplexing on PUSCH in Clause 8.2.3 should be reused.

Proposal 6: For the existing TN BS requirement reused for ATG, the mandatory requirements for ATG should contain normal PUSCH with transform precoding disabled demodulation requirements in Clause 8.2.1 and the requirements for UCI multiplexing on PUSCH in Clause 8.2.3

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: For the test case which reusing existing requirements, cover 256QAM.

Proposal 2: For the new test case with new dedicated ATG requirement, cover 256QAM, MCS 22 in table 2 is preferred.

Proposal 3: For the test case which reusing existing requirements, the test metric cover SNR at 70% and 30% TP.

Proposal 4: For the new test case with new dedicated ATG requirements, the test metric only cover SNR at 70% TP.

Proposal 5: Introduce following new dedicated requirements for PUSCH, which are separately for FDD and TDD cases.

| FDD 5 MHz 15kHz SCS AWGN+200Hz doppler | 1T2R | 16QAM (MCS [16] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [28] in table 1) |
|  |  | 256QAM (MCS [22] in table 2) |

| TDD 10MHz 30kHz SCS AWGN+500Hz doppler | 7D1S2U & FFS on new TDD pattern: 30D4S6U | 1T2R | 16QAM (MCS [16] in table 1) |
| --- | --- | --- | --- |
|  |  |  | 64QAM (MCS [28] in table 1) |
|  |  |  | 256QAM (MCS [22] in table 2) |

Proposal 6: For the existing TN BS requirement reused for ATG, the mandatory requirements for ATG should contain normal PUSCH with transform precoding disabled demodulation requirements in Clause 8.2.1 and the requirements for UCI multiplexing on PUSCH in Clause 8.2.3

- Reference

[1] R4-2309796, WF for ATG demodulation requirements, CMCC.

[2] R4-2311799, Simulation results for ATG PUSCH demodulation, CMCC