# 3GPP TSG-RAN WG4 Meeting # 108	      R4-2311810

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.13.2.1

Source: 	CMCC

Title: 	Discussion on ATG RF UE feature list

Document for:	Discussion

# 1. Introduction

In last meeting, we finish most UE RF requirements including Tx part and Rx part. It’s time to focus on feature list discussion. In this contribution, we list all candidate feature list of ATG UE.

# 2. Discussion

Previous agreements are listed as below for reference.

<Agreement>: The rated maximum output power at maximum modulation order and full PRB configurations is reported by UE via new capability as in R4-2310400. <Agreement>: Upper limit for new capability for maximum conductive output power or TRP is 40dBm <Agreement>: The lower limit of conductive MOP or TRP of ATG UE is 23dBm - <Agreement>: The capability report granularity is 1dB as in R4-2306609 <Agreement>: Introduce the configured output power for ATG UE - Noted: How to define configured output power is still under discussion <Agreement>: UL 256QAM should be supported for ATG UE as optional feature for ATG

Based on above summary, the feature list for ATG UE is listed as below.

- X.NR_ATG-Core

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - X.NR_ATG-Core | x-1 | UE supported rated max output power | UE supported rated maximum output power at maximum modulation order and full PRB configurations |  | Yes | N/A | UE does not support ATG | Per UE | no | FR1 only | NA | Value range from 23dBm to 40dBm with 1dB as granularity | Mandatory with capability signalling |
| - X.NR_ATG-Core | x-2 | 64QAM for PUSCH | 64QAM for PUSCH |  | Yes | N/A |  | Per UE | no | FR1 only | NA |  | Mandatory without capability signalling |
| - X.NR_ATG-Core | x-3 | 256QAM for PDSCH | 256QAM for PDSCH |  | Yes | N/A |  | Per UE | no | FR1 only | NA |  | Mandatory without capability signalling |
| - X.NR_ATG-Core | x-4 | 256QAM for PUSCH | 256QAM for PUSCH |  | Yes | N/A | UE doesn’t support 256QAM for UL | Per UE | no | FR1 only | NA |  | Optional with capability signalling |

Proposal 1: it’s suggested to use above table as baseline for feature discussion.

Besides above features, there is also one IE that need to be updated and send the LS to RAN2.

- P-max for ATG UE.

- Legacy P-max is in the range between -30dBm and 33dBm with 6bits indication. For ATG UE, its max output power is extended to 40dBm which is larger than current P-max upper limit. Therefore legacy P-max range is not applicable anymore. If we assume upper limit as 42 dBm, which is 2dB larger than 40dBm as margin, the lower bound is about -21 dBm which is very near to minimum output power. Therefore, it seems current 6bit P-max indication is enough without any extension. We can only update upper and lower bound of P-max.

Proposal 2: P-max range for ATG is suggested to be updated with -21dBm lower bound and 42dBm upper bound.

Based on above analysis, draft LS to RAN2 about ATG UE is listed as in annex.

# 3. Conclusions

In this contribution, ATG UE feature list is discussed, it’s suggested use above table as basis for feature list discussion. Annex list LS to RAN2 about updating ATG UE RF related capability/IE.

# 4. Annex: LS

1	Overall description

During the discussion of ATG UE RF requirements, RAN4 find following ATG specific rated maximum output power capability and P-max IE need to be added or updated for ATG.

- ATG specific rated maximum output power capability

For the ATG UE, the rated maximum output power is declared via UE capability [RatedMOPATG] at maximum modulation order reported by ATG UE and full PRB configurations within the channel bandwidth of NR carrier unless otherwise stated. UE capability [RatedMOPATG] is an integer value in the range 23 to 40 dBm.

- ATG specific P-max

Considering rated maximum output power range has been updated, P-max range should also be updated accordingly and in the range -21 to 42dBm, i.e. both lower and upper bound should be enlarged by 9dB compared with legacy P-max definition in TS 38.331.

2. Actions:

To RAN2 group.

ACTION: RAN4 respectfully requests RAN2 group to add/update above capability and IE for ATG system.