# 3GPP TSG-RAN WG4 Meeting # 108	      R4-2311812

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.20.2

Source: 	CMCC

Title: 	Discussion on low power WUS architecture

Document for:	Discussion

# 1. Introduction

In RAN4 #107 meeting, one WF[1] and LS[2] are approved as starting point to help further discussion. In this contribution, we focus on the discussion of ACS related issues.

- Discussion

2.1 Categorize ACS requirements for different UE type

Based on RAN1 agreements, RAN4 approve to consider all the UE types mentioned in the SID, e.g. IoT devices, Wearable devices, and e-MBB devices. The cost aspect can also be considered for the design of LP-WUS/WUR scheme. Different types require different power consumption from a few uW to a few hundred uW. Different power consumption will lead to different NF. fig in [3] show the relationship between power consumption and NF. RF requirement analysis may need to categorized by different UE types with different power consumption assumption.

Observation 1: final ACS could be categorized by different UE types with different power consumption assumption.

2.2 target ACS when assume the same coverage as legacy network

Last meeting agreement is listed as below for information.

Issue 1-1-2: Target ACS value for LP-WUR receiver Agreement: - The methodology for guard RB is that at first conclude the relationship between guard RB and adjacent channel selectivity. - WUR ACS should be further discussed in the context of the guard RB design and main receiver test requirement.

Following show the target ACS value analysis based on the same interference level and REFSENSE degradation assumption as legacy UE.

| parameter | value |  | comment |
| --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |
| P_interference: Interference power for ACS requirement | =REFSENS Normal_UE + 45.5dB=-52.48dBm/5MHz | -25dBm | We use n41 as example. The same as the value for ACS testing considering such interference mainly comes from adjacent channel gNB power even for LP-WUR case |
| REFSENSE degradation due to adjacent channel interference | +14dB | -56.5dBm | This is also related to target coverage. +14dB is the same as legacy UE. But when target coverage shrinks for other UE device, large REFSENSE degradation is allowed, which will lead to smaller ACS. |
| ACS | 31.7 for 9dB NF assumption 25.7 for 15dB NF assumption 16.7 for 24dB NF assumption | 31.5 for 9dB NF assumption 25.5 for 15dB NF assumption 16.5 for 24dB NF assumption |  |

Based on above analysis, we have following observations.

Observation 2: if we assume the same coverage as normal UE, target ACS is about 32dB for 9dB NF, 26dB for 15dB NF and 17dB for 24dB NF.

2.3 guard RB evaluation based on filter characteristics

Last meeting agreement is listed as below for information:

Issue 1-1-3: Required number of guard RBs for LP-WUS ACS Agreement: - Companies provide the analyzed results with the RF impairment assumptions. RAN4 target to make decision on required number of guard RBs next meeting. - RF impairments and power cost impacts can be claimed by companies used in the analysis - For each RF impairment could be reported for different RF architecture - Companies are encouraged to provide the text proposals for RAN4 RF architecture evaluation.

Following list our filter evaluation analysis assumption which is almost aligned with in [3][4].

Table 1: LP-WUS evaluation scenarios

| NR RF channel BW | 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1 |
| WUS BW within NR channel | 5.04 MHz |
| WUS RB allocation (Note 1) | 24 RB in 5.04 MHz, total 28 RBs, or other number of RBs |
| WUS placement within NR channel | - Carrier edge; |
| Guard RB size of LP-WUS | - 0 RB, 1RB at each side, 2RBs at each side. - Only symmetric guard RB has been considered |
| ACS interferer | According to RF CBW |
| Filter characteristic | 2rd and 5th order Butterworth Both analog and digital filter can be considered |
| Filter passband BW | The same as WUS bandwidth (number of RBs) |
| LO frequency | Case 1: In the middle of WUS (modeling fixed WUS position) |

Following list the S21 of 2th and 5th order butterworth filters and related parameters.

Figure 1: Frequency response of 2rd Butterworth filters

Table 1: parameters for 2rd butterworth filter

| parameters | values |
| --- | --- |
| Bandpass butterworth filter order | 2rd |
| topology | Capacitive coupled, direct-coupled, serious capacitor |
| Input and output impedance | 50Ω |

Figure 2: Frequency response of 5rd Butterworth filters

Table 2: parameters for 5rd butterworth filter

| parameters | values |
| --- | --- |
| Bandpass butterworth filter order | 5th |
| topology | Capacitive coupled, direct-coupled, serious capacitor |
| Input and output impedance | 50Ω |

Based on above filter performance, 2PRB is enough to achieve 26dB adjacent channel rejection (i.e. 15dB NF) and 4PRB is enough to achieve 32dB adjacent channel rejection (i.e. 9dB NF).

Observation 3: 2PRB (15kHz SCS) is enough to achieve 26dB ACS and 4PRB is enough to achieve 32dB ACS for 5th ButterWorth filter.

2.4 Dedicated LP-WUS operation band

LS has been sent from RAN1 to reply the possibility of same CBW of LP-WUS as the carrier CBW. RAN1 currently doesn’t evaluate the case when LP-WUS occupies the whole carrier. As approved in last meeting  it’s RAN4’s responsibility to decide whether to consider dedicated LP-WUS operation band. Considering this is the study item, RAN4’s main responsibility is to identify RF architecture, ACS and guardband assumption. However ACS/guard band requirements are not needed for the case when LP-WUS occupies the whole operation band.  Besides, dedicated operation band will not introduce extra new architecture. Therefore, it seems we can wait for work item to discuss dedicated LP-WUS operation band.

Observation 4: the case when LP-WUS occupy the whole operation band will not require additional RF requirements or new architecture. It’s better to discuss this issue in formal work item stage.

If companies still have strong view of this dedicated LP-WUS operation band in this study item, n28 and n41 could be used as example band which have been commercially deployed by many operators among the world.

Observation 5: if RAN4 finally approve to define LP-WUS dedicated operation band, band 28 and band 41 are suggested as example band which has been globally deployed by many operators.

# 3. Conclusions

In this contribution, LP-WUR architecture and RF requirements are discussed with following observations:

Observation 1: final ACS could be categorized by different UE types with different power consumption assumption.

Observation 2: if we assume the same coverage as normal UE, target ACS is about 32dB for 9dB NF, 26dB for 15dB NF and 17dB for 24dB NF.

Observation 3: 2PRB (15kHz SCS) is enough to achieve 26dB ACS and 4PRB is enough to achieve 32dB ACS for 5th ButterWorth filter.

Observation 4: the case when LP-WUS occupy the whole operation band will not require additional RF requirements or new architecture. It’s better to discuss this issue in formal work item stage.

Observation 5: if RAN4 finally approve to define LP-WUS dedicated operation band, band 28 and band 41 are suggested as example band which has been globally deployed by many operators.

# 4. Reference

[1] R4-2306617, WF on LP-WUS, vivo

[2] R4-2306618, Reply LS to RAN1 on low-power wake-up receiver architectures, vivo

[3] R4-2307147, Further consideration on LP-WUS/WUR, Huawei, HiSilicon.

[4] R4-2308260, Discussions on low-power wake-up receiver architectures, vivo

[5] R4-2302276, Low-power wake-up receiver RF aspects, Qualcomm Inc.