3GPP TSG-RAN4 Meeting #108	R4-2311829

# Toulouse, France, 21th August 2023 - 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1696 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for NR CA band combinations with two SUL cells in Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2SUL_cell_combos_R18 |  |  |  |  |  | Date: |  |  | 2023-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce band combination as below: CA_n78C_n84A-n89A |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce band combination as below: CA_n78C_n84A-n89A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current spec can’t support band combination as below: CA_n78C_n84A-n89A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2C, 5.5C, 6.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==================Start of 1st changes==================

## 5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, Table 5.2C-2, Table 5.2C-3 and Table 5.2C-4, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

<< Unchanged omitted >>

Table 5.2C-4: Operating SUL band combination with inter-band CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_n78-n80 | n1, n78, n80 |
| CA_n1_n78-n81 | n1, n78, n81 |
| CA_n1_n78-n84 | n1, n78, n84 |
| CA_n3_n41-n80 | n3, n41, n80 |
| CA_n3_n78-n80 | n3, n78, n80 |
| CA_n3_n79-n80 | n3, n79, n80 |
| CA_n8_n78-n81 | n8, n78, n81 |
| CA_n28_n41-n83 | n28, n41, n83 |
| CA_n28_n79-n83 | n28, n79, n83 |
| CA_n41_n79-n80 | n41, n79, n80 |
| CA_n41_n79-n83 | n41, n79, n83 |
| CA_n41_n79-n95 | n41, n79, n95 |
| CA_n41_n79-n97 | n41, n79, n97 |
| CA_n41_n79-n98 | n41, n79, n98 |
| CA_n78_n1-n80 | n1, n78, n80 |
| CA_n78_n1-n81 | n1, n78, n81 |
| CA_n78_n1-n89 | n1, n78, n89 |
| CA_n78_n3-n84 | n3, n78, n84 |
| CA_n79_n41-n80 | n41, n79, n80 |
| CA_n79_n41-n83 | n41, n79, n83 |
| CA_n79_n41-n95 | n41, n79, n95 |
| CA_n79_n41-n97 | n41, n79, n97 |
| CA_n79_n41-n98 | n41, n79, n98 |
| CA_n28-n79_n41-n83 | n28, n41, n79, n83 |
| CA_n28-n41_n79-n83 | n28, n41, n79, n83 |
| CA_n41A-n95A_n79A-n98A | n41, n95, n79, n98 |
| CA_n41A-n98A_n79A-n95A | n41, n98, n79, n95 |
| CA_n41A-n83A_n79A-n98A | n41, n83, n79, n98 |
| CA_n41A-n83A_n79A-n95A | n41, n83, n79, n95 |
| CA_n78C_n80A-n84A | n78, n80, n84 |
| CA_n78C_n81A-n84A | n78, n81, n84 |
| CA_n78C_n84A-n89A | n78, n84, n89 |
| NOTE 1: If a UE is configured with a single cell consisting of a NR UL carrier and a corresponding NR SUL carrier, the switching time between NR UL carrier and the corresponding NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

==================End of 1st changes==================

==================Start of 2nd changes==================

5.5C	Configurations for SUL

The configuration tables for SUL describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 for a given release are still optional for UEs that support BCS4 or BCS5. , where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. Note that the minimum bandwidth is indicated only in BCS5 and BCS5 shall not be indicated together with BCS4 for a SUL configuration. For SUL band combinations including FR1 intra-band CA and with BCS4 or BCS5, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5.

<< Unchanged omitted >>

Table 5.5C-5: Supported channel bandwidths per SUL band combination with inter-band CA (two SUL cells)

| SUL band combination with CA | Uplink CA configuration or SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n98A_n79A-n95A | SUL_n41A-n98A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n98A | SUL_n41A-n83A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n95A | SUL_n41A-n83A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n78C_n80A-n84A | SUL_n78A-n80A SUL_n78A-n84A CA_n78C | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 CA_n78C_BCS1 | 0 |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n78C_n81A-n84A | SUL_n78A-n81A SUL_n78A-n84A CA_n78C | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 CA_n78C_BCS1 | 0 |
|  |  | n81 | 5, 10, 15, 20 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n78C_n84A-n89A | SUL_n78A-n84A SUL_n78A-n89A CA_n78C | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 CA_n78C_BCS1 | 0 |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n89 | 5, 10, 15, 20 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |

==================Start of 3rd changes=================

## 6.2C	Transmitter power for SUL

<< Unchanged omitted >>

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands)

| Band combination for SUL | ΔTIB,c for NR bands (dB)3 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |  |
| CA_n1_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n1_n78-n81 | 0.3 | 0.8 | 0.6 |
| CA_n1_n78-n84 | 0.6 | 0.8 | 0.6 |
| CA_n3_n41-n80 | 0.5 | 0.31 / 0.82 | 0.5 |
| CA_n3_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n3_n79-n80 | 0.3 | 0.8 | 0.3 |
| CA_n8_n78-n81 | 0.6 | 0.8 | 0.6 |
| CA_n28_n41-n83 | 0.3 | 0.3 | 0.3 |
| CA_n28_n79-n83 | 0.5 | 0.8 | 0.5 |
| CA_n41_n79-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n41_n79-n83 | 0.3 | 0.8 | 0.5 |
| CA_n41_n79-n95 | 0.3 | 0.8 | 0.3 |
| CA_n41_n79-n97 | 0.5 | 0.5 | 0.5 |
| CA_n41_n79-n98 | 0.3 | 0.8 | 0.3 |
| CA_n78_n1-n80 | 0.8 | 0.6 | 0.6 |
| CA_n78_n1-n81 | 0.8 | 0.3 | 0.6 |
| CA_n78_n1-n89 | 0.8 | 0.6 | 0.6 |
| CA_n78_n3-n84 | 0.8 | 0.6 | 0.6 |
| CA_n78_n80-n84 | 0.8 | 0.6 | 0.6 |
| CA_n78_n81-n84 | 0.8 | 0.6 | 0.3 |
| CA_n78_n84-n89 | 0.8 | 0.6 | 0.6 |
| CA_n79_n41-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n79_n41-n83 | 0.3 | 0.8 | 0.5 |
| CA_n79_n41-n95 | 0.3 | 0.8 | 0.3 |
| CA_n79_n41-n97 | 0.5 | 0.5 | 0.5 |
| CA_n79_n41-n98 | 0.3 | 0.8 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

## ==================End of 3rd changes=================