3GPP TSG-RAN WG4 Meeting # 108	R4-2311848

# Toulouse, France, August 21 – August 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3441 | rev |  | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction to CSSF outside gap R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are repeated requirements on CSSFoutside_gap,i when a UE being configure with MOs by the E-UTRAN PCell on an NR serving carrier in clause 9.1.5.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove duplicate wording from CSSF requirement in clause 9.1.5.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements on CSSFoutside_gap,i are repetitive when a UE being configure with MOs by the E-UTRAN PCell on an NR serving carrier. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

#### 9.1.5.1	Monitoring of multiple layers outside gaps

For a UE supporting concurrent gaps and when concurrent gaps are configured the carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types :

-	SSB-based intra-frequency measurement with no measurement gap in clause 9.2.5 and 9.2A.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps.

-	SSB-based intra-frequency measurement with no measurement gap in clause 9.2.5 and 9.2A.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps.

-	CSI-RS based intra-frequency measurement in clause 9.10.2, when none of CSI-RS resources for L3 measurement of this intra-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps.

-	CSI-RS based intra-frequency measurement in clause 9.10.2, when all CSI-RS resources for L3 measurement of this intra-frequency measurement object are partially overlapped by the measurement gap or the union of concurrent measurement gaps.

-	SSB-based inter-frequency measurement with no measurement gap in clause 9.3.9, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	SSB-based inter-frequency measurement with no measurement gap in clause 9.3.9, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or the union of concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

Otherwise, the carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	SSB-based intra-frequency measurement with no measurement gap in clause 9.2.5 and 9.2A.5, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap or concurrent measurement gaps.

-	SSB-based intra-frequency measurement with no measurement gap in clause 9.2.5 and 9.2A.5, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap or concurrent measurement gaps.

-	CSI-RS based intra-frequency measurement in clause 9.10.2, when none of CSI-RS resources for L3 measurement of this intra-frequency measurement object are overlapped by the measurement gap or concurrent measurement gaps.

-	CSI-RS based intra-frequency measurement in clause 9.10.2, when all CSI-RS resources for L3 measurement of this intra-frequency measurement object are partially overlapped by the measurement gap or concurrent measurement gaps.

-	SSB-based inter-frequency measurement with no measurement gap in clause 9.3.9, when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	SSB-based inter-frequency measurement with no measurement gap in clause 9.3.9, when part of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap or concurrent measurement gaps, if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network.

-	For a UE in E-UTRA-NR dual connectivity operation, NR SSB-based inter-RAT measurement object configured by the E-UTRAN PCell on an NR serving carrier

-	the SSB is completely contained in the active BWP of the UE, and

-	none or part of the SMTC occasions of this inter-RAT measurement object are overlapped by the measurement gap;

-	Intra-frequency RSSI and channel occupancy measurement with no measurement gap on a carrier subject to CCA when SMTC and RMTC are overlapping and RMTCs are not fully overlapped with measurement gap(s).

The UE is expected to conduct the measurement of this measurement object i only outside the measurement gaps.

For a UE in E-UTRA-NR dual connectivity operation, if a measurement object configured by PSCell and an NR inter-RAT measurment object configured by E-UTRAN PCell are on the same serving carrier, they shall be counted as one intra-frequency measurement object, provided that they meet the measurement object merging conditions [in clause 9.1.3.2].

The number of frequency layers for SSB measurements shall include the total number of MOs with

-	ssb-ConfigMobility configured, or

-	ssb-ConfigMobility not configured but csi-rs-ResourceConfigMobility configured with associatedSSB.

If ssbfrequency, smtc1, smtc2 and ssbSubcarrierSpacing are same in multiple MOs, the multiple MOs are counted as one SSB frequency layer.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFoutside_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

The UE cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2 and  9.10.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with Tmeasure_SFTD1 specified in clause 9.3.8 when no measurement gaps are provided.

The requirements in this clause apply provided that

-	The SMTC on all CCs and inter-frequency layers without measurement gap in FR2 have the same offset, and one of following conditions is met

-	If smtc2 is configured on any FR2 CC,

-	All CCs have the same configuration for smtc1, and

-	All CCs configured with smtc2 have the same configuration for smtc2

-	If smtc2 is not configured on any FR2 CC,

-	The total number of different SMTC periodicities on all serving CCs and inter-frequency layers without measurement gap does not exceed 4

-	The starting point of the first 5ms window for CSI-RS measurement as defined in clause 9.10.1 on all CCs in FR2 is same and one of following conditions is met

-	If any CSI-RS resource is configured in the second 5ms window for CSI-RS measurement as defined in clause 9.10.1 on any FR2 CC,

-	All CCs with CSI-RS resources only in the first 5ms window have the same CSI-RS resource periodcity, and

-	All CCs with CSI-RS resources both in the first and the second 5ms window have the same CSI-RS resource periodcity

-	If no CSI-RS resource is configured in the second 5ms window for CSI-RS measurement as defined in clause 9.10.1 on any FR2 CC,

-	The total number of different CSI-RS resources periodicities on all serving CCs does not exceed 3Note:	Longer delays for cell identification and measurement periods derived based on CSSFoutside_gap,i in clauses 9.2.5.1, 9.2.5.2, can be expected, if the UE is configured with more than 4 different SMTC periodicities on FR2 serving carriers. The longer delay applies for the FR2 intra-frequency measurement objects with the longest SMTC periodicity/periodicities.

# <End of Change #1>