3GPP TSG-RAN WG4 Meeting #108	R4-2311869

# Toulouse, France, August 21 – August 25, 2023

Source:	China Telecom

Title:	Discussion on network B requirements for MUSIM gaps

Agenda item:	8.25.2.4

Document for:	Discussion

1	Introduction

In the last meeting, network B requirements were discussed and the final agreements and open issues can be found in the WF [1].

In this contribution, we provide our views on the open issues.

2	Discussion

Issue 4-1-1: Network B requirements conditions - Proposals - P1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only (vivo CMCC Ericsson Huawei Nokia) - P1-1: The inactive state requirement should be the same as NW B’s Idle state (Ericsson Nokia) - P2: Add the condition “MUSIM gaps will not collide with other MUSIM gaps” when defining NW B requirements. (vivo) - P3: Do not add a condition stating that NW-B requirements only apply if “MUSIM gaps will not collide with other MUSIM gaps (Nokia) - P4: Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer. Agree to continue discussion other conditions during or once NW B requirements are agreed. (Nokia) - P5: Postpone the discussion of additional conditions for defining Network B requirements until there is agreement on the framework for defining the requirements (issue 4-1-2). (Qualcomm)

For Issue 4-1-1, we support P1 and P1-1. Since the behaviors of UE in inactive and idle mode are similar, and the cell re-selection requirements for RRC_INACTIVE state is the same as that for RRC_IDLE state. So we think it is reasonable to define NW B measurement/cell reselection requirements in IDLE/inactive mode, and the inactive state requirement should be the same as NW B’s Idle state.

Proposal 1: Agree to define NW B measurement/cell reselection requirements in IDLE/inactive mode, and the inactive state requirement should be the same as NW B’s Idle state.

Issue 4-1-2: Network B requirements framework - Proposals - P1: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline (vivo CMCC Apple xiaomi ZTE Huawei Qualcomm Ericsson) - P2-1: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. (vivo Apple xiaomi Huawei Qualcomm) - P2-1a: The value set of max(DRX cycle, MGRP_max) is {0.32, 0.64, 1.28, 2.56 5.12} (vivo) - P2-2: DRX cycle is replaced by max(DRX cycle, MGRP); MGRP is the MGRP of the periodic MUSIM gap with highest priority level (CMCC) - P2-3: The NW-B’s requirement should decouple with MUSIM gaps requested by UE. RAN4 to introduce a relaxed NW-B’s IDLE mode requirement as follow. (Ericsson) Table 1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for NW-B DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1; FR2-1; FR2-2. 0.32; 1; 8; 12; 36 x N1 x M2 x R1; 4 x N1 x M2 x R1; 16 x N1 x M2 x R1. 0.64; 5; 8; 28 x N1 x R1; 2 x N1 x R1; 8 x N1 x R1. 1.28; 4; 6; 25 x N1 x R1; 1 x N1 x R1; 5 x N1 x R1. 2.56; 3; 5; 23 x N1 x R1; 1 x N1 x R1; 3 x N1 x R1. Note: R1 = 2.. - P3: P2-1 and P2-2 is not reasonable. The UE measurement requirements for measurement/cell reselection requirements in IDLE/inactive mode for NW B, need to be based on a reasonable balance of the allocated MUSIM gap. (Nokia)

While defining Network B requirements, it is easy to reuse the existing idle/inactive requirements as the baseline. Considering that in MUSIM scenarios, UE can be in connected state on NW A and in idle/inactive state on NW B. Only during the configured MUSIM gaps, UE can switch to NW B to perform measurements. We agree that NW B measurement requirements are related to the configuration of MUSIM gaps.

However, it’s difficult for NW B to obtain the configuration of MUSIM gaps since UE cannot indicate NW B the information of MUSIM gaps when it is in idle/inactive state. So both P2-1 and P2-2 may be infeasible.

Based on this, we support P2-3 that NW-B’s requirement should decouple with MUSIM gaps, and UE should request MUSIM gaps based on the defined NW B requirements. But further study is needed on how to relax the requirements of NW B.

Observation 1: It’s difficult for NW B to obtain the configuration of MUSIM gaps since UE cannot indicate NW B the information of MUSIM gaps when it is in idle/inactive state.

Proposal 2: Support P2-3 that NW-B’s requirement should decouple with MUSIM gaps, and UE should request MUSIM gaps based on the defined NW B requirements. But further study is needed on how to relax the requirements of NW B.

3	Conclusion

In this paper we provided our views on NW B requirements for Rel-17 MUSIM gaps:

Proposal 1: Agree to define NW B measurement/cell reselection requirements in IDLE/inactive mode, and the inactive state requirement should be the same as NW B’s Idle state.

Observation 1: It’s difficult for NW B to obtain the configuration of MUSIM gaps since UE cannot indicate NW B the information of MUSIM gaps when it is in idle/inactive state.

Proposal 2: Support P2-3 that NW-B’s requirement should decouple with MUSIM gaps, and UE should request MUSIM gaps based on the defined NW B requirements. But further study is needed on how to relax the requirements of NW B.

Reference

- R4-2310165, WF on NR Dual TxRx Multi-SIM, vivo