3GPP TSG-RAN WG4 Meeting # 108	                                 	                        R4-2311906

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.4.3.1.2

Source:	Apple Inc.

Title:	Collection of Simulation Results for PDSCH Demodulation Requirements for 8Rx UEs in FR1

Release:	Rel-18

Document for:	Discussion

# Introduction

Overall, this WI has been focusing on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

  - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

    - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers

  - Specify the SDR requirements with 8 MIMO layers

In addition, as per the revised WID RP-231490 in RAN#100, we wait for RAN4 RF to specify UE RF requirements for both single carrier and CA/DC for 8Rx. About specific band combos and configurations, these are FFS.

  - Specify the UE RF requirements to support 8Rx for both single carrier and CA/DC

    - Example band combos and configurations need to be defined

The main aim of this paper is to summarize our PDSCH demodulation results 8Rx UEs.

# Simulation Results

In what follows, we summarize the simulation results for the defined PDSCH requirements. So far, no impairment conditions have been considered yet.

TDD

| Rank | MCS | Propagation Condition | SNR[dB] @ 70% Rel Thr |
| --- | --- | --- | --- |
| 2 | 20 (256QAM Table) | TDLC 300-100 ULA MediumB | 19.7 |
|  | 19 (64QAM Table) |  | 11.2 |
| 4 | 26 (64QAM Table) | TDLA 30-10 ULA Low | 21.2 |
|  | 17 (64QAM Table) |  | 11.0 |
| 8 | 17 (64QAM Table) |  | 20.3 |

FDD

| Rank | MCS | Propagation Condition | SNR[dB] @ 70% Rel Thr |
| --- | --- | --- | --- |
| 2 | 20 (256QAM Table) | TDLC 300-100 ULA MediumB | 20.2 |
|  | 19 (64QAM Table) |  | 11.6 |
| 4 | 26 (64QAM Table) | TDLA 30-10 ULA Low | 20.5 |
|  | 17 (64QAM Table) |  | 10.8 |
| 8 | 17 (64QAM Table) |  | 19.9 |

# References

- R4-2309806, “WF on 8Rx UE performance requirements”, Huawei, HiSilicon, RAN4#107

- R4-2311905, “On the PDSCH Demodulation Requirements for 8Rx UEs in FR1 in TDD, FDD and CA Modes”, Apple Inc.

- R4-2311904, “Further Discussion on General Aspects of 8Rx Requirements in FR1”, Apple Inc.