3GPP TSG-RAN WG4 Meeting #108	R4-2311917

Toulouse, France, 21th- 25th, Aug, 2023

Title:	LS on further clarification for ue-PowerClassPerBandPerBC-r17 (R4 16-8)

Response to:

Release:	Rel-17

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Yuanyuan Zhang

E-mail Address:  tina55.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

In RAN4#106 meeting, RAN4 replied RAN2’s LS on the received clarification questions for the interaction between ue-PowerClassPerBandPerBC-r17 (R4 16-8) and other existing different power class parameters. After extended discussion during RAN4#107 and RAN4#108 meetings, RAN4 would like to have following further clarification with RAN2.

- ue-PowerClassPerBandPerBC-r17 is applicable to only NR inter-band UL CA, i.e. when there is uplink configured in two different operating bands. Each uplink band contains only single UL CC or intra-band contiguous UL CA.

- ue-PowerClassPerBandPerBC-r17 indicates the power class that a UE supports for each individual band within a given band combination, while powerClass or powerClass-v1610 indicates the power class for this band combination.

- ue-PowerClassPerBandPerBC-r17 if indicated for a band shall determine the power class of this constituent band of a band combination (It is not expected that the power class indicated in this field would be higher than the power class of the band combination), otherwise the applicable power class for the individual bands within a band combination is determined as min {ue-PowerClass, powerClass for the BC}.

- ue-PowerClassPerBandPerBC-r17 does not modify powerClass/powerClass-v1610 for a band combination and conversely. The capability definition of powerClass/powerClass-v1610 requires update to include ue-PowerClassPerBandPerBC-r17, i.e., if the power class indicated by powerClass/powerClass-v1610 is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass in BandNR otherwise), the latter determines maximum TX power available in each band.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 clarification into consideration.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #108-bis	9 - 13 October 2023              Xiamen, China

TSG-RAN WG4 Meeting #109	13 - 18 November 2023         Chicago, USA