3GPP TSG-RAN4 Meeting#108	R4-2311919

# Toulouse, France, 21th-25th, Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | ❌ | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 draft CR for 38.101-1 Allow 4-MIMO layer per cc for Type-2 supported UE for collocated deployment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Below description in 38.101-1 makes 4-MIMO layer per cc configuration for Type-2 supported UE not achievable under collocated BS scenario which is against operators’s demand. |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable 4-MIMO layer per cc configuration for Type-2 supported UE for collocated deployment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE MIMO capabability is limited even in collcoated scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 7.10A	Power imbalance for CA

### 7.10A.1	General

Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal.

Power imbalance requirement in this subclause is only applicable for a UE indicating [intraBandNonColocatedCA-r18].

### 7.10A.2	Minimum requirement

For the test parameters in table 7.10A.2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. |  |  |  |  |

For a UE indicating [intraBandNonColocatedCA-r18] for the following CA band combinations in Table 7.10A.2-2, the Rx requirements for two Rx ports are applicable for each component carrier and the requirements for four Rx ports per component carrier do not apply if [new NW IE] is provided.

Table 7.10A.2-2: NR CA combinations

CA combination

CA_n77(2A)

CA_n78(2A)

<End of change>