3GPP TSG-RAN4 Meeting#108	R4-2311920

# Toulouse, France, 21th-25th, Aug, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | ❌ | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 draft CR for 38.101-3 Allow 4-MIMO layer per cc for Type-2 supported UE for collocated deployment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Below description in 38.101-3 makes 4-MIMO layer per cc configuration for Type-2 supported UE not achievable under collocated BS scenario which is against operators’s demand. |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable 4-MIMO layer per cc configuration for Type-2 supported UE for collocated deployment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE MIMO capabability is limited even in collcoated scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

7.10B	power imbalance for DC in FR1

7.10B.3	Inter-band EN-DC within FR1

Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal (LTE or NR) in the presence of another carrier signal (NR or LTE) with 25dB power imbalance at a specific frequency offset from the wanted signal.

Power imbalance requirement in this subclause is only applicable for a UE indicating capability interBandMRDC-WithOverlapDL-Bands-r16.

For these test parameters in table 7.10B.3-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 in 38.101-1).

Table 7.10B.3-1: Test parameters for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship (Center of BWanother Relative to edge of BWwanted) |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier with overlapping DL bands | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier with overlapping DL bands | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier with overlapping DL bands | Power of wanted carrier + 25 |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 24dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 24 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 3: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with overlapping DL bands NOTE 4: REFSENS is the reference sensitivity level or two antenna port in Table 7.3.2-1a or in Table 7.3.2-1b of TS 38.101-1, or in Table 7.3.1-1 of TS 36.101. |  |  |  |  |

The applicability of these test configurations is shown below:

When the capability interBandContiguousMRDC is indicated, test configuration 1 or 2 can be used to verify the RX power imbalance requirement for a UE indicating capability interBandMRDC-WithOverlapDL-Bands-r16.

When the capability interBandContiguousMRDC is absent, test configuration 1, 2 or 3 can be used to verify the RX power imbalance requirement for a UE indicating capability interBandMRDC-WithOverlapDL-Bands-r16.

It’s allowed to use one of the test configurations to verify the RX power imbalance requirement for a UE indicating capability interBandMRDC-WithOverlapDL-Bands-r16.

For a UE indicating interBandMRDC-WithOverlapDL-Bands-r16 for the following EN-DC band combinations in Table 7.10B.3-2, the Rx requirements for four Rx ports do not apply for each band in EN-DC operating mode if [new NW IE] is provided.

Table 7.10B.3-2: EN-DC combinations

EN-DC combination

DC_42_n77

DC_42_n78

<End of change>